R. Pitt

July 6, 2020

Green Roof Performance as Reported in the Literature

Contents

Main Conclusions from the Literature	1

Types of Green Roofs	2

Substrates used in Green Roofs	3

Green Roof Substrate Selection and Potential Amendments to Control Runoff Quality	4

Biochar as a Component of Green Roof Substrates	8

Green Roof Water Quantity	10

Green Roof Water Quality	13

Green Roofs as Sources or Sinks of Pollutants	13

Comparisons of Green Roof Runoff Quality to Other Source Areas	16

Seasonal Patterns of Phosphorus Concentrations in Green Roof Runoff	17

Effects of Green Roof Age on Runoff Quality	19

Suggested WinSLAMM Green Roof Components	20

References	21

# Main Conclusions from the Literature

I reviewed about 50 published papers and reports on green roof runoff quality, with about half containing potentially useful information for use in WinSLAMM and summarized in this white paper. The main conclusions from these papers were:

- Vegetation affects runoff quantity (evapotranspiration is a major water loss mechanism)

- Vegetation does not consistently affect green roof water quality (pollutant uptake is minor, especially in the absence of harvesting). Vegetation uptake of nitrates and orthophosphates were periodically observed.

- Green roof phosphorus runoff quality is dependent on the substrate type and fertilization

- Green roof phosphorus runoff quality is highly seasonal with long-term downward trend, but is not related to rain depth or runoff quantity

The following review includes excerpts from the references supporting these comments, along with suggested WinSLAMM use of these data and information.

# Types of Green Roofs

The two main types of green roofs are intensive and extensive, as illustrated in the following photographs. Extensive green roofs are far more common and suitable for retrofit installations, especially considering the weight of intensive green roof systems. Berndtsson, et al. (2009) noted the release of phosphorus from the extensive green roof (mostly in form of PO4), while no release was noted from the intensive green roof. They state that the probable source of phosphorus was from the fertilizer applied during construction and from phosphorus leaching from the soil substrate, as also concluded in many other studies.

Emilsson, et al. (2007) described typical extensive green roofs in Sweden as having water-saturated weights of approximately 50–55 kg/m2. They have a 3 to 5 cm growing substrate layer composed of inorganic low-density material with large water-holding capacities, such as pumice, scoria or lava. They are dry for long periods and can therefore only support drought-tolerant species unless irrigated. The substrate mixes also have high water permeability and low organic content to prevent decomposition and compaction of the growing layer. The fertilizers used are most often encapsulated controlled release fertilizers, which are designed to release nutrients at a pace similar to the nutrient requirements of the vegetation.

Source: Berndtsson, et al. 2009.

Source: Berndtsson, et al. 2009.

## Substrates used in Green Roofs

The following table lists the variety of substrates that have been reported in extensive green roofs. Jennett and Zheng (2018) report that substrates may behave as either sources or sinks of P, depending on the components they are formulated from. They also found that few direct links have been established among substrate components and their physicochemical characteristics that would affect P-retention.

Source: Jennett and Zheng (2018)

## Green Roof Substrate Selection and Potential Amendments to Control Runoff Quality

The Fertilizer Association of Ireland and Teagasc (2019) describes the P-index and how it relates to phosphorus from soils. The soil index system divides soils into one of four soil index levels based on the soil P test results. They also present the following figure indicating the relationship of soil pH with phosphorus fixation with iron (very low pH less than about 4.5), with aluminum (acidic soils in the pH range of about 4.5 to 6), and with calcium for alkaline soils (soil pH above 7).

Source: Fertilizer Association of Ireland and Teagasc (2019)

Cornell University (2019) published the New York Phosphorus Runoff Index for agricultural areas to determine the relative risk of phosphorus runoff. This process also requires an evaluation of the material (soil or substrate) and the site conditions. The user’s manual and documentation (Czymmek, et al. (undated) describes the features of this index, as summarized in the following tables.

Source: Czymmek, et al. (undated)

Source: Czymmek, et al. (undated)

As an example, a potential application to a green roof may include the following factors:

- Flow distance (FD): <100 ft

- Flooding frequency (FF): frequent (whenever it rains)

- Concentrated flow present (discharge from green roof)

- Soil types depending on texture class of substrate mixture

- Erosion rate: < 1 ton/acre

- Buffer absent

Beecham and Razzaghmanesh (2014) studied green roofs having different substrates in the dry and hot climate of Adelaide, South Australia. They found that the organic substrate mix produced higher orthophosphate concentrations. The scoria mix had intermediate concentrations, while the crushed brick mix had the lowest concentrations. This indicates that leaching of organics in the growing media resulted in higher orthophosphate concentrations in the runoff. However, in the vegetated green roofs, the phosphorous concentration was reduced by 60% to 80%, while there were no differences between the different types of vegetated green roofs. Plant uptake of the orthophosphate was therefore observed during these studies. The following table summarizes these findings.

Source: Beecham and Razzaghmanesh (2014)

Wang, et al. (2017) examined dual-substrate layered green roofs in Beijing. All of the dual-substrate-layered green roofs appeared to be sinks for organics, heavy metals and all forms of nitrogen in all cases, while they acted as sources of phosphorus contaminants during heavy rains. They recommend a mixture of activated charcoal and/or pumice with perlite and vermiculite as the adsorption substrate for long service life and pollutant reduction.

## Biochar as a Component of Green Roof Substrates

Qianqian, et al. (2019) examined the use of biochar as a substrate on green roofs located in China. They did not identify any significance difference in the runoff retention for green roofs having commercial substrates and with biochar substrates (both about 72%). They found that both substrates had gradually decreasing concentrations of total nitrogen, total phosphorus, COD, and iron with time. The biochar substrate resulted in TN (9.8 mg/L) and COD (97 mg/L) concentrations, which were about half of the commercial substrate (TN at 16 mg/L and COD at 172 mg/L).

Kuoppamäki and Lehvävirta (2016) also investigated two birch biochars (prepared at different pyrolysis temperatures and times) on green roofs in Finland. The green roofs were thin-layered (2 to 12 cm of substrate) Sedum/moss roofs, while two were meadow roofs (with substrate depths of 21 and 23 cm). At the beginning of the measurement period they found negligible effects due to the biochar, but after one year, the biochar retained nutrients. The total annual loads (product of both quantity and concentration reductions) of nutrients were significantly reduced by the biochar amendments in both green roof types. The following tables describe the biochar characteristics.

Source: Kuoppamäki and Lehvävirta (2016)

Source: Kuoppamäki and Lehvävirta (2016)

Kuoppamäki, et al. (2016) in another publication stated that the biochar reduced the cumulative leaching of nutrients, even though biochar did not significantly reduce nutrient concentrations. In the laboratory experiments, one type of biochar reduced nutrient concentrations and loads in the runoff, while another type had an opposite effect. The properties and effects of biochar can vary considerably, requiring specific testing to measure their benefits for green roof use. Pyrolysis conditions and the raw material have strong effects on the characteristics of a biochar, with varying runoff results due to the quality of the biochar.

Although grasses have been shown to be more effective in reducing runoff than Sedum, Kuoppamäki, et al. (2016) did not find any effects of vegetation type (Sedum vs. meadow) in terms of water retention. This was probably due to the poor survival of grasses and herbs in the green roofs monitored. They found that substrate depth, rather than vegetation type, can determine retention capacity. They concluded that avoiding fertilization and careful selection of substrate material, including use of studied amendments, are appropriate means to control nutrient leaching from green roofs.

Source: Kuoppamäki, et al. (2016)

# Green Roof Water Quantity

The following figure, from Peczkowski, et al. (2018), shows a typical cross-section of an extensive green roof and the water loss mechanisms.

Source: Peczkowski, et al. (2018)

Carter and Rasmussen (2006) monitored a paired test of a green roof and a control roof in Georgia over a one-year period. The rains ranged from 0.28 to 8.43 cm for the 31 events monitored. As shown on the following graph, the green roof rainfall capture decreased with increasing rainfall depth, ranging from just under 90 percent for the smaller storms (< 2.54 cm) to slightly less than 50 percent for the larger storms (> 7.62 cm). Runoff from the green roof was also delayed with an average increase of about 18 minutes for the green roof compared to the control roof.

Source: Carter and Rasmussen (2006)

Fassman-Beck, et al. (2013) monitored green roofs in Auckland, New Zeeland. The roof was irrigated on each day having no rainfall. They found this was too much as the irrigation caused increased runoff due to lengthy periods of higher saturated substrate. The following table compares the retention rate and peak flow reductions for the green roof compared to control roof conditions.

Source: Fassman-Beck, et al. (2013)

They also found seasonal variations in the hydrologic response of the green roof, as shown in the following table.

Source: Fassman-Beck, et al. (2013)

During their tests, they also compared green roof runoff conditions for the two large installations with a series of small green roofs on small sheds. The shed green roofs responded quite differently (faster runoff responses) due to the shorted path to the roof outlet, and different permeability of the substrate material.

Fassman-Beck, et al. (2013) also compared evapotranspiration for stressed (overly dry conditions) to unstressed conditions for several different plants and growing environments, as shown in the table below.

Source: Fassman-Beck, et al. (2013)

Fassman-Beck and Simcock (2013) also reported that Individual rains had 56%-72% runoff retention by the green roofs which had 100 to 150 mm substrate depths which were designed to maximize water storage.

# Green Roof Water Quality

## Green Roofs as Sources or Sinks of Pollutants

Fassman-Beck, et al, (2013) found little agreement in the literature as to green roofs being sinks or sources of nutrients. However, there is much agreement that the composition of the substrate and the application of fertilizer affects green roof runoff quality.

Gnecco, et al. (2013) compared the pollutant loads from atmospheric deposition with green roof runoff. Infiltration through the green roof substrate resulted in increasing concentrations for solids, K, Ca and Fe, while Zn and Cu were reduced. They concluded that the substrate can be a source of some metals, while reducing other pollutant concentrations.

Vijayaraghavan and Joshi (2014) also investigated green roofs as sinks for various pollutants. They found that Ca, Mg, Al, Fe, Cr, Cu, Ni, Zn, Pb, and Cd were reduced by the green roofs, along with neutralizing acid rain. They stated that low-cost and locally available materials such as perlite, vermiculite, sand, crushed brick, and coco-peat produced better runoff in comparison with that of local garden soil. They also note that runoff from unplanted green roofs appeared earlier and had higher dissolved pollutant concentrations than the planted (P. grandiflora) green roofs. They concluded that the selection of plant species for green roofs should not be entirely dictated by aesthetics and drought-tolerant potential, but also on their phytoremediation potential (which is not well known for plants being used on green roofs).

Fassman-Beck and Simcock (2013) during their green (living) roof monitoring in Auckland found that neither the green roof nor the conventional roof surfaces produced elevated suspended solids (TSS) or nitrate+nitrite concentrations. Copper may be from the green roof substrates, while both copper and zinc are from roofing materials. Soluble Reactive Phosphorus (SRP) and Total Kjeldhal Nitrogen (TKN) are the predominant nutrients discharged at elevated concentrations from the green roofs, as shown on the following table. Runoff from all the green roofs were a source of nitrogen, primarily in the form of TKN as opposed to NOx. NOx is readily taken up by plants, while TKN is less plant-available and is comprised of ammonia, ammonium, and organic nitrogen.

Source: Fassman-Beck and Simcock (2013) (Tamaki are the small shed test green roofs while the WWC are full-sized green roofs.

Culligan, et al. (2014) monitored green roofs and control roofs in New York City. The roofs were located on a variety of buildings and represented a wide range of extensive green roof installation types, including vegetated mat, built up, and modular tray systems, as well as different plant types. A number of constituents had greater concentrations in the runoff from the green roof runoffs compared to the runoff from the control roofs (as shown in the following table). However, there is an overall reduction in the volume of runoff from green roofs, and therefore less mass of pollutants discharged from the green roofs. They also state that improved management by reduced fertilization of green roofs would also result in reduced nutrient discharges.

Source: Culligan, et al. (2014)

Ferrans, et al. (2018) studied green roofs in Bogota. Rainfall and green roof runoff from 12 rain events were monitored for total Kjeldahl nitrogen, nitrates, nitrites, ammonia, total phosphorus, phosphates, pH, total dissolved solids, total suspended solids, color, turbidity, biological oxygen demand, chemical oxygen demand, total coliforms, metals (i.e., zinc, copper, nickel, lead, selenium, aluminum, barium, boron, calcium, strontium, iron, lithium, magnesium, manganese, potassium, sodium), and polyaromatic hydrocarbons, as shown on the following table. The results showed that green roofs can neutralize pH, but they were also a source of the rest of the above listed constituents, excluding PAHs, ammonia, TSS, selenium and lithium. Substrate type, event size, and rainfall characteristics were all found to be significant variables for explaining runoff water quality. The retention efficiencies were not significantly different for both the vegetated (86%) and non-vegetated (85%) systems.

Source: Ferrans, et al. (2018)

## Comparisons of Green Roof Runoff Quality to Other Source Areas

Barr, et al. (2017) compared fertilized green roof runoff water quality with other vegetated locations and stormwater control measure effluents, near Villanova University, PA. The vegetated sites included a grassed site, a wooded site, and a mixed-use site with pavement and grass on or adjacent to the Villanova University campus. The stormwater control measures included a constructed stormwater wetland and a bioinfiltration rain garden. They found that the green roof discharges much greater phosphorus concentrations than the other locations, except for the fertilized grass area. They also did not identify any significant correlations between green roof runoff concentrations and rainfall volume.

Source: Barr, et al. (2017)

## Seasonal Patterns of Phosphorus Concentrations in Green Roof Runoff

Mitchell, et al. (2017) measured phosphate in green roof runoff over a 4-year period at an extensive green roof in Cincinnati, OH. They found that the phosphate concentrations were similar to runoff from heavily fertilized agricultural fields. The pattern of the phosphate concentrations displayed a strong seasonal pattern, along with a rapid decline over the 4-year study (as shown in the following figure and table).

Source: Mitchell, et al. (2017)

Source: Mitchell, et al. (2017)

They found that even after 5 years following green roof installation, the phosphate concentrations were still high relative to natural systems and even regularly fertilized agricultural areas, often by an order of magnitude, or more. They also found that common green roof plants (such as Sedum), that are selected for stress and drought resistance have reduced nutrient requirements and growth rates, therefore, they do not require the high phosphorus levels found in green roof substrates, let alone additional fertilization.

Buffam, et al. (2016) also discussed the Civic Garden Center green roof in Cincinnati. As noted above, they observed strong seasonal patterns in bioactive elements, with carbon, nitrogen, phosphorus, and base cation concentrations highest in the summer, and positively correlated with temperature. They concluded that the dominant mechanism responsible for seasonality in runoff water quality from this green roof are most closely linked to variations in temperature, rather than hydrology (rain size/type) or growing season (plant activity). The following table presents green roof runoff concentrations, by season.

Source: Buffam, et al. (2016)

Carpenter, et al. (2016) studied green roofs in Syracuse, NY. They monitored wet deposition and roof runoff. Water quality was measured during 87 storms during about a 12-month period in 2011 and 2012. Water and nutrient (total phosphorus, total nitrogen, and dissolved organic carbon) mass balances were conducted on an event basis to evaluate retention annually and during the growing and nongrowing seasons. Green roof runoff had high concentrations of nutrients, especially total nitrogen and dissolved organic carbon, during the warm temperature growing season. Overall, nutrient mass discharges were low because of the large retention of water. However, there were large variations in the retention of the nutrients by season due to variations in concentrations in the roof runoff.

Vijayaraghavan, et al. (2012) conducted green roof field tests in Singapore. Concentrations of most of the chemicals in the runoff were highest at the beginning of rain events and decreased during subsequent rain events. The concentration of the chemical components in the roof runoff strongly depends on the nature of the substrates used in the green roof and the volume of rain.

## Effects of Green Roof Age on Runoff Quality

Okita, et al. (2018) evaluated the water quality of stormwater runoff from a regular (non-vegetated) roof, a green roof installed 6 months previously, and a green roof installed 6 years ago in Portland, Oregon. Samples of runoff were taken during every rain event for 10 months, and analyzed for total phosphorus, phosphate, total nitrogen, nitrate, ammonia, copper, and zinc. Runoff from the green roofs had higher concentrations of TP and PO4 and lower concentrations of Zn compared to the regular roof. Average TP concentrations from the 6-year old roof and 6-month old roof were 6.3 and 14.6 times higher, respectively, than concentrations from the regular roof, and average PO4 concentrations from the 6-year old roof and 6-month old roof were 13.5 and 26.6 times higher, respectively, compared to the regular roof. The 6-month old green roof phosphorus and phosphate concentrations were about twice the concentrations as from the 6-year old green roof, while the copper and zinc concentrations were about 30% greater from the 6-year old green roof compared to the newer green roof. The following table shows these concentrations.

Source: Okita, et al. (2018)

# Suggested WinSLAMM Green Roof Components

- If a green roof is intensive (rare), then use standard landscaping runoff quality factors.

- If a green roof is extensive and fertilized, and/or has organic substrate material (such as compost or soils) having high P-index (≥2) or P-loss risk scores, then use the following seasonal and long-term phosphate trends, depending on the roof age:

Approximate phosphate and nitrates seasonal trends:

|  | Spring Concentration (range) | Summer Concentration (range) | Fall Concentration (range) | Winter Concentration (range) |
| --- | --- | --- | --- | --- |
| Dissolved organic nitrogen (mg/L) | 1.6 (1.1 – 7.2) | 10.3 (2.6 – 35.8) | 1.6 (0.1 – 19.3) | 1.2 (0.7 – 2.0) |
| Nitrate, NO3 (mg/L) | 0.1 (0.0 – 1.9) | 3.1 (0.7 – 13.2) | 2.5 (0.0 – 8.5) | 0.1 (0.0 – 0.5) |
| Phosphate, PO4 (mg/L) | 1.6 (0.9 – 2.4) | 2.8 (1.7 – 4.2) | 2.3 (1.0 – 3.5) | 1.2 (0.8 – 1.6) |

Source: summarized from Buffam, et al. (2016)

Approximate phosphate trends with age of green roof (to adjust the above seasonal values):

|  | Phosphate (mg/L) | Ratio with year 1 |
| --- | --- | --- |
| Year 1 | 2.2 | 1.0 |
| Year 2 | 1.5 | 0.68 |
| Year 3 | 1.2 | 0.55 |
| Year 4 and later | 0.9 | 0.41 |

Source: calculated from Mitchell, et al. (2017)

Concentrations of non-nutrient contaminants do not undergo the seasonal or time changes, or respond to fertilizers or organic substrates, so TSS, copper, and zinc concentration values can be selected from the table in the following section.

- If a green roof is extensive with inert substrate materials (such as sand, clay and clay-like material, lightweight inorganics, and some inert organics such as coir and peat) having low P-index (1) or P-loss risk scores, and not fertilized, use the following constant phosphate values:

Approx.  concentrations for unfertilized green roofs:

|  | Median Concentration (COV) |
| --- | --- |
| TSS (mg/L) | 1.4 (1.1) |
| NOx (mg/L) | 0.48 (0.50) |
| TN (mg/L) | 2.0 (1.5) |
| Soluble reactive phosphorus (mg/L) | 0.40 (0.40) |
| Total phosphorus (mg/L) | 0.41 (0.39) |
| Phosphorus particulate strength (mg/kg) | 7.1 |
| Filtered copper (ug/L) | 14 (0.16) |
| Total copper (ug/L) | 16 (0.13) |
| Copper particulate strength (mg/kg) | 1.4 |
| Filtered zinc (ug/L) | 12 (0.13) |
| Total zinc (ug/L) | 13 (0.18) |
| Zinc particulate strength (mg/kg) | 0.7 |

Sources:

calculate from Fassman-Beck and Simcock (2013) full scale roofs

Amendment (such as biochar) benefits for green roofs are not included due to their highly varying runoff quality benefits reported in the literature. Plant uptake of nutrients is also not considered also due to varying benefits in the literature (no benefit to uptake of NOx and phosphates, if healthy and unstressed).

# References

Barr, C.M.. P. M. Gallagher, B. M. Wadzu and A. L. Welker. “Water quality impacts of green roofs compared with other vegetated sites.” J. Sustainable Water Built Environ., 2017, 3(3): 04017007.

Beecham, S. and M. Razzaghmanesh. “Water quality and quantity investigation of green roofs in a dry climate.” Water Research. 70 (2015) 370-384. http://dx.doi.org/10.1016/j.watres.2014.12.015.

Berndtsson, J. C., L. Bengtsson, and K. Jinno. “Runoff water quality from intensive and extensive vegetated roofs.” Ecological Engineering 35 (2009) 369–380.

Buffam, I., M.E. Mitchell, R.D. Durtsche. “Environmental drivers of seasonal variation in green roof runoff water quality.” Ecological Engineering. 91 (2016) 506–514.

Carpenter, C.M.G., D. Todorov, C. T. Driscoll, and M. Montesdeoca. “Water quantity and quality response of a green roof to storm events: Experimental and monitoring observations.” Environmental Pollution 218 (2016) 664-672.

Carter, T. L. and T. C. Rasmussen. “Hydrologic behavior of vegetated roofs.” Journal of the American Water Resources Association. 42(5):1261-1274. 2006.

Cornell University. New York Phosphorus Runoff Index. Cornell University Nutrient Management Spear Program. 2019.  http://nmsp.cals.cornell.edu/publications/pindex.html

Culligan, P., et al. Evaluation of Green Roof Water Quantity and Quality Performance in an Urban Climate. EPA/600/R-14/180. Office of Research and Development. U.S. Environmental Protection Agency. September 2014.

Czymmek, K.J., Q. M. Ketterings, M. Ros, S. Cela, S. Crittenden, D. Gates, T. Walter, S. Latessa, L. Klaiber, and G. Albrecht. The New York Phosphorus Runoff Index: Version 2.0, User’s Manual and Documentation. 2019.  http://nmsp.cals.cornell.edu/publications/pindex.html

Emilsson,T., J. C. Berndtsson, J. E. Mattsson, and K. Rolf. “Effect of using conventional and controlled release fertilizer on nutrient runoff from various vegetated roof systems.” Ecological Engineering 29 (2007) 260–271.

Fassman-Beck, E., and R. Simcock. “Hydrology and water quality of living roofs in Auckland.” NOVATECH 2013.

Fassman-Beck, E.A., R. Simcock, E.A. Voyde and Y.S. Hong. Extensive Green (Living) Roofs for Stormwater Mitigation, Part 2: Performance Monitoring. Prepared by Auckland UniServices for Auckland Council. Auckland Council technical report, TR2010/018. 2013.

Ferrans, P., C. V. Rey, G. Pérez, J. P. Rodríguez and M. Díaz-Granados. “Effect of green roof configuration and hydrological variables on runoff water quantity and quality.” Water, 10,960. 2018.

Fertilizer Association of Ireland in association with Teagasc. The Efficient Use of Phosphorus in Agricultural Soils. Technical Bulletin Series – No. 4, February 2019. https://www.teagasc.ie/media/website/crops/soil-and-soil-fertility/Efficient-Use-of-Phosphorus-In-Agriculture-Tech-Bulletin-No.-4.pdf

Gnecco, L., A. Palla, L.G. Lanza, and P. La Barbera. “The role of green roofs as a source/sink of pollutants in storm water outflows.” Water Resources Management (2013) 27:4715–4730. DOI 10.1007/s11269-013-0414-0.

Jennett, T.S. and Y. Zheng. “Component characterization and predictive modeling for green roof substrates optimized to adsorb P and improve runoff quality: A review.” Environmental Pollution. 237 (2018) 988-999.

Kuoppamäki, K. and S. Lehvävirta. “Mitigating nutrient leaching from green roofs with biochar.” Landscape and Urban Planning. 152 (2016) 39–48.

Kuoppamäki, K., M. Hagner, S. Lehvävirta, and H.Setäläa. “Biochar amendment in the green roof substrate affects runoff quality and quantity.” Ecological Engineering. 88 (2016) 1–9.

Mitchell, M.E., S. F. Matter, R. D. Durtsche, and I. Buffam. “Elevated phosphorus: dynamics during four years of green roof development.” Urban Ecosystems. (2017) 20:1121–1133.

Okita, J., C. Poor, J.M. Kleiss and T. Eckmann. Effect of green roof age on runoff water quality in Portland, Oregon. Journal of Green Building. Volume 13, Number 2. 2018.

Peczkowski, G., T. Kowalczyk, K. Szawernoga, W. Orzepowski, R. Z˙muda and R. Pokładek. “Hydrological performance and runoff water quality of experimental green roofs.” Water. 2018, 10, 1185; doi:10.3390/w10091185

Qianqian, Z., M. Liping, W. Huiwei, and W. Long. “Analysis of the effect of green roof substrate amended with biochar on water quality and quantity of rainfall runoff.” Environmental Monitoring Assessment (2019) 191: 304

Vijayaraghavan, K. and U.M. Joshi. “Can green roof act as a sink for contaminants? A methodological study to evaluate runoff quality from green roofs.” Environmental Pollution. 194 (2014) 121-129.

Vijayaraghavan, K., U.M. Joshi, and R. Balasubramanian. “A field study to evaluate runoff quality from green roofs.” Water Research 4 6 (2012) 1337 - 1345.

Wang, X., Y. Tian, and X. Zhao. “The influence of dual-substrate-layer extensive green roofs on rainwater runoff quantity and quality.” Science of the Total Environment. 592 (2017) 465-476.