R. Pitt

July 28, 2020

Rainfall Interception by Japanese Maple Trees in the Southeast US

Tipping bucket rain gages were placed under two Japanese Maple trees to calculate throughfall compared to a nearby similar rain gage placed in the open. The following are photographs of these rain gage installations.

Open area rain gage.

Backyard Japanese Maple rain gage location.

Sky view above backyard Japanese Maple rain gage.

Front yard Japanese Maple rain gage.

Sky view above front yard Japanese Maple rain gage.

A total of 21.53 inches or rain over 37 rains were recorded at the open area location between March 29 and July 27, 2020. The March, April, and May rains are in the spring season (18 rains), while the June and July rains are in the summer season (19 rains). The rain characteristics at the open area rain gage were as follows:

| Spring Season (60 days, 12.8 inches total rain) | minimum | maximum | average | median | COV |
| --- | --- | --- | --- | --- | --- |
| Interevent period (days) | 0.46 | 9.0 | 3.2 | 2.4 | 0.91 |
| Rain duration (hrs) | 0.065 | 27 | 6.9 | 5.2 | 1.1 |
| Total rain depth (inches) | 0.02 | 3.29 | 0.71 | 0.33 | 1.4 |
| Average rain intensity (in/hr) | 0.007 | 0.73 | 0.18 | 0.14 | 0.98 |

| Summer Season (48 days, 8.7 inches total rain) | minimum | maximum | average | median | COV |
| --- | --- | --- | --- | --- | --- |
| Interevent period (days) | 0.29 | 12.3 | 7.7 | 1.1 | 1.22 |
| Rain duration (hrs) | 0.25 | 20 | 6.1 | 2.4 | 1.1 |
| Total rain depth (inches) | 0.04 | 1.24 | 0.46 | 0.43 | 0.81 |
| Average rain intensity (in/hr) | 0.01 | 1.6 | 0.29 | 0.09 | 1.3 |

Concurrent throughfall measurements were made beneath the two medium sized Japanese Maple trees (Appendix A). The throughfall depths were plotted to compare them to the open area rain depths, as shown below. The Spring and Summer throughfall values were compared for each tree with no statistically significant difference, although the front yard tree visually indicated less throughfall for rains less than 0.5 inches compared to the backyard tree. The front yard tree has several layers of branches above the rain gage, while the backyard rain gage had fewer.

The front yard rain gage measurements near the end of the Spring monitoring period were uncertain due to the rain gage orifice clogging from algae. These later Spring rains at this location were therefore not used in these calculations. The regression relationships between throughfall and the open area rainfall and the slope coefficients of the equations are highly significant (p < 0.0001 calculated from ANOVA analyses). Overall, the throughfall values are about 13 and 14% less than the rainfall, indicating low amounts of rainfall interception by these trees during the Spring and Summer periods. While these photographs indicate extensive leafing of these maples, these medium sized trees apparently do not have nearly the amount of interception capacity as the larger pine and oak trees previously monitored.

Appendix: Throughfall Measurements under Japanese Maple Trees

Standard Rain Gage Surrounded by Garden and Grass

| event # | season | beginning time | ending time | interevent time (days) | rain duration (hrs) | Grass total rain (inches) | avg int (in/hr) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | spring | 03/29/20 04:34:38AM | 03/29/20 04:39:34AM | n/a | 0.1 | 0.06 | 0.73 |
| 2 | spring | 03/31/20 06:59:42AM | 03/31/20 08:18:32AM | 2.1 | 1.3 | 0.49 | 0.37 |
| 3 | spring | 04/07/20 12:35:43PM | 04/07/20 06:31:13PM | 7.2 | 5.9 | 0.34 | 0.06 |
| 4 | spring | 04/08/20 07:30:39AM | 04/08/20 04:36:47PM | 0.5 | 9.1 | 0.07 | 0.01 |
| 5 | spring | 04/09/20 05:52:35AM | 04/09/20 06:58:49AM | 0.6 | 1.1 | 0.16 | 0.14 |
| 6 | spring | 04/12/20 11:17:43AM | 04/13/20 01:17:44AM | 3.2 | 14.0 | 3.04 | 0.22 |
| 7 | spring | 04/18/20 01:42:06AM | 04/18/20 06:11:08AM | 5.0 | 4.5 | 0.3 | 0.07 |
| 8 | spring | 04/19/20 04:53:14AM | 04/20/20 08:12:56AM | 0.9 | 27.3 | 3.29 | 0.12 |
| 9 | spring | 04/22/20 04:52:16PM | 04/23/20 12:27:28PM | 2.4 | 19.6 | 1.62 | 0.08 |
| 10 | spring | 04/29/20 06:28:09PM | 04/29/20 06:28:14PM | 6.3 | 0.1 | 0.02 | 0.24 |
| 11 | spring | 05/08/20 09:16:13AM | 05/08/20 03:12:37PM | 8.6 | 5.9 | 0.07 | 0.01 |
| 12 | spring | 05/17/20 03:29:15PM | 05/17/20 10:52:06PM | 9.0 | 7.4 | 1.25 | 0.17 |
| 13 | spring | 05/18/20 07:41:30PM | 05/18/20 08:05:05PM | 0.9 | 0.4 | 0.07 | 0.18 |
| 14 | spring | 05/22/20 06:07:58PM | 05/22/20 09:21:47PM | 3.9 | 3.2 | 0.82 | 0.25 |
| 15 | spring | 05/23/20 07:14:13PM | 05/23/20 07:18:07PM | 0.9 | 0.1 | 0.02 | 0.31 |
| 16 | spring | 05/26/20 07:54:04AM | 05/26/20 07:28:54PM | 2.5 | 11.6 | 0.44 | 0.04 |
| 17 | spring | 05/27/20 06:34:10AM | 05/27/20 05:25:22PM | 0.5 | 10.9 | 0.42 | 0.04 |
| 18 | spring | 05/28/20 05:14:05AM | 05/28/20 07:27:33AM | 0.5 | 2.2 | 0.32 | 0.14 |
| 19 | summer | 6/5/2020 13:58 | 6/5/2020 21:18 | 8.3 | 7.32 | 0.18 | 0.02 |
| 20 | summer | 6/8/2020 07:56 | 6/9/2020 03:44 | 2.4 | 19.80 | 0.65 | 0.03 |
| 21 | summer | 6/9/2020 14:13 | 6/10/2020 08:49 | 0.4 | 18.61 | 0.55 | 0.03 |
| 22 | summer | 6/22/2020 15:00 | 6/23/2020 07:47 | 12.3 | 16.78 | 1.03 | 0.06 |
| 23 | summer | 6/23/2020 16:25 | 6/23/2020 20:19 | 0.4 | 3.90 | 1.24 | 0.32 |
| 24 | summer | 6/24/2020 13:27 | 6/25/2020 01:22 | 0.7 | 11.93 | 0.48 | 0.04 |
| 25 | summer | 6/25/2020 11:57 | 6/25/2020 12:43 | 0.4 | 0.77 | 0.04 | 0.05 |
| 26 | summer | 6/25/2020 20:41 | 6/25/2020 22:48 | 0.3 | 2.13 | 0.19 | 0.09 |
| 27 | summer | 6/29/2020 18:20 | 6/29/2020 19:07 | 3.8 | 0.78 | 0.39 | 0.50 |
| 28 | summer | 7/1/2020 07:05 | 7/1/2020 22:14 | 1.5 | 15.14 | 1.21 | 0.08 |
| 29 | summer | 7/6/2020 16:59 | 7/6/2020 19:25 | 4.8 | 2.44 | 0.61 | 0.25 |
| 30 | summer | 7/7/2020 02:17 | 7/7/2020 11:23 | 0.3 | 9.10 | 0.07 | 0.01 |
| 31 | summer | 7/7/2020 20:49 | 7/8/2020 00:00 | 0.4 | 3.19 | 0.04 | 0.01 |
| 32 | summer | 7/12/2020 13:35 | 7/12/2020 13:58 | 4.6 | 0.39 | 0.61 | 1.57 |
| 33 | summer | 7/19/2020 14:23 | 7/19/2020 14:38 | 7.0 | 0.25 | 0.07 | 0.28 |
| 34 | summer | 7/20/2020 16:45 | 7/20/2020 17:22 | 1.1 | 0.62 | 0.32 | 0.51 |
| 35 | summer | 7/21/2020 13:09 | 7/21/2020 14:14 | 0.8 | 1.07 | 0.43 | 0.40 |
| 36 | summer | 7/22/2020 18:58 | 7/22/2020 19:14 | 1.2 | 0.28 | 0.19 | 0.68 |
| 37 | summer | 7/23/2020 19:56 | 7/23/2020 20:42 | 1.0 | 0.76 | 0.43 | 0.56 |

| Spring grass | interevent time (days) | rain duration (hrs) | Grass total rain (inches) | avg int (in/hr) |
| --- | --- | --- | --- | --- |
| sum: |  |  | 12.8 |  |
| count | 17 | 18 | 18 | 18 |
| min | 0.46 | 0.07 | 0.02 | 0.01 |
| max | 9.01 | 27.33 | 3.29 | 0.73 |
| average | 3.23 | 6.93 | 0.71 | 0.18 |
| median | 2.36 | 5.20 | 0.33 | 0.14 |
| stdev | 2.95 | 7.52 | 0.99 | 0.17 |
| COV | 0.91 | 1.09 | 1.40 | 0.98 |
| Summer grass | interevent time (days) | rain duration (hrs) | Grass total rain (inches) | avg int (in/hr) |
| sum: |  |  | 8.73 |  |
| count | 19 | 19 | 19 | 19 |
| min | 0.29 | 0.25 | 0.04 | 0.01 |
| max | 12.26 | 19.80 | 1.24 | 1.57 |
| average | 2.72 | 6.07 | 0.46 | 0.29 |
| median | 1.09 | 2.44 | 0.43 | 0.09 |
| stdev | 3.33 | 6.95 | 0.37 | 0.38 |
| COV | 1.22 | 1.14 | 0.81 | 1.31 |
| All Seasons grass | interevent time (days) | rain duration (hrs) | Grass total rain (inches) | avg int (in/hr) |
| sum: |  |  | 21.53 |  |
| count | 36 | 37 | 37 | 37 |
| min | 0.29 | 0.07 | 0.02 | 0.01 |
| max | 12.26 | 27.33 | 3.29 | 1.57 |
| average | 2.96 | 6.48 | 0.58 | 0.23 |
| median | 1.35 | 3.23 | 0.39 | 0.14 |
| stdev | 3.12 | 7.14 | 0.74 | 0.30 |
| COV | 1.05 | 1.10 | 1.28 | 1.27 |

Rain Gage Located Beneath Backyard Japanese Maple Tree

| event # | beginning time | ending time | interevent time (days) | rain duration (hrs) | rear maple total rain (inches) | avg int (in/hr) | back maple/grass rain ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 03/29/20 04:36:12AM | 03/29/20 04:41:43AM | n/a | 0.1 | 0.04 | 0.44 | 0.67 |
| 2 | 03/31/20 07:02:30AM | 03/31/20 08:35:47AM | 2.1 | 1.6 | 0.42 | 0.27 | 0.86 |
| 3 | 04/07/20 01:24:10PM | 04/07/20 06:33:06PM | 7.2 | 5.1 | 0.32 | 0.06 | 0.94 |
| 4 | 04/08/20 05:33:16AM | 04/08/20 03:55:12PM | 0.5 | 10.4 | 0.05 | 0.00 | 0.71 |
| 5 | 04/09/20 05:53:15AM | 04/09/20 06:59:13AM | 0.6 | 1.1 | 0.15 | 0.14 | 0.94 |
| 6 | 04/12/20 11:43:12AM | 04/12/20 11:08:01PM | 3.2 | 11.4 | 2.65 | 0.23 | 0.87 |
| 7 | 04/18/20 01:44:16AM | 04/18/20 06:16:25AM | 5.1 | 4.5 | 0.24 | 0.05 | 0.80 |
| 8 | 04/19/20 04:54:53AM | 04/20/20 08:26:54AM | 0.9 | 27.5 | 2.91 | 0.11 | 0.88 |
| 9 | 04/22/20 09:00:53PM | 04/23/20 07:04:51AM | 2.5 | 10.1 | 1.37 | 0.14 | 0.85 |
| 10 | 04/29/20 06:28:09PM |  |  |  | 0 |  | 0.00 |
| 11 | 05/08/20 11:25:42AM | 05/08/20 12:06:26PM | 15.2 | 0.7 | 0.03 | 0.04 | 0.43 |
| 12 | 05/17/20 03:35:24PM | 05/18/20 12:03:24AM | 9.1 | 8.5 | 1.03 | 0.12 | 0.82 |
| 13 | 05/18/20 08:01:32PM | 05/18/20 08:34:22PM | 0.8 | 0.5 | 0.04 | 0.07 | 0.57 |
| 14 | 05/22/20 06:26:18PM | 05/23/20 01:58:24AM | 3.9 | 7.5 | 0.76 | 0.10 | 0.93 |
| 15 | 05/23/20 07:14:13PM |  |  |  | 0 |  | 0.00 |
| 16 | 05/26/20 09:14:50AM | 05/26/20 07:27:22PM | 3.3 | 10.2 | 0.33 | 0.03 | 0.75 |
| 17 | 05/27/20 01:53:33PM | 05/27/20 05:58:33PM | 0.8 | 4.1 | 0.33 | 0.08 | 0.79 |
| 18 | 05/28/20 05:24:52AM | 05/28/20 07:34:26AM | 0.5 | 2.2 | 0.28 | 0.13 | 0.88 |
| 19 | 6/5/2020 14:01 | 6/5/2020 21:20 | 198.4 | 0.12 | 0.12 | 0.97 | 0.67 |
| 20 | 6/8/2020 08:11 | 6/8/2020 23:46 | 58.8 | 15.58 | 0.51 | 0.03 | 0.78 |
| 21 | 6/9/2020 14:15 | 6/10/2020 08:46 | 14.5 | 18.52 | 0.46 | 0.02 | 0.84 |
| 22 | 6/22/2020 15:02 | 6/23/2020 07:55 | 294.3 | 16.88 | 0.91 | 0.05 | 0.88 |
| 23 | 6/23/2020 16:29 | 6/23/2020 22:07 | 8.6 | 5.64 | 1.01 | 0.18 | 0.81 |
| 24 | 6/24/2020 13:31 | 6/24/2020 21:18 | 15.4 | 7.78 | 0.38 | 0.05 | 0.79 |
| 25 | 6/25/2020 11:58 | 6/25/2020 12:48 | 14.7 | 0.84 | 0.04 | 0.05 | 1.00 |
| 26 | 6/25/2020 20:43 | 6/25/2020 22:34 | 7.9 | 1.84 | 0.14 | 0.08 | 0.74 |
| 27 | 6/29/2020 18:56 | 6/29/2020 19:07 | 92.4 | 0.19 | 0.27 | 1.44 | 0.69 |
| 28 | 6/30/2020 06:15 | 7/2/2020 01:55 | 11.1 | 43.66 | 0.93 | 0.02 | 0.77 |
| 29 | 7/6/2020 17:03 | 7/6/2020 19:27 | 111.1 | 2.40 | 0.57 | 0.24 | 0.93 |
| 30 | 7/7/2020 08:01 | 7/7/2020 11:36 | 12.6 | 3.58 | 0.05 | 0.01 | 0.71 |
| 31 | 7/7/2020 21:46 | 7/7/2020 23:55 | 10.2 | 2.15 | 0.01 | 0.00 | 0.25 |
| 32 | 7/12/2020 13:37 | 7/12/2020 14:32 | 109.7 | 0.91 | 0.49 | 0.54 | 0.80 |
| 33 | 7/19/2020 14:29 | 7/19/2020 14:44 | 168.0 | 0.24 | 0.06 | 0.25 | 0.86 |
| 34 | 7/20/2020 16:49 | 7/20/2020 17:35 | 26.1 | 0.77 | 0.25 | 0.32 | 0.78 |
| 35 | 7/21/2020 13:13 | 7/21/2020 14:26 | 19.6 | 1.22 | 0.38 | 0.31 | 0.88 |
| 36 | 7/22/2020 19:00 | 7/22/2020 21:17 | 28.6 | 2.29 | 0.18 | 0.08 | 0.95 |
| 37 | 7/23/2020 19:59 | 7/23/2020 20:36 | 22.7 | 0.63 | 0.37 | 0.59 | 0.86 |

| Spring backyard Japanese Maple | interevent time (days) | rain duration (hrs) | rear maple total rain (inches) | avg int (in/hr) | back maple/grass rain ratio |
| --- | --- | --- | --- | --- | --- |
| sum: |  |  | 10.95 |  |  |
| count | 15 | 16 | 18 | 16 | 18 |
| min | 0.46 | 0.09 | 0.00 | 0.00 | 0.00 |
| max | 15.18 | 27.53 | 2.91 | 0.44 | 0.94 |
| average | 3.72 | 6.59 | 0.61 | 0.13 | 0.70 |
| median | 2.52 | 4.84 | 0.30 | 0.10 | 0.81 |
| stdev | 4.10 | 6.87 | 0.87 | 0.11 | 0.29 |
| COV | 1.10 | 1.04 | 1.44 | 0.85 | 0.41 |
| Summer backyard Japanese Maple | interevent time (days) | rain duration (hrs) | rear maple total rain (inches) | avg int (in/hr) | back maple/grass rain ratio |
| sum: |  |  | 7.13 |  |  |
| count | 19 | 19 | 19 | 19 | 19 |
| min | 7.92 | 0.12 | 0.01 | 0.00 | 0.25 |
| max | 294.27 | 43.66 | 1.01 | 1.44 | 1.00 |
| average | 64.45 | 6.59 | 0.38 | 0.28 | 0.79 |
| median | 22.69 | 2.15 | 0.37 | 0.08 | 0.80 |
| stdev | 80.01 | 10.75 | 0.31 | 0.38 | 0.16 |
| COV | 1.24 | 1.63 | 0.82 | 1.37 | 0.20 |
| All Seasons backyard Japanese Maple | interevent time (days) | rain duration (hrs) | rear maple total rain (inches) | avg int (in/hr) | back maple/grass rain ratio |
| sum: |  |  | 18.08 |  |  |
| count | 34 | 35 | 37 | 35 | 37 |
| min | 0.46 | 0.09 | 0.00 | 0.00 | 0.00 |
| max | 294.27 | 43.66 | 2.91 | 1.44 | 1.00 |
| average | 37.66 | 6.59 | 0.49 | 0.21 | 0.75 |
| median | 10.65 | 2.40 | 0.32 | 0.10 | 0.80 |
| stdev | 66.60 | 9.05 | 0.65 | 0.29 | 0.23 |
| COV | 1.77 | 1.37 | 1.33 | 1.42 | 0.31 |

Rain Gage Located Beneath Front Yard Japanese Maple Tree

| event # | beginning time | ending time | interevent time (days) | rain duration (hrs) | front maple total rain (inches) | avg int (in/hr) | front maple/grass rain ratio | front/back maple rain ratio |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 03/29/20 04:39:40AM | 03/29/20 04:46:30AM | n/a | 0.11 | 0.02 | 0.18 | 0.33 | 0.50 |
| 2 | 03/31/20 07:06:52AM | 03/31/20 08:33:02AM | 2.10 | 1.44 | 0.37 | 0.26 | 0.76 | 0.88 |
| 3 | 04/07/20 02:03:48PM | 04/07/20 07:07:12PM | 7.23 | 5.06 | 0.16 | 0.03 | 0.47 | 0.50 |
| 4 | 04/08/20 03:57:44AM | 04/08/20 08:03:11AM | 0.37 | 4.09 | 0.02 | 0.00 | 0.29 | 0.40 |
| 5 | 04/09/20 05:55:35AM | 04/09/20 06:57:53AM | 0.91 | 1.04 | 0.08 | 0.08 | 0.50 | 0.53 |
| 6 | 04/12/20 11:57:08AM | 04/13/20 09:23:24AM | 3.21 | 21.44 | 2.86 | 0.13 | 0.94 | 1.08 |
| 7 | 04/18/20 01:46:09AM | 04/18/20 06:17:02AM | 4.68 | 4.51 | 0.23 | 0.05 | 0.77 | 0.96 |
| 8 | 04/19/20 05:06:49AM | 04/21/20 01:53:38PM | 0.95 | 56.78 | 2.62 | 0.05 | 0.80 | 0.90 |
| 9 | 04/22/20 09:19:58PM | 04/23/20 12:18:40PM | 1.31 | 14.98 | 1.15 | 0.08 | 0.71 | 0.84 |
| 10 | 04/29/20 06:28:09PM |  |  |  | 0 |  | 0.00 | 0.00 |
| 11 | 05/08/20 11:25:42AM |  |  |  | 0 |  | 0.00 | 0.00 |
| 12 | 05/17/20 03:36:58PM | 05/18/20 07:16:22AM | 24.14 | 15.66 | 0.93 | 0.06 | 0.74 | 0.90 |
| 13 | 05/18/20 08:08:46PM | 05/18/20 08:46:47PM | 0.54 | 0.63 | 0.02 | 0.03 | 0.29 | 0.50 |
| 14 | 05/22/20 06:57:54PM | 05/23/20 11:39:57AM | 3.92 | 16.70 | 0.68 | 0.04 | 0.83 | 0.89 |
| 15 | 05/23/20 07:14:13PM | n/a | n/a | n/a | n/a | n/a | n/a | n/a |
| 16 | 05/26/20 09:14:50AM | n/a | n/a | n/a | n/a | n/a | n/a | n/a |
| 17 | 05/27/20 01:53:33PM | n/a | n/a | n/a | n/a | n/a | n/a | n/a |
| 18 | 05/28/20 05:24:52AM | n/a | n/a | n/a | n/a | n/a | n/a | n/a |
| 19 | 6/5/2020 14:01 | 6/5/2020 14:12 | n/a | 0.182 | 0.08 | 0.440 | 0.44 | 0.67 |
| 20 | 6/8/2020 08:36 | 6/8/2020 23:42 | 2.77 | 15.098 | 0.32 | 0.021 | 0.49 | 0.63 |
| 21 | 6/9/2020 14:14 | 6/10/2020 08:50 | 0.6 | 18.60 | 0.36 | 0.02 | 0.65 | 0.78 |
| 22 | 6/22/2020 15:03 | 6/23/2020 09:05 | 12.3 | 18.03 | 0.77 | 0.04 | 0.75 | 0.85 |
| 23 | 6/23/2020 16:31 | 6/24/2020 01:36 | 0.3 | 9.08 | 1.04 | 0.11 | 0.84 | 1.03 |
| 24 | 6/24/2020 13:53 | 6/25/2020 02:10 | 0.5 | 12.28 | 0.27 | 0.02 | 0.56 | 0.71 |
| 25 | 6/25/2020 20:44 | 6/25/2020 22:38 | 0.8 | 1.89 | 0.12 | 0.06 | 3.00 | 3.00 |
| 26 | 6/25/2020 20:41 |  |  |  | 0 |  | 0.00 | 0.00 |
| 27 | 6/29/2020 18:59 | 6/29/2020 21:22 | 3.8 | 2.39 | 0.26 | 0.11 | 0.67 | 0.96 |
| 28 | 7/1/2020 07:07 | 7/2/2020 01:10 | 1.4 | 18.04 | 1.09 | 0.06 | 0.90 | 1.17 |
| 29 | 7/6/2020 17:05 | 7/6/2020 19:48 | 4.7 | 2.71 | 0.49 | 0.18 | 0.80 | 0.86 |
| 30 | 7/7/2020 11:16 | 7/7/2020 11:21 | 0.6 | 0.08 | 0.01 | 0.12 | 0.14 | 0.20 |
| 31 | 7/7/2020 20:49 |  |  |  | 0 |  | 0.00 | 0.00 |
| 32 | 7/12/2020 13:36 | 7/12/2020 14:12 | 5.1 | 0.60 | 0.71 | 1.19 | 1.16 | 1.45 |
| 33 | 7/19/2020 15:15 | 7/19/2020 15:20 | 7.0 | 0.08 | 0.01 | 0.12 | 0.14 | 0.17 |
| 34 | 7/20/2020 16:50 | 7/20/2020 17:44 | 1.1 | 0.90 | 0.21 | 0.23 | 0.66 | 0.84 |
| 35 | 7/21/2020 13:17 | 7/21/2020 14:29 | 0.8 | 1.20 | 0.37 | 0.31 | 0.86 | 0.97 |
| 36 | 7/22/2020 06:39 | 7/22/2020 19:23 | 0.7 | 12.73 | 0.13 | 0.01 | 0.68 | 0.72 |
| 37 | 7/23/2020 19:59 | 7/23/2020 20:50 | 1.0 | 0.84 | 0.34 | 0.40 | 0.79 | 0.92 |

| Spring front yard Japanese Male | interevent time (days) | rain duration (hrs) | front maple total rain (inches) | avg int (in/hr) | front maple/grass rain ratio | front/back maple rain ratio |
| --- | --- | --- | --- | --- | --- | --- |
| sum: |  |  | 9.14 |  |  |  |
| count | 11 | 12 | 14 | 12 | 14 | 14 |
| min | 0.37 | 0.11 | 0.00 | 0.00 | 0.00 | 0.00 |
| max | 24.14 | 56.78 | 2.86 | 0.26 | 0.94 | 1.08 |
| average | 4.49 | 11.87 | 0.65 | 0.08 | 0.53 | 0.63 |
| median | 2.10 | 4.79 | 0.20 | 0.06 | 0.60 | 0.69 |
| stdev | 6.85 | 15.98 | 0.96 | 0.07 | 0.31 | 0.34 |
| COV | 1.53 | 1.35 | 1.47 | 0.88 | 0.58 | 0.54 |
| Summer front yard Japanese Maple | interevent time (days) | rain duration (hrs) | front maple total rain (inches) | avg int (in/hr) | front maple/grass rain ratio | front/back maple rain ratio |
| sum: |  |  | 6.58 |  |  |  |
| count | 16 | 17 | 19 | 17 | 19 | 19 |
| min | 0.31 | 0.08 | 0.00 | 0.01 | 0.00 | 0.00 |
| max | 12.26 | 18.60 | 1.09 | 1.19 | 3.00 | 3.00 |
| average | 2.72 | 6.75 | 0.35 | 0.20 | 0.71 | 0.84 |
| median | 1.04 | 2.39 | 0.27 | 0.11 | 0.67 | 0.84 |
| stdev | 3.25 | 7.35 | 0.34 | 0.29 | 0.64 | 0.65 |
| COV | 1.20 | 1.09 | 0.97 | 1.41 | 0.90 | 0.78 |
| All Seasons front yard Japanese Maple | interevent time (days) | rain duration (hrs) | front maple total rain (inches) | avg int (in/hr) | front maple/grass rain ratio | front/back maple rain ratio |
| sum: |  |  | 15.72 |  |  |  |
| count | 27 | 29 | 33 | 29 | 33 | 33 |
| min | 0.31 | 0.08 | 0.00 | 0.00 | 0.00 | 0.00 |
| max | 24.14 | 56.78 | 2.86 | 1.19 | 3.00 | 3.00 |
| average | 3.44 | 8.87 | 0.48 | 0.15 | 0.64 | 0.75 |
| median | 1.31 | 4.09 | 0.26 | 0.08 | 0.67 | 0.84 |
| stdev | 4.99 | 11.74 | 0.68 | 0.23 | 0.53 | 0.54 |
| COV | 1.45 | 1.32 | 1.42 | 1.50 | 0.83 | 0.72 |