R. Pitt

June 24, 2020

# Stormwater Phosphorus Characteristics and WinSLAMM Modifications

Contents

Summary	1

New York State Phosphorus Report Summary	3

Recommendations to Incorporate Changing Seasonal Phosphorus Concentrations in WinSLAMM	5

Seasonal Ratios for EPA Rain Zone 1 (Upper Midwest and Northeast)	7

Seasonal Ratios for EPA Rain Zone 2 (Mid-Atlantic)	8

Seasonal Ratios for EPA Rain Zone 3 (Southeast)	9

Seasonal Ratios for EPA Rain Zone 4 (Lower Mississippi)	10

Seasonal Ratios for EPA Rain Zone 5 (most of Texas and some surrounding areas)	11

Seasonal Ratios for EPA Rain Zone 6 (Southwest)	12

Seasonal Ratios for EPA Rain Zone 7 (Coastal Northwest)	14

Seasonal Ratios for EPA Rain Zone 8 (Interior Northwest)	15

Seasonal Ratios for EPA Rain Zone 9 (Northern Plains)	16

Conclusions	17

References	17

Appendix A: Analysis of Total Phosphorus Stormwater Monitoring Information as Contained in the National Stormwater Quality Database (NSQD), Version 4.02	19

Introduction	19

Observations	20

Land use and geographical region analyses	21

Evaluation of Total Phosphorus Stormwater Concentrations	23

Appendix B: Seasonal, Land Use, and Location Values from NSQD ver 4.	48

# Summary

It has long been desired to incorporate an ability to modify the phosphorus concentrations for each season in the WinSLAMM parameter files. The following is a summary of constituent seasonal variations are reported in the National Stormwater Quality Database (ver 4.02) (Maestre and Pitt 2006). Appendix A contains details of this seasonal analyses, stressing the upper Midwest and Northeast region of the US (from the NY report noted below). Appendix B contains data used in this memo to calculate the ratios of the median seasonal concentrations to the annual concentrations by location and land use for all constituents that had significant seasonal variations.

Single land use concentration data were examined using full factorial analyses to identify significant main factors (seasons, land use, and location) and significant interactions. Two-way and three-way interactions were included in the analyses. Table 1 shows the results from the factorial analyses. A p-value smaller than 0.05 indicates that the factor, or interaction of factors, was found to be significant at least at the 5% significance level.

The results indicated that the factors for land use, and the EPA Rain Zone (location), plus the interaction land use–rain zone were the most important factors. Therefore, local data should be used for the specific land uses under consideration. Season alone was not significant for any constituent except for TKN. Note that bacteria were not evaluated due to missing data in some of the important categories. Historically, bacteria have been found to vary by season (generally much lower in colder seasons for northern areas). For all the constituents except TSS, concentrations in the open space land use were lower than for the other land uses, while, samples collected at freeways and industrial sites had the highest concentrations. Total phosphorus and TKN appear to be high during the summer and low during the winter. No clear variations in concentration were observed between spring and fall. Samples collected in EPA Rain Zone 3 (the southeast) had the lowest concentrations for all the constituents. The largest concentrations were observed in EPA Rain Zone 6 (the southwest) followed by rain zones 1 (Great Lakes and northeast) and 4 (Mississippi valley).

Table 1. p-values for main factors and interactions

| Constituent | Land Use (LU) | Season (SN) | EPA Rain Zone (EPA) | LU*SN | LU*EPA | SN*EPA | LU*EPA*SN |
| --- | --- | --- | --- | --- | --- | --- | --- |
| TSS mg/L | <0.0001 | 0.74 | <0.0001 | 0.02 | <0.0001 | 0.18 | <0.0001 |
| BOD mg/L | <0.0001 | 0.16 | <0.0001 | 0.0008 | <0.0001 | 0.001 | 0.22 |
| COD mg/L | <0.0001 | 0.13 | <0.0001 | 0.03 | <0.0001 | 0.014 | 0.009 |
| TP mg/L | <0.0001 | 0.69 | <0.0001 | 0.055 | <0.0001 | 0.0004 | <0.0001 |
| NO2+NO3 mg/L | <0.0001 | 0.11 | <0.0001 | 0.052 | <0.0001 | 0.034 | 0.057 |
| TKN mg/L | 0.0026 | 0.02 | <0.0001 | 0.99 | <0.0001 | <0.0001 | 0.17 |
| Cu µg/L | <0.0001 | 0.11 | <0.0001 | 0.62 | <0.0001 | 0.038 | 0.14 |
| Pb µg/L | <0.0001 | 0.76 | <0.0001 | 0.42 | <0.0001 | 0.29 | 0.012 |
| Zn µg/L | <0.0001 | 0.91 | <0.0001 | 0.94 | <0.0001 | 0.014 | <0.0001 |

Yellow high-lighted cells indicate significant differences (p<0.05)

All of the constituents listed above have significant seasonal effects when examining the seasonal effects interaction with location, or with both location and land use. Therefore, it may be worthwhile to expand seasonal options for all constituents, not just for phosphorus. The ratios of seasonal concentrations to annual concentrations for each land use and location are shown in this memo, reflecting these factorial analyses.

It is suggested that a similar approach previously presented for particle size modifications be used for seasonal constituent differences. This would use ratio factors showing the adjustments for each season compared to the current overall constituent values. These factors need to be applied

to both the particulate strength values and the filtered concentration values. Not all constituents have data reflecting seasonal changes, and not all seasons will be modified. For situations of no change, the ratio factors would be 1.0. Limited data are available for seasons, but it may be most useful to allow changes for each month, as seasonal ranges vary. If using just seasonal changes, then the beginning months of each season would have to be specified (such as using the current season definitions for the tree interception equations). Also, these analyses are for the land use concentrations, while WinSLAMM parameter files use land use source area concentration values. Source area seasonal data are limited and not available for these analyses. In the absence of these seasonal source area data, these land use factors could be applied to the source areas.

The following is a summary from a series of reports I prepared for the New York State Attorney General’s office of phosphorus characteristics and treatment that has some more detailed information concerning phosphorus variations and trends.

# New York State Phosphorus Report Summary

Several years ago, I prepared a report for the New York State Attorney General’s office: Review of Stormwater Phosphorus Characteristics and Treatment for New Development in the New York City Watershed (September 5, 2017). The following is the executive summary of that report:

‘This report addresses the following question: “Can treated stormwater from a development project proposed at an undeveloped site (e.g. forest, meadow) be expected to achieve no net increase in total phosphorus (TP) loading?” The short answer is “highly unlikely.”  This report also evaluates the effectiveness of certain stormwater controls for new development projects in the New York City Watershed and provides recommendations for improvements that can be implemented through the 2017 Filtration Avoidance Determination (FAD).

Stormwater regulations of the New York City Department of Environmental Protection (DEP) do not require applicants to perform phosphorous loading analyses which would ensure that effluents from proposed development projects do not increase phosphorus discharges, compared to pre-development levels. (This is also true for New York State Department of Environmental Conservation (DEC) permits, but the subject of this study is stormwater policies within the New York City Watershed within the context of potential improvements by DEP pursuant to the 2017 FAD.) This presents a particular difficulty for the City’s Croton system reservoirs that do not currently meet water quality standards due to eutrophic conditions from excessive phosphorus discharges within their watersheds. Moreover, phosphorous discharges within the Delaware system reservoirs can also contribute to this problem in Croton system reservoirs. Water from the Delaware System’s Rondout reservoir is fed by the other Delaware system reservoirs (the Cannonsville, Neversink, and Pepacton) and can travel through the Delaware Aqueduct into the West Branch Reservoir and then into the Croton system’s eutrophic Croton Falls, Muscoot, and New Croton reservoirs.

As discussed below, even when stormwater controls are properly designed and implemented, we can generally expect a net increase in phosphorus discharges from new development. To address this problem, I recommend that sponsors of development projects be required to establish that their planned stormwater controls would achieve no net increase in phosphorus discharges. TP and total suspended solids (TSS) are usually closely correlated, along with some metals. Therefore, removal of TP in sedimentation ponds or wetlands will also result in the removal of about half the TSS. Metals would be removed somewhat less than the TSS (lead the most, copper and zinc less). Infiltration controls (with no underdrains) would reduce all pollutants at the same rate, as the infiltrating water removes all pollutants from the surface runoff.

Recent research has shown that some of the stormwater controls allowed by DEP regulations -- grassed filter strips, grass swales, and bioretention systems – can actually increase phosphorus concentrations in stormwater. These controls can therefore lead to increased phosphorus discharge loading (concentrations times runoff volumes).  Therefore, the use of these controls needs to be carefully evaluated to ensure that increases in phosphorus concentrations are offset by runoff volume reductions, in addition to meeting other control objectives.”

As part of this report, I prepared several extensive appendices (Appendix A is included as part of this memo):

“• Appendix A: provides an analysis of TP data in the National Stormwater Quality Database. According to this analysis, location, land use, and season can all affect the expected total TP concentrations in stormwater. Data from the upper Midwest and Northeast U.S. were examined and three statistically significant land use groups were identified: 1) residential land uses (low density suburban to high density residential areas), 2) open space (such as parks, golf courses, and cemeteries) plus industrial land uses, and 3) institutional land uses (such as schools and government facilities) plus commercial land uses (central business districts, shopping centers and strip commercial). (There were too few observations from freeway locations in these regions for comparison).

Limited data indicates that snowmelt from developed urban areas can contribute relatively large TP loads to a receiving water. These discharges, and their control, also need to be considered when evaluating changes in TP loads associated with development.

• Appendix B: The International Stormwater BMP Database evaluates the performance of stormwater controls. This appendix summarizes the effectiveness of stormwater treatment on TP and includes: biological filtration, bioretention, dry detention basins, grass filter strips, grass swales, inlet inserts, media filters, oil/grit separators, physical separators, physical separators with volume control, porous pavement, sand filtration, treatment trains, wetland basins, wetland channels, and wet retention ponds.

• Appendix C: presents state regulations restricting the use of TP in fertilizers and laundry detergents. These regulations are an important part of TP control in developed areas but need to be used in conjunction with other stormwater controls.

• Appendix D: presents information for a number of models that can evaluate TP sources, discharges, and controls in urban areas. Many of the most common models were developed to assist with drainage design, where large single storms were evaluated. Some users have adapted these methods to evaluate smaller precipitation events and their impacts on water quality. If only outfall flow conditions are desired, and data are available to verify the model, these models can be reasonably accurate. However, these modified models are often in error when quantifying pollutant sources and can lead to large errors when applied to smaller precipitation events and to water quality analyses. Models used without verification may be suitable for preliminary analyses, with the understanding that greater uncertainty will accompany the results.

• Appendix E: Hidden Meadow Development Stormwater Analysis Summary. As noted earlier, the Hidden Meadow development is on a forested site. It is about 35 acres in size, with about 10 acres developed in multi-family residences, parks, and roadways, and 5 acres of undeveloped area that drains onto the developed area. The stormwater quality model, WinSLAMM (version 10.3), was used to calculate the expected post-development stormwater quality (focusing on total phosphorus) for this typical residential development.

The total phosphorus load after development is expected to increase by about 0.22 pounds per year, even with the extensive stormwater controls at the site and offsite stormwater treatment. Detailed information for this analysis is included in this appendix.

• Appendix F: Costco Stormwater Analysis Summary. The Costco site is a redeveloped area that was originally occupied by a plant nursery/lawn mower service center, two residential properties, a former motel, and a fence company. The re-developed area is comprised of large parking areas and a large Costco building, along with several outbuildings. The total area is about 24 acres. The pre- and post- redeveloped site areas were evaluated using the stormwater quality model, WinSLAMM (version 10.3), to calculate stormwater total phosphorus discharges for this typical redevelopment commercial project.

The total phosphorus load after re-development is expected to increase by about 1.37 pounds per year, even with the extensive stormwater controls at the site and offsite stormwater treatment. Detailed information for this analysis is included in this appendix.”

# Recommendations to Incorporate Changing Seasonal Phosphorus Concentrations in WinSLAMM

The NSQD (ver 4) was sorted by major land use categories, then by location, and finally by season. The particulate strengths were calculated based on the total minus filtered concentrations divided by the TSS concentrations and multiplied by a factor to result in mg/kg units. These particulate concentrations and the filtered concentrations were then summarized by land use, location, and season. Appendix B contains these data. Ratios of the seasonal median values to the annual median values, by location and land use for the filtered concentrations and particulate strength values are shown in the following tables. The following are the constituents summarized on these tables (if data are available):

TSS (mg/L)

Fecal Coliform (#/100 mL)

Ammonia (mg/L)

N02+NO3 (mg/L as N)

Phosphorous Filtered as P (mg/L)

Phosphorus particulate strength (mg/kg)

Phosphate Ortho as P (mg/L)

Cadmium Filtered (ug/L)

Cadmium part strength (ug/kg)

Chromium Filtered (ug/L)

Chromium part strength (ug/kg)

Copper Filtered (ug/L)

Copper part strength (ug/kg)

Iron, Filtered as Fe (ug/L)

Iron part strength (ug/kg)

Lead Filtered After 1984 (ug/L)

Lead part strength (ug/kg)

Nickel Filtered (ug/L)

Nickel part strength (ug/kg)

Zinc Filtered (ug/L)

Few seasons, locations, or land uses have complete sets of data, while most have data for TSS, bacteria, and nutrients. Blanks on the tables reflect an absence of information and should be replaced with ratios of 1.0 (no seasonal variations). A few ratios are yellow-highlighted reflecting ratios of <0.1 or >10. Most of these extreme ratios are for fecal coliform bacteria, although some of the metals also indicate extreme ratios. Some of the regions also have few data (especially region 8 which only had one NSQD city and only residential data.

## Seasonal Ratios for EPA Rain Zone 1 (Upper Midwest and Northeast)

| ratio of medians (seasonal to annual value) | TSS (mg/L) | Fecal Coliform (#/100 mL) | Ammonia (mg/L) | N02+NO3 (mg/L as N) | Phosphorous Filtered as P (mg/L) | Phos part strength (mg/kg) | Phosphate Ortho as P (mg/L) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| resid EPA1 Winter | 0.93 | 0.12 | 0.64 | 1.33 | 1.23 | 0.68 | 0.34 |
| resid EPA1 Spring | 1.11 | 0.22 | 1.51 | 0.93 | 0.94 | 1.01 | 0.80 |
| resid EPA1 Summer | 1.29 | 13.35 | 1.25 | 1.00 | 0.92 | 0.99 | 1.03 |
| resid EPA1 Fall | 0.60 | 6.32 | 0.82 | 0.92 | 1.72 | 1.18 | 1.16 |
| commercial EPA1 Winter | 1.46 | 0.18 | 0.81 | 1.10 | 1.00 | 0.70 |  |
| commercial EPA1 Spring | 1.96 | 2.51 | 1.09 | 1.05 | 0.93 | 0.55 |  |
| commercial EPA1 Summer | 1.06 | 21.46 | 1.12 | 0.99 | 0.80 | 1.30 |  |
| commercial EPA1 Fall | 0.69 | 1.07 | 1.19 | 0.89 | 1.17 | 1.56 |  |
| institutional EPA1 Winter |  |  |  |  |  |  |  |
| institutional EPA1 Spring | 0.65 |  |  | 0.93 |  |  |  |
| institutional EPA1 Summer | 1.02 | 1.13 |  | 0.94 | 1.00 | 1.00 |  |
| institutional EPA1 Fall | 0.18 | 0.47 | 1.00 | 1.38 | 1.00 | 2.58 |  |
| industrial EPA1 Winter | 2.30 | 0.05 | 0.96 | 1.12 | 5.75 | 0.86 |  |
| industrial EPA1 Spring | 1.12 | 0.22 | 1.15 | 1.08 | 0.75 | 0.97 |  |
| industrial EPA1 Summer | 0.90 | 13.85 | 1.26 | 1.04 | 0.65 | 1.05 |  |
| industrial EPA1 Fall | 0.83 | 1.00 | 0.66 | 0.75 | 1.15 | 1.17 |  |
| open space EPA1 Winter |  |  |  |  |  |  |  |
| open space EPA1 Spring | 1.21 | 3.83 | 0.97 | 1.30 |  |  |  |
| open space EPA1 Summer |  |  |  |  |  |  |  |
| open space EPA1 Fall | 1.00 | 1.00 | 1.03 | 0.74 |  |  | 1.00 |

## Seasonal Ratios for EPA Rain Zone 2 (Mid-Atlantic)

| ratio of medians (seasonal to annual value) | TSS (mg/L) | Fecal Coliform (#/100 mL) | Ammonia (mg/L) | N02+NO3 (mg/L as N) | Phosphorous Filtered as P (mg/L) | Phos part strength (mg/kg) | Phosphate Ortho as P (mg/L) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| resid EPA2 Winter | 0.86 | 0.60 | 0.76 | 0.86 | 0.81 | 1.01 |  |
| resid EPA2 Spring | 1.22 | 0.63 | 1.21 | 1.08 | 0.81 | 1.01 |  |
| resid EPA2 Summer | 1.14 | 3.75 | 1.62 | 1.18 | 1.09 | 0.93 |  |
| resid EPA2 Fall | 0.85 | 1.92 | 0.93 | 0.93 | 1.13 | 1.04 |  |
| commercial EPA2 Winter | 1.17 | 0.35 | 0.82 | 0.88 | 0.82 | 0.73 |  |
| commercial EPA2 Spring | 1.13 | 1.09 | 1.13 | 1.13 | 1.00 | 1.08 |  |
| commercial EPA2 Summer | 0.78 | 0.93 | 0.99 | 1.19 | 1.00 | 1.13 |  |
| commercial EPA2 Fall | 0.91 | 1.71 | 1.03 | 0.90 | 1.27 | 1.00 |  |
| institutional EPA2 Winter | 1.11 |  | 0.74 | 0.80 | 0.96 | 7.07 |  |
| institutional EPA1 Spring | 1.18 |  | 1.03 | 0.55 | 1.20 | 1.17 |  |
| institutional EPA2 Summer | 0.93 |  | 1.55 | 0.94 | 1.28 | 1.00 |  |
| institutional EPA2 Fall | 1.11 |  | 0.74 | 0.80 | 0.96 | 7.07 |  |
| industrial EPA2 Winter | 1.15 | 0.31 | 0.74 | 0.84 | 0.80 | 0.78 |  |
| industrial EPA2 Spring | 1.05 | 0.50 | 1.07 | 1.23 | 1.10 | 0.93 |  |
| industrial EPA2 Summer | 0.78 | 4.56 | 1.54 | 1.15 | 1.05 | 0.98 |  |
| industrial EPA2 Fall | 0.96 | 2.43 | 0.93 | 0.76 | 1.15 | 1.11 |  |
| freeway EPA2 Winter | 2.28 | 0.91 | 1.41 | 1.16 | 1.06 | 1.04 |  |
| freeway EPA2 Spring | 0.82 | 3.48 | 1.08 | 1.05 | 0.81 | 0.29 |  |
| freeway EPA2 Summer | 1.24 | 0.99 |  | 0.88 |  |  |  |
| freeway EPA2 Fall | 0.78 | 1.27 | 0.68 | 1.14 | 1.00 | 10.90 |  |
| open space EPA2 Winter | 0.50 | 1.02 | 0.41 | 0.63 | 6.00 | 0.65 |  |
| open space EPA2 Spring | 0.55 | 2.49 | 1.51 | 1.10 | 0.33 | 1.81 |  |
| open space EPA2 Summer | 1.52 | 1.46 | 1.47 | 1.11 | 3.00 | 0.37 |  |
| open space EPA2 Fall | 2.00 | 0.66 |  | 0.71 | 0.83 | 0.66 |  |

| ratio of medians (seasonal to annual value) | Cadmium Filtered (ug/L) | Cadmium part strength (ug/kg) | Chromium Filtered (ug/L) | Chromium part strength (ug/kg) | Copper Filtered (ug/L) | Copper part strength (ug/kg) | Lead Filtered After 1984 (ug/L) | Lead part strength (ug/kg) | Zinc Filtered (ug/L) | Zinc part strength (ug/kg) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| resid EPA2 Winter | 0.80 |  | 0.75 | 0.07 | 0.84 | 1.03 | 0.80 | 0.98 | 0.95 | 0.96 |
| resid EPA2 Spring | 1.59 | 1.00 | 1.13 | 1.16 | 1.42 | 1.22 | 0.60 |  | 1.14 | 1.52 |
| resid EPA2 Summer | 0.67 |  |  |  | 0.98 | 0.94 | 0.90 | 1.15 | 0.93 | 1.44 |
| resid EPA2 Fall |  |  |  |  | 1.32 | 0.65 | 3.20 | 0.57 | 1.18 | 0.54 |
| commercial EPA2 Winter |  |  |  |  | 1.19 | 0.68 | 1.25 | 0.75 | 1.13 | 1.03 |
| commercial EPA2 Spring |  |  | 0.74 | 1.95 | 1.81 | 0.88 | 0.63 | 0.80 | 1.55 | 0.83 |
| commercial EPA2 Summer | 1.00 | 1.00 |  |  | 0.59 | 1.27 | 0.75 | 3.58 | 0.49 | 1.43 |
| commercial EPA2 Fall |  |  | 1.26 | 0.05 | 0.93 | 1.48 | 1.25 | 1.00 | 1.04 | 0.18 |

## Seasonal Ratios for EPA Rain Zone 3 (Southeast)

| ratio of medians (seasonal to annual value) | TSS (mg/L) | Fecal Coliform (#/100 mL) | Ammonia (mg/L) | N02+NO3 (mg/L as N) | Phosphorous Filtered as P (mg/L) | Phos part strength (mg/kg) |
| --- | --- | --- | --- | --- | --- | --- |
| resid EPA3 Winter | 1.83 | 0.66 | 3.17 | 0.89 | 0.69 | 0.73 |
| resid EPA3 Spring | 0.96 | 0.46 | 1.00 | 1.76 | 1.39 | 1.07 |
| resid EPA3 Summer | 0.83 | 5.01 | 0.83 | 2.02 | 1.09 | 1.01 |
| resid EPA3 Fall | 0.87 | 6.09 |  | 0.72 | 1.09 | 1.28 |
| commercial EPA3 Winter | 0.99 | 1.00 | 2.62 | 0.34 | 1.10 | 1.41 |
| commercial EPA3 Spring | 2.22 | 0.30 | 0.76 | 5.25 | 0.45 | 0.60 |
| commercial EPA3 Summer | 0.64 | 3.23 | 0.90 | 1.00 | 0.92 | 0.80 |
| commercial EPA3 Fall | 1.19 |  |  |  | 1.85 | 2.10 |
| institutional EPA3 Winter |  |  |  | 0.71 |  |  |
| institutional EPA3 Spring |  |  |  | 0.90 |  |  |
| institutional EPA3 Summer |  |  |  | 1.23 |  |  |
| institutional EPA3 Fall |  |  |  | 0.61 |  |  |
| industrial EPA3 Winter | 0.77 | 0.10 | 1.13 | 1.36 | 1.11 | 0.88 |
| industrial EPA3 Spring | 1.67 | 0.62 | 0.80 | 2.17 | 0.82 | 1.79 |
| industrial EPA3 Summer | 0.80 | 3.90 | 0.90 | 1.49 | 0.71 | 1.47 |
| industrial EPA3 Fall | 1.29 | 0.54 | 1.67 | 0.96 | 1.14 | 1.26 |
| freeway EPA3 Winter | 0.73 |  |  |  | 0.99 | 1.45 |
| freeway EPA3 Spring | 1.01 |  |  |  | 5.20 | 1.33 |
| freeway EPA3 Summer | 1.70 |  |  |  | 0.87 | 0.79 |
| freeway EPA3 Fall | 0.94 |  |  |  | 1.07 | 1.00 |

| ratio of medians (seasonal to annual value) | Cadmium Filtered (ug/L) | Cadmium part strength (ug/kg) | Copper Filtered (ug/L) | Copper part strength (ug/kg) | Lead Filtered After 1984 (ug/L) | Lead part strength (ug/kg) | Zinc Filtered (ug/L) | Zinc part strength (ug/kg) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| commercial EPA3 Winter |  |  | 1.00 | 1.27 | 1.00 | 1.00 | 0.67 | 1.00 |
| commercial EPA3 Spring |  |  | 0.48 |  |  |  | 0.78 | 0.47 |
| commercial EPA3 Summer | 0.37 | 0.28 | 0.69 | 0.89 |  |  | 1.39 | 1.16 |
| commercial EPA3 Fall | 1.63 | 1.72 | 4.48 |  |  |  | 5.28 | 2.86 |

## Seasonal Ratios for EPA Rain Zone 4 (Lower Mississippi)

| ratio of medians (seasonal to annual value) | TSS (mg/L) | Fecal Coliform (#/100 mL) | Ammonia (mg/L) | N02+NO3 (mg/L as N) | Phosphorous Filtered as P (mg/L) | Phos part strength (mg/kg) | Phosphate Ortho as P (mg/L) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| resid EPA4 Winter | 0.67 | 0.27 | 1.13 | 0.82 | 0.66 | 0.65 |  |
| resid EPA4 Spring | 1.36 | 0.27 | 1.55 | 1.00 | 1.05 | 1.03 |  |
| resid EPA4 Summer | 0.88 | 1.49 | 0.54 | 1.12 | 1.04 | 1.00 | 3.20 |
| resid EPA4 Fall | 1.17 | 1.78 | 1.46 | 0.96 | 1.04 | 5.56 | 0.90 |
| commercial EPA4 Winter | 1.00 | 0.15 | 1.00 | 0.86 | 1.00 | 0.83 |  |
| commercial EPA4 Spring | 1.08 | 0.40 |  | 0.63 | 0.59 | 1.43 | 0.67 |
| commercial EPA4 Summer | 0.95 | 1.71 | 1.50 | 1.17 | 0.91 | 2.00 | 1.24 |
| commercial EPA4 Fall | 0.64 | 1.80 |  | 1.00 | 1.27 | 0.69 | 1.33 |
| institutional EPA4 Winter |  |  |  |  |  |  |  |
| institutional EPA4 Spring | 1.67 |  |  |  |  |  |  |
| institutional EPA4 Summer | 0.93 |  |  |  |  |  |  |
| institutional EPA4 Fall | 0.87 |  |  |  |  |  | 1.00 |
| industrial EPA4 Winter | 1.22 | 0.68 | 2.00 | 1.16 | 1.38 | 0.47 |  |
| industrial EPA4 Spring | 1.66 | 0.61 | 0.54 | 1.01 | 0.58 | 0.78 | 4.28 |
| industrial EPA4 Summer | 0.97 | 1.24 | 1.19 | 1.31 | 1.31 | 1.70 | 1.00 |
| industrial EPA4 Fall | 0.91 | 6.29 | 0.00 | 0.78 | 0.77 | 1.06 | 0.22 |
| open space EPA4 Winter | 1.21 | 0.28 | 0.91 | 0.58 | 2.34 | 1.50 |  |
| open space EPA4 Spring | 2.75 | 0.27 |  | 0.55 | 1.19 | 0.27 |  |
| open space EPA4 Summer | 1.65 | 1.51 | 1.09 | 1.54 | 0.44 | 0.92 |  |
| open space EPA4 Fall | 0.28 | 2.60 |  | 2.10 | 1.00 | 1.06 |  |

## Seasonal Ratios for EPA Rain Zone 5 (most of Texas and some surrounding areas)

| ratio of medians (seasonal to annual value) | TSS (mg/L) | Fecal Coliform (#/100 mL) | Ammonia (mg/L) | N02+NO3 (mg/L as N) | Phosphorous Filtered as P (mg/L) | Phos part strength (mg/kg) |
| --- | --- | --- | --- | --- | --- | --- |
| resid EPA5 Winter | 0.78 | 0.34 |  | 0.89 | 0.78 | 0.87 |
| resid EPA5 Spring | 1.03 | 0.97 |  | 1.11 | 1.04 | 0.98 |
| resid EPA5 Summer | 1.09 | 5.86 |  | 1.07 | 0.90 | 0.35 |
| resid EPA5 Fall | 0.97 | 1.64 |  | 0.83 | 1.15 | 1.17 |
| commercial EPA5 Winter | 0.99 | 0.24 |  | 1.00 | 0.60 | 0.98 |
| commercial EPA5 Spring | 0.81 | 1.01 |  | 1.06 | 1.20 | 1.03 |
| commercial EPA5 Summer |  |  |  |  |  |  |
| commercial EPA5 Fall | 1.80 | 1.86 |  | 0.77 | 1.36 | 0.96 |
| industrial EPA5 Winter | 0.97 | 0.68 |  | 1.01 | 1.20 | 1.33 |
| industrial EPA5 Spring | 0.96 | 0.39 |  | 1.06 | 1.00 | 0.88 |
| industrial EPA5 Summer | 0.62 | 2.53 |  | 1.03 | 0.96 | 0.56 |
| industrial EPA5 Fall | 1.12 | 1.75 |  | 0.91 | 1.17 | 0.86 |
| open space EPA5 Winter | 1.77 | 0.32 |  | 1.03 | 1.00 | 1.00 |
| open space EPA5 Spring | 0.98 | 1.84 |  | 0.63 | 1.00 | 1.94 |
| open space EPA5 Summer | 0.84 | 32.43 |  | 1.05 | 1.00 | 0.28 |
| open space EPA5 Fall | 2.33 | 5.05 |  | 1.62 | 5.05 | 1.41 |

## Seasonal Ratios for EPA Rain Zone 6 (Southwest)

| ratio of medians (seasonal to annual value) | TSS (mg/L) | Fecal Coliform (#/100 mL) | Ammonia (mg/L) | N02+NO3 (mg/L as N) | Phosphorous Filtered as P (mg/L) | Phos part strength (mg/kg) | Phosphate Ortho as P (mg/L) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| resid EPA6 Winter | 0.98 | 2.11 | 0.85 | 0.85 | 0.91 | 0.66 | 0.71 |
| resid EPA6 Spring | 0.84 | 0.24 | 0.91 | 0.89 | 0.83 | 1.21 | 0.98 |
| resid EPA6 Summer | 1.16 | 0.18 | 2.99 | 1.14 | 1.83 | 3.33 | 1.33 |
| resid EPA6 Fall | 1.63 | 0.20 | 1.89 | 1.00 | 2.17 | 1.10 | 1.10 |
| commercial EPA6 Winter | 1.43 | 1.48 | 0.84 | 0.65 | 0.63 | 0.82 | 0.52 |
| commercial EPA6 Spring | 1.16 | 1.52 | 0.20 | 0.91 | 0.75 | 1.12 | 0.83 |
| commercial EPA6 Summer | 1.44 | 0.52 | 1.63 | 1.27 | 1.63 | 0.80 | 1.34 |
| commercial EPA6 Fall | 0.35 | 1.33 | 0.81 | 0.95 | 1.28 | 2.73 | 0.79 |
| institutional EPA6 Winter | 1.12 |  | 0.83 |  | 0.90 | 0.64 |  |
| institutional EPA6 Spring | 0.59 |  | 1.07 |  | 0.75 | 1.73 |  |
| institutional EPA6 Summer |  |  |  |  |  |  |  |
| institutional EPA6 Fall | 1.23 |  | 1.52 |  | 2.44 | 1.00 |  |
| industrial EPA6 Winter | 0.77 | 1.61 | 0.59 | 0.71 | 0.70 | 1.01 | 0.79 |
| industrial EPA6 Spring | 1.05 | 0.89 | 0.77 | 0.88 | 0.75 | 0.73 | 1.04 |
| industrial EPA6 Summer | 1.79 | 0.92 | 2.09 | 1.24 | 1.28 | 1.53 | 0.95 |
| industrial EPA6 Fall | 1.05 | 1.02 | 1.70 | 0.94 | 1.65 | 1.15 | 1.63 |
| freeway EPA6 Winter | 1.03 | 0.76 | 0.75 |  | 1.00 | 0.83 | 0.89 |
| freeway EPA6 Spring | 0.87 | 1.18 | 0.83 |  | 0.78 | 0.71 | 1.11 |
| freeway EPA6 Summer |  |  |  |  |  |  |  |
| freeway EPA6 Fall | 0.72 | 7.65 | 1.30 |  | 1.87 | 3.09 | 3.67 |
| open space EPA6 Winter | 2.18 | 3.90 | 1.12 |  | 1.14 | 0.52 |  |
| open space EPA6 Spring | 0.45 | 1.00 | 0.79 |  | 0.87 | 2.74 |  |
| open space EPA6 Summer | 2.05 | 0.47 | 3.36 |  |  |  |  |
| open space EPA6 Fall | 0.50 | 0.44 | 1.56 |  | 0.86 | 4.04 |  |

| ratio of medians (seasonal to annual value) | Cadmium Filtered (ug/L) | Cadmium part strength (ug/kg) | Chromium Filtered (ug/L) | Chromium part strength (ug/kg) | Copper Filtered (ug/L) | Copper part strength (ug/kg) | Iron, Filtered as Fe (ug/L) | Iron part strength (ug/kg) | Lead Filtered After 1984 (ug/L) | Lead part strength (ug/kg) | Nickel Filtered (ug/L) | Nickel part strength (ug/kg) | Zinc Filtered (ug/L) | Zinc part strength (ug/kg) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| resid EPA6 Winter |  |  | 0.73 |  | 0.89 | 0.82 | 1.02 | 1.90 | 0.71 | 1.00 | 0.83 | 16.62 | 1.00 | 0.63 |
| resid EPA6 Spring | 0.93 | 1.00 | 1.05 |  | 0.74 | 1.21 | 0.79 | 0.43 |  |  | 0.75 | 1.07 | 0.84 | 1.72 |
| resid EPA6 Summer |  |  |  |  | 2.18 | 1.47 |  |  | 2.69 |  |  |  | 3.80 |  |
| resid EPA6 Fall | 2.40 | 0.00 | 1.47 |  | 1.60 | 1.41 | 1.49 | 1.77 | 1.08 |  | 1.14 | 0.93 | 1.99 | 2.02 |
| commercial EPA6 Winter | 1.00 |  |  |  | 1.18 | 16.60 | 1.84 | 1.00 | 0.35 |  |  |  | 0.66 | 0.31 |
| commercial EPA6 Spring |  |  |  |  | 0.91 | 0.74 | 0.67 | 1.16 | 1.00 |  | 1.00 | 1.00 | 1.15 | 1.51 |
| commercial EPA6 Summer |  |  |  |  | 0.91 |  | 1.81 | 0.71 | 1.42 |  |  |  | 1.42 |  |
| commercial EPA6 Fall |  |  |  |  | 1.09 | 2.67 | 1.00 | 10.68 | 0.78 | 1.00 |  |  | 2.84 | 2.83 |
| institutional EPA6 Winter | 0.85 | 4.56 |  |  | 0.94 | 0.81 | 1.22 | 1.26 |  |  | 0.99 | 1.50 | 0.77 | 0.74 |
| institutional EPA6 Spring |  |  |  |  | 1.17 | 2.57 | 0.70 | 0.34 |  |  | 1.11 | 1.41 | 1.00 | 1.52 |
| institutional EPA6 Summer |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| institutional EPA6 Fall | 1.42 | 0.18 | 1.00 |  | 1.60 | 1.44 | 1.99 | 5.66 | 1.00 | 1.00 | 1.17 | 0.34 | 1.40 | 1.34 |
| industrial EPA6 Winter | 1.63 | 1.34 | 1.00 | 1.45 | 0.90 | 1.28 | 1.00 | 1.00 | 1.00 | 0.98 | 1.50 | 1.00 | 1.47 | 1.43 |
| industrial EPA6 Spring | 0.41 | 0.60 | 1.50 | 0.68 | 1.00 | 0.86 |  |  | 0.08 | 1.02 | 1.00 | 1.12 | 0.92 | 0.93 |
| industrial EPA6 Summer |  |  |  |  | 2.00 | 0.51 |  |  | 3.94 |  |  |  | 0.12 | 0.56 |
| industrial EPA6 Fall |  |  |  |  | 1.00 |  |  |  | 2.21 |  |  |  | 0.63 | 0.00 |
| freeway EPA6 Winter | 0.30 | 1.17 | 0.97 | 1.00 | 0.88 | 0.85 | 1.60 | 1.35 | 0.75 | 1.00 | 0.71 | 0.89 | 0.83 | 1.03 |
| freeway EPA6 Spring | 2.41 | 1.25 | 0.77 | 1.15 | 1.21 | 1.16 |  |  | 0.86 | 1.14 | 0.96 | 0.95 | 0.94 | 0.95 |
| freeway EPA6 Summer |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| freeway EPA6 Fall | 1.07 | 0.77 | 1.35 | 0.69 | 2.33 | 0.78 | 1.00 | 0.59 | 2.31 | 0.71 | 2.67 | 1.18 | 2.08 | 0.92 |
| open space EPA6 Winter |  |  |  |  | 0.67 | 0.73 | 0.90 | 0.38 |  |  |  |  |  |  |
| open space EPA6 Spring |  |  |  |  | 1.00 | 1.36 | 1.14 | 4.54 |  |  |  |  |  |  |
| open space EPA6 Summer |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| open space EPA6 Fall |  |  |  |  | 2.78 |  | 0.82 | 2.51 |  |  |  |  |  |  |

## Seasonal Ratios for EPA Rain Zone 7 (Coastal Northwest)

| ratio of medians (seasonal to annual value) | TSS (mg/L) | Fecal Coliform (#/100 mL) | Ammonia (mg/L) | N02+NO3 (mg/L as N) | Phosphorous Filtered as P (mg/L) | Phos part strength (mg/kg) | Phosphate Ortho as P (mg/L) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| resid EPA7 Winter | 0.98 | 0.54 | 0.65 | 1.29 | 0.53 | 1.00 | 0.70 |
| resid EPA7 Spring | 0.90 | 0.78 | 0.57 | 0.97 | 0.55 | 1.70 | 0.60 |
| resid EPA7 Summer | 1.71 | 1.22 |  | 0.47 |  |  | 0.40 |
| resid EPA7 Fall | 0.92 | 1.66 | 1.67 | 0.57 | 8.55 | 0.76 | 1.70 |
| commercial EPA7 Winter | 1.39 | 0.72 | 0.40 | 1.40 | 1.38 | 0.80 | 1.00 |
| commercial EPA7 Spring | 0.68 | 1.37 | 1.20 | 0.96 | 0.57 | 1.48 | 1.70 |
| commercial EPA7 Summer | 4.40 | 1.82 |  | 0.60 |  |  | 0.40 |
| commercial EPA7 Fall | 0.75 | 1.58 | 1.07 | 1.28 | 2.95 | 0.69 | 0.85 |
| industrial EPA7 Winter | 0.90 | 0.40 | 0.90 | 0.88 | 1.59 | 0.64 | 1.42 |
| industrial EPA7 Spring | 0.95 | 4.75 | 1.00 | 0.52 | 0.41 | 1.36 | 0.15 |
| industrial EPA7 Summer | 1.14 | 1.00 | 3.20 | 1.90 |  |  | 1.85 |
| industrial EPA7 Fall | 1.18 | 1.65 | 0.70 | 1.03 |  |  | 0.46 |
| freeway EPA7 Winter | 1.04 | 0.23 | 1.00 | 1.43 |  |  | 1.10 |
| freeway EPA7 Spring | 2.30 | 1.73 | 1.21 | 0.61 |  |  | 0.40 |
| freeway EPA7 Summer | 2.41 | 2.14 | 1.33 | 5.36 |  |  | 0.40 |
| freeway EPA7 Fall | 0.55 | 1.32 | 1.17 | 0.82 |  |  | 1.20 |

| ratio of medians (seasonal to annual value) | Cadmium Filtered (ug/L) | Cadmium part strength (ug/kg) | Chromium Filtered (ug/L) | Chromium part strength (ug/kg) | Copper Filtered (ug/L) | Copper part strength (ug/kg) | Lead Filtered After 1984 (ug/L) | Lead part strength (ug/kg) | Nickel Filtered (ug/L) | Nickel part strength (ug/kg) | Zinc Filtered (ug/L) | Zinc part strength (ug/kg) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| resid EPA7 Winter | 1.00 | 1.00 | 1.00 | 1.00 | 0.70 | 0.83 | 3.00 | 0.86 | 1.00 | 0.79 | 1.00 | 1.00 |
| resid EPA7 Spring |  |  | 1.00 | 1.17 | 1.00 | 0.86 | 1.00 | 0.91 | 1.00 | 2.04 | 0.68 | 1.24 |
| resid EPA7 Summer |  |  | 0.90 | 0.83 | 0.80 | 1.07 | 0.50 | 1.71 |  |  | 0.28 | 0.92 |
| resid EPA7 Fall |  |  | 2.50 | 2.37 | 1.60 | 1.59 | 1.00 | 2.28 | 1.50 | 1.31 | 1.25 | 0.98 |
| commercial EPA7 Winter | 0.83 | 1.00 | 1.00 | 0.99 | 0.75 | 0.65 | 1.50 | 1.15 | 1.20 | 1.03 | 0.98 | 0.58 |
| commercial EPA7 Spring | 1.20 |  | 0.50 | 1.00 | 1.67 | 1.96 | 0.50 | 0.48 | 0.80 | 1.06 | 0.96 | 1.16 |
| commercial EPA7 Summer |  |  | 0.50 | 1.16 | 1.00 | 2.12 | 0.75 | 2.08 |  |  | 0.48 | 1.78 |
| commercial EPA7 Fall | 3.00 |  | 1.50 | 0.86 | 2.00 | 1.00 | 0.85 | 1.08 | 1.20 | 0.89 | 1.67 | 1.03 |
| industrial EPA7 Winter | 2.25 | 0.73 | 0.88 | 0.76 | 1.14 | 1.04 | 2.00 | 1.13 | 1.00 | 0.63 | 0.80 | 0.96 |
| industrial EPA7 Spring | 0.63 | 1.07 | 1.75 | 1.64 | 0.64 | 0.69 | 1.00 | 0.80 | 3.64 | 1.22 | 0.80 | 0.87 |
| industrial EPA7 Summer |  |  | 1.50 | 0.73 | 1.86 | 1.26 | 0.50 | 0.69 | 3.86 | 0.66 | 1.38 | 1.07 |
| industrial EPA7 Fall | 1.00 | 1.04 | 1.88 | 6.99 | 0.79 | 1.22 | 0.60 | 1.21 | 1.64 | 1.29 | 0.92 | 1.00 |
| freeway EPA7 Winter | 1.33 | 0.94 | 1.00 | 1.00 | 0.75 | 0.78 | 1.09 | 1.07 | 0.57 | 0.71 | 0.90 | 0.93 |
| freeway EPA7 Spring | 1.00 | 0.89 | 0.67 | 1.22 | 0.83 | 0.75 | 1.74 | 0.74 | 0.86 | 0.86 | 1.10 | 1.02 |
| freeway EPA7 Summer | 0.83 | 1.14 | 1.67 | 0.90 | 4.67 | 1.65 | 1.74 | 1.06 | 5.43 | 1.50 | 5.67 | 1.08 |
| freeway EPA7 Fall | 1.00 | 3.98 | 1.17 | 0.75 | 1.17 | 1.12 | 0.87 | 0.97 | 1.14 | 1.14 | 1.00 | 1.33 |

## Seasonal Ratios for EPA Rain Zone 8 (Interior Northwest)

| ratio of medians (seasonal to annual value) | TSS (mg/L) | Fecal Coliform (#/100 mL) | Ammonia (mg/L) | N02+NO3 (mg/L as N) | Phosphorous Filtered as P (mg/L) | Phos part strength (mg/kg) | Phosphate Ortho as P (mg/L) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| resid EPA8 Winter |  |  |  |  |  |  |  |
| resid EPA8 Spring | 0.96 | 0.18 | 0.90 | 1.00 |  |  | 0.96 |
| resid EPA8 Summer | 0.87 | 6.67 | 1.19 |  |  |  | 2.88 |
| resid EPA8 Fall | 1.66 | 1.21 | 1.22 |  |  |  | 1.00 |

| ratio of medians (seasonal to annual value) | Copper Filtered (ug/L) | Copper part stremgth (ug/kg) | Nickel Filtered (ug/L) | Nickel part strength (ug/kg) | Zinc Filtered (ug/L) | Zinc part strength (ug/kg) |
| --- | --- | --- | --- | --- | --- | --- |
| resid EPA8 Winter |  |  |  |  |  |  |
| resid EPA8 Spring | 0.39 | 1.00 | 1.00 | 1.00 | 0.91 | 0.90 |
| resid EPA8 Summer | 1.61 |  |  |  | 2.62 | 1.00 |
| resid EPA8 Fall |  |  |  |  | 0.82 |  |

## Seasonal Ratios for EPA Rain Zone 9 (Northern Plains)

| ratio of medians (seasonal to annual value) | TSS (mg/L) | Fecal Coliform (#/100 mL) | Ammonia (mg/L) | N02+NO3 (mg/L as N) | Phosphorous Filtered as P (mg/L) | Phos part strength (mg/kg) | Phosphate Ortho as P (mg/L) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| resid EPA9 Winter | 0.35 |  |  |  |  |  |  |
| resid EPA9 Spring | 0.92 | 0.46 | 0.90 | 0.85 | 1.06 | 0.97 | 0.78 |
| resid EPA9 Summer | 1.22 | 1.13 | 1.03 | 0.98 | 1.00 | 1.17 | 1.15 |
| resid EPA9 Fall | 0.77 |  |  | 1.91 | 1.34 | 1.03 | 0.43 |
| commercial EPA9 Winter | 0.46 |  |  |  |  |  |  |
| commercial EPA9 Spring | 0.74 | 0.04 | 1.00 | 0.79 | 0.85 | 1.00 | 1.36 |
| commercial EPA9 Summer | 1.32 | 1.77 | 0.81 | 1.06 | 0.78 | 0.85 | 0.89 |
| commercial EPA9 Fall | 0.34 |  | 0.00 |  | 2.33 | 2.17 | 1.02 |
| institutional EPA9 Winter |  |  |  |  |  |  |  |
| institutional EPA9 Spring | 1.45 |  |  | 0.99 | 0.83 | 0.92 | 1.27 |
| institutional EPA9 Summer | 1.18 |  |  | 0.99 | 1.00 | 0.95 | 0.90 |
| institutional EPA9 Fall | 0.20 |  |  | 1.28 | 2.17 | 2.16 | 0.93 |
| industrial EPA9 Winter | 0.72 |  |  |  |  |  |  |
| industrial EPA9 Spring | 2.29 | 0.52 | 1.13 | 0.72 | 2.88 | 1.03 |  |
| industrial EPA9 Summer | 1.00 | 5.19 | 1.00 | 1.00 | 0.92 | 1.00 |  |
| industrial EPA9 Fall | 2.00 |  |  |  |  |  |  |
| open space EPA9 Winter |  |  |  |  |  |  |  |
| open space EPA9 Spring | 0.96 |  | 0.82 |  | 0.80 | 1.00 |  |
| open space EPA9 Summer | 3.37 |  | 1.05 |  | 1.40 | 0.67 |  |
| open space EPA9 Fall |  |  |  |  |  |  |  |

# Conclusions

EPA Rain Zone, the interaction between EPA Rain Zone and land use, and land use alone, were all statistically significant when examining variations in stormwater pollutant concentrations. However, based on the analyses, season alone is not a significant factor. Yet, the interaction between season and EPA Rain Zone is significant for all the standard pollutants, except for TSS and total lead. For land use categories, the variability between sampling locations is greater than between land uses, which justifies the need to examine many sampling sites at one geographical area. Therefore, seasonal variations should be considered when examining a land use at a specific location, if the data are available (the 3-way interaction was significant for many of the pollutants).

Detailed analyses examined total phosphorus concentrations contained in the NSQD, version 4.02. These analyses indicate that total phosphorus concentrations varied by geographical location (as reflected by the EPA Rain Zone), land use, and season. An example analysis was therefore conducted focusing only on the total phosphorus concentrations observed in the EPA Rain Zone 1 (the northeast and upper Midwest areas of the U.S.). About 1,020 observations for total phosphorus were available for this area from 69 locations in eight states (IL, IN, MA, MI, MN, NH, NY, and WI), with data collected from June 1970 through December 2003. Trend analyses of the residential area total phosphorus data did show a significant, but very small, decrease in total phosphorus concentrations with time during this sampling period. Less than a 10% decrease was found during this 34 year period. There were no significant trends in total phosphorus residential concentrations according to rain depth. Analyses of the filterable fraction of the total residential area phosphorus concentrations indicated that about 50 to 60% of the total phosphorus was in filterable forms.

As part of the preparation of this memo, sorted NSQD data by location, land use, and season allowed ratios of median seasonal values to median annual values for many of the constituents. These ratio factors can be used to adjust the WinSLAMM pollutant concentration values (for filtered pollutants and particulate strengths) by season. These seasonal ratio factors may be applied for source area pollutant values in the absence of seasonal values for source areas.

# References

American Society of Civil Engineers (ASCE), United States Environmental Protection Agency (EPA). (2002). Urban Stormwater BMP Performance Monitoring. A Guidance Manual for Meeting the National Stormwater BMP Database Requirements. Office of Water. Washington D.C.

Driver, N.E., Mustard, M.H., Rhinesmith, R.B., and Middleburg, R.F. (1985). U.S. Geological Survey Urban Stormwater Database for 22 Metropolitan Areas Throughout the United States. U.S. Geological Survey Open File Report 85-337. Denver, CO.

Maestre, A., Pitt, R. E., and Derek Williamson (2004). “Nonparametric statistical tests comparing first flush with composite samples from the NPDES Phase 1 municipal stormwater monitoring data.” Stormwater and Urban Water Systems Modeling. In: Models and Applications to Urban Water Systems, Vol. 12 (edited by W. James). CHI. Guelph, Ontario, pp. 317 – 338.

Maestre, A., R. Pitt, S.R. Durrans, and S. Chakraborti (2005). “Stormwater quality descriptions using the three parameter lognormal distribution.” Stormwater and Urban Water Systems Modeling. In: Models and More for Urban Water Systems, Monograph 13. (edited by W. James, K.N. Irvine, E.A. McBean, and R.E. Pitt). CHI. Guelph, Ontario.

Maestre, A. and R. Pitt. (2006). “Identification of significant factors affecting stormwater quality using the National Stormwater Quality Database.” In: Stormwater and Urban Water Systems Modeling, Monograph 14. (edited by W. James, K.N. Irvine, E.A. McBean, and R.E. Pitt). CHI. Guelph, Ontario, pp. 287 – 326.

Maestre, A. and R. Pitt (2007). “Stormwater databases: NURP, USGS, International BMP Database, and NSQD.” Chapter 20 in: Contemporary Modeling of Urban Water Systems, ISBN 0-9736716-3-7, Monograph 15. (edited by W. James, E.A. McBean, R.E. Pitt, and S.J. Wright). CHI. Guelph, Ontario. pp. 385 – 409.

Pitt, R. Maestre A., Morquecho R (2003a). “The National Stormwater Quality Database (NSQD, version 1.0)” Water Environment Federation Technical Exposition and Conference, Los Angeles, CA.

Pitt, R., Maestre A., and Morquecho R. (2003b). Evaluation of NPDES Phase I Municipal Stormwater Monitoring Data. In: National Conference on Urban Stormwater: Enhancing the Programs at the Local Level. EPA/625/R-03/003.

Pitt, R. E., A. Maestre, R. Morquecho, and Derek Williamson (2004). “Collection and examination of a municipal separate storm sewer system database.” Stormwater and Urban Water Systems Modeling. In: Models and Applications to Urban Water Systems, Vol. 12 (edited by W. James). CHI. Guelph, Ontario, pp. 257 – 294.

Smullen, J.T. and Cave K.A. (2002). “National stormwater runoff pollution database.” In: Wet-Weather Flow in the Urban Watershed (edited by R. Field, and D. Sullivan). Boca Raton, Florida: Lewis Publishers.

United States Environmental Protection Agency (EPA). (1983). Results of the Nationwide Runoff Program. Water Planning Division, PB 84-185552. Washington D.C.

# Appendix A: Analysis of Total Phosphorus Stormwater Monitoring Information as Contained in the National Stormwater Quality Database (NSQD), Version 4.02

## Introduction

The University of Alabama and the Center for Watershed Protection were awarded a U.S. Environmental Protection Agency, Office of Water 104(b)3 grant in 2001 to collect and evaluate stormwater data from a portion of  NPDES (National Pollutant Discharge Elimination System) MS4 (municipal separate storm sewer system) stormwater permits. Version 3 of this database was completed under continued 104(b)3 support from the EPA, while Version 4 was completed in 2015 with support from University of Alabama, Wright Water Engineers, and Geosyntec Consultants. The general approach in data collection was to contact EPA regional offices to obtain state contacts for the MS4 data, then the individual municipalities with Phase I permits were targeted for data collection. Version 4.02 of the NSQD represents major urban area land uses, such as: residential, commercial, and open space. This monitoring data, collected over nearly a fifteen-year period from more than 250 municipalities throughout the country, was used to create the National Stormwater Quality Database (NSQD). The NSQD has several applications. For example, it can characterize national stormwater quality, provide guidance for future sampling needs, enhance local stormwater management activities in areas having limited data, and be compared to historical benchmarks. The NSQD also provides recommendations for improving the quality and management of future NPDES monitoring efforts.

Only stormwater quality outfall data are included in the NSQD, representing untreated stormwater leaving the sites. Receiving water data, source area sheetflow data, and data from locations affected by stormwater control practices are not included in the database (see the International BMP Database for stormwater performance data). Other data incorporated into the NSQD comes from various sources including; Nationwide Urban Runoff Program (NURP) (US EPA, 1983), the International Best Management Practice (BMP) Database (ASCE 2002), and specialized research projects conducted by states and by the U.S. Geological Survey Urban-Stormwater Database (Smullen and Case 2002; Driver, et al. 1985). All of the data sources were carefully reviewed (in fact, most of the effort in compiling the NSQD was spent in Quality Assurance/Quality Control (QA/QC) reviews of the data). QA/QC reviews were very important to verify the correctness of data that was added to the database. The NSQD contains extensive data for most land uses and locations for a broad range of conventional pollutants, nutrients, bacteria, heavy metals, along with some organic compounds. As an example, early lead data are included separately as a special constituent for trend analyses). The early NURP data were very helpful in filling data gaps from the upper Midwest and northeastern areas of the country, and to extend the time period of the observations.

More than 9,000 stormwater monitoring events are recorded in version 4.02 of the NSQD, representing more than 600 sampling locations (median of 10 samples per site, with a maximum of 115 per site). Table 1 shows the 6 land uses, 9 EPA Rain Zones and four seasons used as data identifiers in the NSQD database. When more than one land use was present, it was considered a mixed site with the major land use noted. About 48% of the storm events stored in the database were collected in residential land use areas, followed by commercial areas with 20%, industrial areas with 13%, open space and freeways with 6% each, and institutional at 5%.

Figure 1 presents a map showing the EPA Rain Zones in the U.S., along with the locations of the communities represented in the database.

Table 1. Main Land Uses, EPA Rain Zones and Seasons Identified in the NSQD

| Major Land Uses | EPA Rain Zones | Season |
| --- | --- | --- |
| Commercial | Zone 1 – Great Lakes and Northeast | Spring: February – April |
| Freeway | Zone 2 – Mid-Atlantic | Summer: May – July |
| Industrial | Zone 3 – Southeast | Fall: August – October |
| Institutional | Zone 4 – Lower Mississippi Valley | Winter: November – January |
| Open Space | Zone 5 – Texas |  |
| Residential | Zone 6 – Southwest |  |
|  | Zone 7 – Coastal Northwest |  |
|  | Zone 8 – Interior Northwest |  |
|  | Zone 9 – Northern Plains |  |

Figure 1. Sampling Locations for Data Contained in the National Stormwater Quality Database, version 4.02, by EPA Rain Zones.

The complete NSQD, version 4.02, is available as a large (7.5Mb) Excel spreadsheet at: http://www.bmpdatabase.org/nsqd.html. This website (which also contains the International BMP database) includes a new NSQD data extraction tool to explore and assess data prior to in-depth analyses. Detailed statistical analyses, site descriptions, and early versions of the database are available in a 2005 EPA final report entitled “The National Stormwater Quality Database, Version 1.1: A Compilation and Analysis of NPDES Stormwater Monitoring Information” located at: http://unix.eng.ua.edu/~rpitt/Publications/Stormwater%20Characteristics/NSQD%20EPA.pdf

## Observations

There are many factors that may affect stormwater quality. These factors include, but are not limited to the following; land use, geographical region (EPA Rain Zone), and season. The NSQD includes sampling location information such as city, state, land use, drainage area, and EPA Rain Zone. The database also includes sampling information such as date, season, and rain depth, along with the concentrations for many constituents. Supplemental information available at each sampling location includes the exact sampling locations (street intersections or longitude/latitude), breakdown of land use by percentage, aerial photographs, topographic maps, information on sampling procedures, and quality control. It is important to note that the supplemental information available can vary for different sites.

Land use has an important impact on the quality of stormwater. For example, the concentrations of heavy metals are generally higher for industrial land use areas due to manufacturing processes and other activities that generate these materials. Seasons could also be a factor in the variation of nutrient concentrations in stormwater due to seasonal uses of fertilizers and seasonal leaf drop. Most studies also report lower bacteria concentrations in the winter than in the summer. Lead concentrations in stormwater also have significantly decreased since the elimination of lead in gasoline (as noted, the earlier NURP lead data were therefore not included in the main body of the NSQD).

Multivariate statistical analyses can be used to compare different constituent concentrations with land use and geographical location, along with different sampling methods. Some of the analyses used earlier versions of the NSQD when project resources enabled more extensive data analyses. For the most recent project efforts, most of our time was used to acquire additional data and to conduct needed quality control. More detailed discussions of the earlier NSQD results are found in various references, including Maestre, et al. (2004; 2005; 2006; 2007) and Pitt, et al. (2003 and 2004a/b). As noted above, metal concentrations are generally higher for industrial and commercial land uses, however EPA Rain Zone effects on pollutant concentrations appear to be important also. Lower concentrations for total dissolved solids (TDS), 5-day biochemical oxygen demand (BOD5), and bacteria as fecal coliforms were also observed in industrial land use areas. Fecal coliform concentrations are relatively high for residential and mixed residential land uses, and nitrate concentrations are higher for the freeway land use. Open space land use areas show consistently low concentrations for the constituents examined.

### Land use and geographical region analyses

Maestre and Pitt (2007) prepared a general linear model (GLM) that included the interaction effects between land use, geographical region, and season. The following discussion summarizes findings from this report which are relevant today. This analysis examined only a few of the constituents that were well represented in each category and included total suspended solids (TSS), BOD5, chemical oxygen demand (COD), total phosphorus (TP), nitrite (NO2)+nitrate (NO3), total Kjeldahl nitrogen (TKN), copper (Cu), lead (Pb), and zinc (Zn).

EPA Rain Zone 8 was eliminated from these analyses due to lack of data, while all four seasons were examined as they were well represented in all of the selected land uses and EPA Rain Zones. Two-way and three-way interactions were included in the model, with the results shown in Table 2. A p-value smaller than 0.05 indicates that the factor, or interaction of factors, was found to be significant at least at the 5% significance level.

The results indicated that the factors for land use, and the EPA Rain Zone, plus the interaction land use–rain zone were the most important factors used in the model. There were significant differences by EPA Rain Zone, and land use for all the constituents. None of the seasonal factors alone were significant for any constituent, except for TKN, but seasonal effects interacting with the other factors were significant for many constituents. The interaction between land uses and EPA Rain Zone is very important and must be considered when determining likely stormwater concentrations for a site.

For all the constituents except TSS, concentrations in the open space land use were lower than for the other land uses, while, samples collected at freeways and industrial sites had the highest concentrations. Total phosphorus and TKN appear to be high during the summer and low during the winter. No clear variations in concentration were observed between spring and fall. Samples collected in EPA Rain Zone 3 (the southeast) had the lowest concentrations for all the constituents. The largest concentrations were observed in EPA Rain Zone 6 (the southwest) followed by EPA Rain Zones 1 (Great Lakes and northeast) and 4 (Mississippi valley).

Table 2. P-Values for Main Factors and Interactions (Values Less Than 0.05 Are Traditionally Considered Significant)

| Constituent | Land Use (LU) | Season (SN) | EPA Rain Zone (EPA) | LU*SN | LU*EPA | SN*EPA | LU*EPA*SN |
| --- | --- | --- | --- | --- | --- | --- | --- |
| TSS mg/L | <0.0001 | 0.74 | <0.0001 | 0.017 | <0.0001 | 0.18 | <0.0001 |
| BOD mg/L | <0.0001 | 0.16 | <0.0001 | 0.0008 | <0.0001 | 0.0011 | 0.22 |
| COD mg/L | <0.0001 | 0.13 | <0.0001 | 0.034 | <0.0001 | 0.014 | 0.0085 |
| TP mg/L | <0.0001 | 0.69 | <0.0001 | 0.055 | <0.0001 | 0.0004 | <0.0001 |
| NO2+NO3 mg/L | <0.0001 | 0.11 | <0.0001 | 0.052 | <0.0001 | 0.034 | 0.057 |
| TKN mg/L | 0.0026 | 0.024 | <0.0001 | 0.99 | <0.0001 | <0.0001 | 0.17 |
| Cu µg/L | <0.0001 | 0.11 | <0.0001 | 0.62 | <0.0001 | 0.038 | 0.14 |
| Pb µg/L | <0.0001 | 0.76 | <0.0001 | 0.42 | <0.0001 | 0.28 | 0.012 |
| Zn µg/L | <0.0001 | 0.91 | <0.0001 | 0.94 | <0.0001 | 0.014 | <0.0001 |

### Evaluation of Total Phosphorus Stormwater Concentrations

This data analysis focuses on total phosphorus concentrations contained in the newest version of the NSQD (version 4.02). Figure 2 is a grouped box and whisker plot (prepared using SigmaPlot 13) comparing the concentration distributions from six major land uses. The central box indicates the 25th, 50th, and 75th percentile concentrations, while the ends of the whiskers indicate the 5th and 95th percentile values. The separate dots outside of the whiskers indicate values less than the 5th and greater than the 95th percentile values. This plot indicates that the data distributions are reasonably symmetrical and similar for each land use, with the institutional land use area generally having the lowest total phosphorus concentrations and the residential areas generally having the highest total phosphorus concentrations.

Figure 2. Box and Whisker Plots Comparing Total Phosphorus Concentrations for all Land Uses Using Data Contained in the NSQD, version 4.02.

Table 3 summarizes the Kruskal-Wallis analysis (SigmaPlot 13) results comparing the total phosphorus concentrations for each land use, as shown on the above box and whisker plot. These results indicate that at least one land use is significantly different from the others (p < 0.001).

Table 3. All Total Phosphorus Data by Land Use using Kruskal-Wallis One Way Analysis of Variance on Ranks

| Group | N | Missing | Median | 25% | 75% |
| --- | --- | --- | --- | --- | --- |
| All open space TP | 446 | 0 | 0.21 | 0.1 | 0.42 |
| All resid TP | 3855 | 0 | 0.29 | 0.17 | 0.482 |
| All instit TP | 414 | 0 | 0.14 | 0.08 | 0.223 |
| All commer TP | 1599 | 1 | 0.21 | 0.12 | 0.4 |
| All freeways TP | 378 | 0 | 0.27 | 0.17 | 0.436 |
| All indus TP | 1000 | 0 | 0.23 | 0.12 | 0.41 |
| H = 336.254 with 5 degrees of freedom. (P = <0.001) |  |  |  |  |  |
| The differences in the median values among the treatment groups are greater than would be expected by chance; there is a statistically significant difference (P = <0.001) |  |  |  |  |  |

Table 4 shows the individual comparisons indicating significant differences. Most land uses

are significantly different from each other, based on pairwise multiple comparison tests (Dunn’s test in SigmaPlot 13). The exceptions, as shown in Table 4, are open space vs. industrial; residential vs. freeways; and freeway vs. industrial land use areas. The grouped box and whisker plots also indicate potential overlapping data for these pairs, while the others are more distinctly different.

Table 4. Probability of Significant Differences Between Land Use Pairs (Dunn’s Test)

|  | open space | residential | institutional | commercial | freeway | industrial |
| --- | --- | --- | --- | --- | --- | --- |
| open space | X | <0.001 | <0.001 | <0.001 | <0.001 | 1.0 |
| residential | <0.001 | X | <0.001 | <0.001 | 1.0 | <0.001 |
| institutional | <0.001 | <0.001 | X | <0.001 | <0.001 | <0.001 |
| commercial | <0.001 | <0.001 | <0.001 | X | <0.001 | <0.001 |
| Freeway | <0.001 | 1.0 | <0.001 | <0.001 | X | 0.017 |
| industrial | 1.0 | <0.001 | <0.001 | 1.0 | 0.017 | X |

Tables 5 through 10 summarize the observed total phosphorus concentration statistical characteristics for each land use category. The overall data distribution is shown (as plotted in the box and whisker graph), along with data distributions for each EPA Rain Zone, and for each season. Not all land uses had sufficient data in all rain zones for these summaries. The coefficients of variation (COV, the ratio of the standard deviation to the average values) are relatively large, indicating wide data spreads. However, with large numbers of data in many categories, valid statistical analyses are still possible, as shown in the following discussions, especially considering the similar distribution patterns shown on the box and whisker plots.

Table 5. NSQD, version 4.02, Open Space (OS) Total Phosphorus Concentrations (mg/L as P)

|  | Overall | EPA Rain Zone |  |  |  |  |  |  | Season |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | All Open Space TP as P (mg/L) | Zone 1 OS TP as P (mg/L) | Zone 2 OS TP as P (mg/L) | Zone 3 OS TP as P (mg/L) | Zone 4 OS TP as P (mg/L) | Zone 5 OS TP as P (mg/L) | Zone 6 OS TP as P (mg/L) | Zone 9 OS TP as P (mg/L) | Fall OS TP as P (mg/L) | Winter OS TP as P (mg/L) | Spring OS TP as P (mg/L) | Summer OS TP as P (mg/L) |
| number | 446 | 78 | 103 | 91 | 18 | 67 | 75 | 14 | 106 | 128 | 93 | 119 |
| average | 0.363 | 0.290 | 0.319 | 0.471 | 1.135 | 0.373 | 0.149 | 0.506 | 0.378 | 0.371 | 0.263 | 0.421 |
| median | 0.210 | 0.187 | 0.200 | 0.355 | 0.315 | 0.250 | 0.069 | 0.470 | 0.235 | 0.160 | 0.190 | 0.330 |
| minimum | 0.005 | 0.035 | 0.005 | 0.005 | 0.005 | 0.010 | 0.025 | 0.210 | 0.025 | 0.010 | 0.010 | 0.005 |
| maximum | 15.400 | 2.500 | 2.500 | 2.855 | 15.400 | 2.290 | 2.510 | 1.000 | 2.855 | 15.400 | 1.500 | 2.500 |
| standard deviation | 0.813 | 0.361 | 0.361 | 0.460 | 3.566 | 0.413 | 0.313 | 0.260 | 0.469 | 1.385 | 0.264 | 0.404 |
| COV | 2.24 | 1.25 | 1.13 | 0.98 | 3.14 | 1.11 | 2.10 | 0.51 | 1.24 | 3.73 | 1.00 | 0.96 |

Table 6. NSQD, version 4.02, Residential Total Phosphorus Concentrations (mg/L as P)

|  | Overall | EPA Rain Zone |  |  |  |  |  |  |  |  | Season |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | All resid TP as P (mg/L) | Zone 1 resid TP as P (mg/L) | Zone 2 resid TP as P (mg/L) | Zone 3 resid TP as P (mg/L) | Zone 4 resid TP as P (mg/L) | Zone 5 resid TP as P (mg/L) | Zone 6 resid TP as P (mg/L) | Zone 7 resid TP as P (mg/L) | Zone 8 resid TP as P (mg/L) | Zone 9 resid TP as P (mg/L) | Fall resid TP as P (mg/L) | Winter resid TP as P (mg/L) | Spring resid TP as P (mg/L) | Summer resid TP as P (mg/L) |
| number | 3855 | 551 | 1865 | 304 | 122 | 206 | 228 | 290 | 15 | 274 | 943 | 785 | 1091 | 1036 |
| average | 0.427 | 0.402 | 0.427 | 0.308 | 0.611 | 0.467 | 0.410 | 0.305 | 0.851 | 0.625 | 0.408 | 0.287 | 0.454 | 0.524 |
| median | 0.290 | 0.295 | 0.280 | 0.150 | 0.357 | 0.360 | 0.310 | 0.200 | 0.696 | 0.470 | 0.290 | 0.210 | 0.300 | 0.333 |
| minimum | 0.005 | 0.005 | 0.010 | 0.010 | 0.025 | 0.080 | 0.025 | 0.010 | 0.219 | 0.071 | 0.005 | 0.005 | 0.015 | 0.010 |
| maximum | 21.200 | 6.690 | 19.900 | 21.200 | 5.330 | 4.190 | 4.960 | 3.610 | 2.950 | 6.420 | 5.330 | 4.250 | 19.900 | 21.200 |
| standard deviation | 0.710 | 0.432 | 0.750 | 1.246 | 0.774 | 0.417 | 0.429 | 0.374 | 0.712 | 0.599 | 0.431 | 0.293 | 0.797 | 0.975 |
| COV | 1.66 | 1.08 | 1.76 | 4.04 | 1.27 | 0.89 | 1.05 | 1.22 | 0.84 | 0.96 | 1.06 | 1.02 | 1.75 | 1.86 |

Table 7. NSQD, version 4.02, Institutional Total Phosphorus Concentrations (mg/L as P)

|  | Overall | EPA Rain Zone |  |  |  |  |  | Season |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | All instit TP as P (mg/L) | Zone 1 instit TP as P (mg/L) | Zone 2 instit TP as P (mg/L) | Zone 3 instit TP as P (mg/L) | Zone 4 instit TP as P (mg/L) | Zone 6 instit TP as P (mg/L) | Zone 9 instit TP as P (mg/L) | Fall instit TP as P (mg/L) | Winter instit TP as P (mg/L) | Spring instit TP as P (mg/L) | Summer instit TP as P (mg/L) |
| number | 414 | 13 | 45 | 175 | 37 | 49 | 95 | 65 | 77 | 75 | 197 |
| average | 0.197 | 0.167 | 0.237 | 0.116 | 0.112 | 0.303 | 0.308 | 0.198 | 0.175 | 0.237 | 0.190 |
| median | 0.140 | 0.150 | 0.190 | 0.103 | 0.100 | 0.230 | 0.178 | 0.140 | 0.144 | 0.176 | 0.121 |
| minimum | 0.007 | 0.026 | 0.025 | 0.007 | 0.025 | 0.115 | 0.011 | 0.010 | 0.018 | 0.011 | 0.007 |
| maximum | 2.310 | 0.370 | 0.980 | 0.449 | 0.270 | 1.020 | 2.310 | 1.020 | 0.550 | 0.780 | 2.310 |
| standard deviation | 0.219 | 0.093 | 0.194 | 0.077 | 0.064 | 0.191 | 0.355 | 0.185 | 0.118 | 0.194 | 0.263 |
| COV | 1.11 | 0.56 | 0.82 | 0.66 | 0.57 | 0.63 | 1.15 | 0.93 | 0.67 | 0.82 | 1.39 |

Table 8. NSQD, version 4.02 Commercial Total Phosphorus Concentrations (mg/L as P)

|  | Overall | EPA Rain Zone |  |  |  |  |  |  |  |  | Season |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | All commercial TP as P (mg/L) | Zone 1 commer TP as P (mg/L) | Zone 2 commer TP as P (mg/L) | Zone 3 commer TP as P (mg/L) | Zone 4 commer TP as P (mg/L) | Zone 5 commer TP as P (mg/L) | Zone 6 commer TP as P (mg/L) | Zone 7 commer TP as P (mg/L) | Zone 8 commer TP as P (mg/L) | Zone 9 comer TP as P (mg/L) | Fall commer TP as P (mg/L) | Winter commer TP as P (mg/L) | Spring commer TP as P (mg/L) | Summer commer TP as P (mg/L) |
| number | 1598 | 293 | 613 | 698 | 139 | 111 | 84 | 61 | 7 | 178 | 376 | 294 | 419 | 452 |
| average | 0.376 | 0.240 | 0.364 | 0.441 | 0.288 | 0.600 | 0.460 | 0.398 | 0.568 | 0.373 | 0.391 | 0.328 | 0.365 | 0.388 |
| median | 0.210 | 0.140 | 0.230 | 0.220 | 0.160 | 0.160 | 0.375 | 0.280 | 0.497 | 0.230 | 0.217 | 0.193 | 0.210 | 0.200 |
| minimum | 0.005 | 0.022 | 0.005 | 0.005 | 0.025 | 0.010 | 0.025 | 0.005 | 0.163 | 0.008 | 0.010 | 0.005 | 0.008 | 0.010 |
| maximum | 15.600 | 8.600 | 6.720 | 15.600 | 3.550 | 15.600 | 2.000 | 3.300 | 1.080 | 6.300 | 15.600 | 3.670 | 8.600 | 10.300 |
| standard deviation | 0.701 | 0.534 | 0.479 | 0.891 | 0.464 | 1.874 | 0.353 | 0.493 | 0.350 | 0.601 | 0.900 | 0.443 | 0.654 | 0.715 |
| COV | 1.86587 | 2.22 | 1.32 | 2.02 | 1.61 | 3.12 | 0.77 | 1.24 | 0.62 | 1.61 | 2.30 | 1.35 | 1.79 | 1.84 |

Table 9. NSQD, version 4.02, Freeways Total Phosphorus Concentrations (mg/L as P)

|  | Overall | EPA Rain Zone |  |  |  |  |  |  | Season |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | All freeways TP as P (mg/L) | Zone 1 free TP as P (mg/L) | Zone 2 free TP as P (mg/L) | Zone 3 free TP as P (mg/L) | Zone 5 free TP as P (mg/L) | Zone 6 free TP as P (mg/L) | Zone 7 free TP as P (mg/L) | Zone 9 free TP as P (mg/L) | Fall free TP as P (mg/L) | Winter free TP as P (mg/L) | Spring free TP as P (mg/L) | Summer free TP as P (mg/L) |
| number | 378 | 3 | 90 | 14 | 14 | 211 | 24 | 22 | 74 | 151 | 108 | 45 |
| average | 0.406 | 0.433 | 0.317 | 0.165 | 0.198 | 0.477 | 0.347 | 0.425 | 0.468 | 0.366 | 0.466 | 0.292 |
| median | 0.270 | 0.410 | 0.240 | 0.125 | 0.125 | 0.298 | 0.280 | 0.315 | 0.365 | 0.250 | 0.250 | 0.260 |
| minimum | 0.015 | 0.353 | 0.020 | 0.071 | 0.020 | 0.015 | 0.110 | 0.100 | 0.015 | 0.025 | 0.020 | 0.070 |
| maximum | 7.191 | 0.537 | 3.030 | 0.459 | 0.750 | 7.191 | 0.900 | 2.600 | 2.600 | 2.600 | 7.191 | 0.910 |
| standard deviation | 0.558 | 0.094 | 0.345 | 0.122 | 0.204 | 0.676 | 0.198 | 0.527 | 0.425 | 0.353 | 0.877 | 0.194 |
| COV | 1.38 | 0.22 | 1.09 | 0.74 | 1.03 | 1.42 | 0.57 | 1.24 | 0.91 | 0.97 | 1.88 | 0.66 |

Table 10. NSQD, version 4.02, Industrial Total Phosphorus Concentrations (mg/L as P)

|  | Overall | EPA Rain Zone |  |  |  |  |  |  |  |  | Season |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | All indus TP as P (mg/L) | Zone 1 indus TP as P (mg/L) | Zone 2 indus TP as P (mg/L) | Zone 3 indus TP as P (mg/L) | Zone 4 indus TP as P (mg/L) | Zone 5 indus TP as P (mg/L) | Zone 6 indus TP as P (mg/L) | Zone 7 indus TP as P (mg/L) | Zone 8 indus TP as P (mg/L) | Zone 9 indus TP as P (mg/L) | Fall indus TP as P (mg/L) | Winter indus TP as P (mg/L) | Spring indus TP as P (mg/L) | Summer indus TP as P (mg/L) |
| number | 1000 | 106 | 362 | 136 | 78 | 108 | 150 | 30 | 1 | 29 | 245 | 254 | 256 | 245 |
| average | 0.398 | 0.291 | 0.318 | 0.233 | 0.578 | 0.257 | 0.797 | 0.527 | 0.313 | 0.426 | 0.344 | 0.380 | 0.411 | 0.460 |
| median | 0.230 | 0.210 | 0.220 | 0.175 | 0.255 | 0.185 | 0.450 | 0.460 |  | 0.330 | 0.249 | 0.228 | 0.220 | 0.230 |
| minimum | 0.010 | 0.040 | 0.020 | 0.024 | 0.010 | 0.010 | 0.025 | 0.064 |  | 0.090 | 0.020 | 0.010 | 0.010 | 0.020 |
| maximum | 10.800 | 1.500 | 4.880 | 1.500 | 10.800 | 2.640 | 7.900 | 1.400 |  | 1.300 | 2.500 | 4.600 | 10.800 | 7.900 |
| standard deviation | 0.654 | 0.266 | 0.467 | 0.217 | 1.345 | 0.304 | 0.981 | 0.335 |  | 0.294 | 0.367 | 0.571 | 0.825 | 0.748 |
| COV | 1.640676 | 0.91 | 1.47 | 0.93 | 2.33 | 1.18 | 1.23 | 0.64 |  | 0.69 | 1.07 | 1.50 | 2.01 | 1.63 |

#### Trend Analyses of NSQD Residential Land Use Phosphorus Data

A time series analyses was conducted for residential total phosphorus data in the NSQD to observe any concentration trends associated with both time and rain depth. More than 3,800 total phosphorus observations are available in the NSQD, version 4.02, for residential land uses.

The Figure 3 time series plot illustrates the total phosphorus residential area concentrations by sampling data (all EPA Rain Zones and seasons combined) as contained in the NSQD (version 4.02). Stormwater nutrient concentrations, especially in residential areas, are likely affected by homeowner fertilizer use, vegetation debris, and other landscaping practices (including erosion of nutrient-rich soils). Residential area fertilizer use has likely changed over time, especially in areas having phosphate restrictions. The analysis of variance (ANOVA) on the regression coefficients (using Excel) for the very large number of observations indicated a statistically significant but very small trend (p = 0.01). Over a 31-year monitoring period, from 1979 to 2011, the total phosphorus concentrations decreased by about 10% (by 0.087 mg/L). Several hundred monitoring locations are included in this dataset, with most only active for short periods. It is expected that phosphorus trends may be more obvious if data were available for an individual location over many years, compared to this combined analysis of multiple locations. Similar analyses for the other land uses resulted in similar findings.

Figure 3. Time Series of Total Phosphorus Residential Area Concentrations (NSQD version 4.02).

The Figure 4 scatterplot compares the residential area total phosphorus concentrations plotted against the rain depths associated with these same monitored events. Typical assumptions about stormwater concentrations and rains include:  larger rains produce greater erosion and delivery of contaminants, but that the larger discharge volumes tend to dilute the runoff, resulting in lower overall concentration values. It is also sometimes assumed that smaller rains are associated with “first-flush” higher concentrations. These assumptions imply that larger concentrations are associated with smaller rains rather than larger rains.

During this 31 year period of time, the rains ranged from 0.01 to 9.28 inches, with an average rain depth of 0.72 inches (and a median depth of 0.52 inches). The rains represented in this analysis therefore include a broad range, with most events of intermediate rain depths. The ANOVA on the regression analysis (using Excel) for these data pairs indicated no significant relationship or trend of total phosphorus with changes in rain depth, even with more than 2,100 data pairs (p = 0.53).

Figure 4. Total Phosphorus Concentrations vs. Rain Depth for All Residential Land Uses from the NSQD, version 4.02.

Figures 5 and 6 and Table 11 compare the total phosphorus concentrations with concurrent filterable phosphorus concentrations (material passing through a nominal 0.45 µm membrane filter, which can include a variety of pollutant forms, including colloids and organic complexes; not all of the filterable material are in “dissolved” or ionic forms) for the residential land use area data for all locations and seasons in the NSQD, version 4.02. The filterable phosphorus portion of the total phosphorus concentration is usually much more difficult to remove using conventional stormwater management practices (which usually rely on sedimentation or filtering). It is also readily available for aquatic plant growth and more likely to cause eutrophication problems. The smaller the filterable fraction (and larger the particulate-bound fraction), the greater removal usually occurs with conventional stormwater control practices. The filterable fraction usually needs to be treated using chemical (coagulation/precipitation, ion exchange, or sorption) or biological (nutrient uptake) processes. Infiltration can also reduce phosphorus surface water discharges by capturing the phosphorus in surface soils during infiltration, as long as groundwater contamination potential is low.

The Figure 5 scatterplot of total vs. filterable phosphorus concentrations indicate that the largest ratio approaches 1 (the apparent upper line of observations along the 1:1 slope), but most are below this upper limit (a few data observations indicate filterable concentrations larger than the total concentrations reflecting minor, but common, processing and analysis errors).

Figure 5. Scatterplot of Total vs. Filterable Phosphorus From Residential Land Uses in NSQD, version 4.02.

The above scatterplot of total vs. filterable phosphorus concentrations resulted in a very significant regression relationship between these variables (p <0.001) with a slope term (the overall average ratio of filterable to total phosphorus) of 0.53. This is in good agreement with the usually assumed filterable fraction of about 50%. Table 11 summarizes the total and filterable concentrations, and their ratio for the NSQD, version 4.02, residential area observations. The median and the average ratios are both 0.57 when examining the available 1,468 data pairs.

Table 11. Total and Filterable Phosphorus Concentrations at NSQD, version 4.02, Residential Land Use Areas

|  | Total Phosphorous (mg/L as P) | Filterable Phosphorous (mg/L as P) | Ratio of Filterable to Total Phosphorus |
| --- | --- | --- | --- |
| Number | 1,468 | 1,468 | 1,468 |
| Average | 0.46 | 0.23 | 0.57 |
| Median | 0.33 | 0.16 | 0.57 |
| Minimum | 0.03 | 0.01 | 0.01 |
| Maximum | 21.2 | 19.3 | 10.7 |
| Standard deviation | 0.77 | 0.54 | 0.39 |
| COV | 1.68 | 2.34 | 0.69 |

Figure 6 is a log-normal probability plot (using Minitab 16) of the calculated filterable to total phosphorus ratios for these data, visually showing the spread of these data ratios. About 2% of the filterable concentrations were greater than the total concentrations (these are the data points in the scatterplot that are above the 1:1 slope), reflecting typical data variations associated with sample processing and analyses.

Figure 6. Log-Normal Probability Plot of Filterable to Total Phosphorus Concentrations from Residential Areas in the NSQD, version 4.02.

#### Total Phosphorus Concentration Relationships by Geographical Area and by Season for All Land Uses in the NSQD, version 4.02

More detailed geographical area and seasonal analyses were conducted focusing on residential area total phosphorus concentrations only, due to their large number of observations compared to the other land uses. The Figure 7 box and whisker plot for the different EPA rain zones shows reasonably symmetrical and similar distributions for all geographical area groups, but with larger differences between the areas. The Kruskal-Wallis analyses on Table 12 indicated at least one of the areas was significantly different from the others (p < 0.001).

Figure 7. Box and Whisker Plots for NSQD, version 4.02, Residential Total Phosphorus Concentrations by Geographical Location.

Table 12. Residential Area TP Data by EPA Rain Zone using Kruskal-Wallis One Way Analysis of Variance on Ranks

| Group | N | Missing | Median | 25% | 75% |
| --- | --- | --- | --- | --- | --- |
| Z1 res TP | 551 | 0 | 0.30 | 0.18 | 0.48 |
| Z2 res TP | 1865 | 0 | 0.28 | 0.16 | 0.48 |
| Z3 res TP | 304 | 0 | 0.15 | 0.08 | 0.28 |
| Z4 res TP | 122 | 0 | 0.36 | 0.24 | 0.68 |
| Z5 res TP | 206 | 0 | 0.36 | 0.25 | 0.54 |
| Z6 res TP | 228 | 0 | 0.31 | 0.17 | 0.52 |
| Z7 res TP | 290 | 0 | 0.2 | 0.13 | 0.31 |
| Z8 res TP | 15 | 0 | 0.70 | 0.38 | 1.03 |
| Z9 res TP | 274 | 0 | 0.47 | 0.30 | 0.75 |
| H = 356.756 with 8 degrees of freedom. (P = <0.001) |  |  |  |  |  |
| The differences in the median values among the treatment groups are greater than would be expected by chance; there is a statistically significant difference (P = <0.001) |  |  |  |  |  |

These analyses also indicated significant differences between most of the EPA rain zone areas, as indicated on the Dunn’s multi-pairwise Table 13.

Table 13. Dunn’s Multi-Pairwise Comparisons between all EPA Rain Zones for Residential Total Phosphorus Concentrations

|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | X | 1.0 | <0.001 | 0.035 | 0.004 | 1.0 | <0.001 | 0.013 | <0.001 |
| 2 | 1.0 | X | <0.001 | 0.001 | <0.001 | 1.0 | <0.001 | 0.005 | <0.001 |
| 3 | <0.001 | <0.001 | X | <0.001 | <0.001 | <0.001 | 0.063 | <0.001 | <0.001 |
| 4 | 0.035 | 0.001 | <0.001 | X | 1.0 | 0.20 | <0.001 | 0.99 | 0.61 |
| 5 | 0.004 | <0.001 | <0.001 | 1.0 | X | 0.068 | <0.001 | 0.77 | 0.11 |
| 6 | 1.0 | 1.0 | <0.001 | 0.20 | 0.068 | X | <0.001 | 0.022 | <0.001 |
| 7 | <0.001 | <0.001 | 0.063 | <0.001 | <0.001 | <0.007 | X | <0.001 | <0.001 |
| 8 | 0.013 | 0.005 | <0.001 | 0.99 | 0.77 | 0.022 | <0.001 | X | 1.0 |
| 9 | <0.001 | <0.001 | <0.001 | 0.61 | 0.11 | <0.001 | <0.001 | 1.0 | X |

Potential significant data overlaps may occur for EPA Rain Zones 1, 2, and 6; 3 and 7; and 4, 5, 8, and 9, resulting in three geographical groups for residential areas.

The Figure 8 box and whisker plot and Tables 14 and 15 show the same residential land use data separated by season (all geographical areas combined). The Kruskal-Wallis analyses indicated that at least one season is significantly different from the others (p < 0.001).

Figure 8. Box and Whisker Plot of NSQD, version 4.02, Total Phosphorus Residential Area Concentrations Grouped by Season.

Table 14. Residential Area TP Data by Season using Kruskal-Wallis One Way Analysis of Variance on Ranks

| Group | N | Missing | Median | 25% | 75% |
| --- | --- | --- | --- | --- | --- |
| resid TP fall | 943 | 0 | 0.29 | 0.17 | 0.48 |
| resid TP winter | 785 | 0 | 0.21 | 0.13 | 0.36 |
| resid TP spring | 1091 | 0 | 0.30 | 0.18 | 0.52 |
| resid TP summer | 1036 | 0 | 0.33 | 0.20 | 0.57 |
| H = 149.709 with 3 degrees of freedom. (P = <0.001) |  |  |  |  |  |
| The differences in the median values among the treatment groups are greater than would be expected by chance; there is a statistically significant difference (P = <0.001) |  |  |  |  |  |

The Dunn’s pair-wise comparisons of all of the NSQD residential land use total phosphorus data indicate that the winter season had the lowest total phosphorus concentrations, summer had the highest concentrations, and fall and spring had intermediate concentrations.

Table 15. Dunn’s Pair-Wise Comparisons of Residential Total Phosphorus Concentrations by Season

|  | fall | winter | spring | summer |
| --- | --- | --- | --- | --- |
| fall | X | <0.001 | 1.0 | <0.001 |
| winter | <0.001 | X | <0.001 | <0.001 |
| spring | 1.0 | <0.001 | X | 0.017 |
| summer | <0.001 | <0.001 | 0.017 | X |

#### EPA Rain Zone 1 Total Phosphorus Analyses

The following analyses examine EPA Rain Zone 1 total phosphorus concentrations for all land uses by season. The prior analyses indicated that geographical area, land use, and season all affect the likely total phosphorus concentrations for an area. These data are from 69 locations in 8 states, and total 1,019 observations. Samples were collected between 1979 and 2003, covering a period of about 24 years. Tables 16 through 23 list the station names by state and the amount of total phosphorus data available from each in EPA Rain Zone 1.

Table 16. Illinois Monitoring Locations in EPA Rain Zone 1, NSQD, version 4.02

| Illinois Station Name | First event | Last event | # of events | # stations in state | # events in state | first event in state | last event in state |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Lake_Ellyn_1_Inflow_Lake_Ellyn | 4/3/1980 | 6/8/1981 | 18 | 1 | 18 | 4/3/1980 | 6/8/1981 |

Table 17. Indiana Monitoring Locations in EPA Rain Zone 1, NSQD, version 4.02

| Indiana Station Name | First event | Last event | # of events | # stations in state | # events in state | first event in state | last event in state |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Basin_C1_control | 7/17/1999 | 12/5/2003 | 12 |  |  |  |  |
| Basin_C2_test | 7/17/1999 | 12/5/2003 | 12 |  |  |  |  |
| Basin_I1_control | 7/17/1999 | 12/5/2003 | 11 |  |  |  |  |
| Basin_I2_test | 7/17/1999 | 12/5/2003 | 11 |  |  |  |  |
| Basin_C3 | 10/8/1999 | 12/5/2003 | 10 |  |  |  |  |
| Basin_R1_control | 10/8/1999 | 10/14/2003 | 9 |  |  |  |  |
| Basin_R2_test | 10/8/1999 | 10/14/2003 | 9 | 7 | 74 | 7/17/1999 | 12/5/2003 |

Table 18. Massachusetts Monitoring Locations in EPA Rain Zone 1, NSQD, version 4.02

| Massachusetts Station Name | First event | Last event | # of events | # stations in state | # events in state | first event in state | last event in state |
| --- | --- | --- | --- | --- | --- | --- | --- |
| P4_Anna_St_Fitzgerald_Brook | 6/16/1980 | 9/26/1980 | 6 |  |  |  |  |
| P5_Convent_Tributary to Coal Mine Brook | 6/20/1980 | 8/11/1980 | 8 |  |  |  |  |
| P1_Jordan_Pond_Inlet | 6/29/1980 | 9/18/1980 | 8 |  |  |  |  |
| P2_Rt_9_Belmont_St | 7/17/1980 | 9/18/1980 | 5 |  |  |  |  |
| P3_Locust_St_Tributary to Belmont St | 7/17/1980 | 9/26/1980 | 6 |  |  |  |  |
| Addison Wesley | 4/4/1981 | 8/5/1981 | 5 |  |  |  |  |
| Hemlock Road | 4/14/1981 | 8/5/1981 | 5 |  |  |  |  |
| Brighton_25E037 | 4/11/1992 | 6/24/1992 | 3 |  |  |  |  |
| Charlestown_29J212 | 4/11/1992 | 8/14/1992 | 3 |  |  |  |  |
| Dorchester_8J102 | 4/11/1992 | 6/24/1992 | 3 |  |  |  |  |
| Hyde_Park_2F120 | 4/17/1992 | 6/24/1992 | 3 |  |  |  |  |
| West_Roxebury_13D077_078 | 4/17/1992 | 3/18/1998 | 15 |  |  |  |  |
| Beaver Brook | 3/28/2000 | 9/28/2003 | 11 |  |  |  |  |
| Brookdale Street | 3/28/2000 | 9/28/2003 | 12 |  |  |  |  |
| Camp Street | 3/28/2000 | 9/28/2003 | 11 |  |  |  |  |
| Middle Brook | 3/28/2000 | 9/28/2003 | 11 |  |  |  |  |
| New Bond Street | 3/28/2000 | 9/28/2003 | 11 |  |  |  |  |
| Olean Street | 3/28/2000 | 9/28/2003 | 12 |  |  |  |  |
| Park Ave and Maywood St | 3/28/2000 | 9/28/2003 | 12 |  |  |  |  |
| Mount_Vernon_26K099 | 6/2/2001 | 7/17/2001 | 3 |  |  |  |  |
| Wesley_G_Ross_6G108 | 9/25/2001 | 9/25/2001 | 1 |  |  |  |  |
| 27K397_Mount_Vernon_St_Charlestown | 3/3/2002 | 4/28/2002 | 3 |  |  |  |  |
| 5F208_HydeParkAve_Hyde_Park | 4/11/2003 | 10/26/2002 | 11 | 23 | 168 | 6/16/1980 | 9/28/2003 |

Table 19. Michigan Monitoring Locations in EPA Rain Zone 1, NSQD, version 4.02

| Michigan Station Name | First event | Last event | # of events | # stations in state | # events in state | first event in state | last event in state |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PITAARETBNNINLT Pitt AA RB N PNI | 6/29/1979 | 5/10/1981 | 13 |  |  |  |  |
| Pittsfield_Retention_Basin_1_in_Pittsfield_Retent | 6/30/1979 | 10/24/1980 | 7 |  |  |  |  |
| GCO_MI006_Waverly_Hills | 4/3/1980 | 10/17/1981 | 35 |  |  |  |  |
| MI1001_Grand_River | 4/3/1980 | 11/19/1981 | 22 |  |  |  |  |
| Upsized_Pipe_1_Inlet_MI008_Grace_St_N_Inlet | 4/3/1980 | 11/19/1981 | 40 |  |  |  |  |
| MI1002_Industrial_Drain | 5/30/1980 | 10/19/1981 | 18 |  |  |  |  |
| Traver_Creek_Detention_Basin_1_in_Traver_Crk_Retention | 4/11/1981 | 6/13/1981 | 5 | 7 | 140 | 6/29/1979 | 11/19/1981 |

Table 20. Minnesota Monitoring Locations in EPA Rain Zone 1, NSQD, version 4.02

| Minnesota Station Name | First event | Last event | # of events | # stations in state | # events in state | first event in state | last event in state |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Carver_Ravine_Wetland_Detention_Facility_01_in_Carver_Ravine | 4/26/1988 | 4/4/1989 | 19 |  |  |  |  |
| Carver_Ravine_Wetland_Detention_Facility_03_out_Carver_Ravine | 5/8/1988 | 4/4/1989 | 9 |  |  |  |  |
| Site1_Harriet_Pkwy_44th_St | 1/30/2001 | 10/4/2002 | 14 |  |  |  |  |
| Site5a_Snelling_ave_S_and_E_24th_St | 3/9/2001 | 10/8/2002 | 24 |  |  |  |  |
| Site2_Luella_St_at_Orange_ave | 3/15/2001 | 10/7/2002 | 13 |  |  |  |  |
| Site3_Vandalia_St_350_ft_S_of_Capp_Rd | 3/15/2001 | 10/7/2002 | 13 |  |  |  |  |
| Site4_Charles_ave_Mackubin_to_Arundel_St | 3/15/2001 | 10/7/2002 | 11 |  |  |  |  |
| Charles Ave | 5/6/2001 | 11/12/2001 | 9 |  |  |  |  |
| E_Harriet_Pkwy_W44_St | 5/6/2001 | 10/13/2001 | 10 |  |  |  |  |
| Luella St Orange Ave | 5/6/2001 | 10/13/2001 | 9 |  |  |  |  |
| E_29_St_31_Ave_S | 5/20/2001 | 11/12/2001 | 10 |  |  |  |  |
| Vandalia St | 5/20/2001 | 11/12/2001 | 10 | 12 | 151 | 4/26/1988 | 10/8/2002 |

Table 21. New Hampshire Monitoring Locations in EPA Rain Zone 1, NSQD, version 4.02

| New Hampshire Station Name | First event | Last event | # of events | # stations in state | # events in state | first event in state | last event in state |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1_PKG_Shop_n_Save_Parking_Lot | 7/29/1980 | 10/6/1981 | 27 | 1 | 27 | 7/29/1980 | 10/6/1981 |

Table 22. New York Monitoring Locations in EPA Rain Zone 1, NSQD, version 4.02

| New York Station Name | First event | Last event | # of events | # stations in state | # events in state | first event in state | last event in state |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Cedar Lane Storm Sewer | 9/2/1980 | 11/20/1981 | 32 |  |  |  |  |
| Sherriff’s Dock SS | 9/2/1980 | 11/20/1981 | 33 |  |  |  |  |
| NY_1021_Unqua_Influent | 9/18/1980 | 7/20/1981 | 8 |  |  |  |  |
| NY1031_Carlls_River_Street_Sweeping | 11/17/1980 | 10/25/1982 | 24 | 4 | 97 | 9/2/1980 | 10/25/1982 |

Table 23. Wisconsin Monitoring Locations in EPA Rain Zone 1, NSQD, version 4.02

| Wisconsin Station Name | First event | Last event | # of events | # stations in state | # events in state | first event in state | last event in state |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Wood Center | 3/14/1980 | 10/17/1981 | 47 |  |  |  |  |
| Post Office Capital Court South | 4/3/1980 | 10/17/1981 | 60 |  |  |  |  |
| Rustler Capital Court North | 4/4/1980 | 10/17/1981 | 37 |  |  |  |  |
| State Fair | 5/28/1980 | 10/17/1981 | 28 |  |  |  |  |
| N Hastings | 6/2/1980 | 7/25/1981 | 24 |  |  |  |  |
| N_Burbank_633 | 6/6/1980 | 10/17/1981 | 45 |  |  |  |  |
| Lincoln Creek | 4/4/1981 | 10/6/1981 | 23 |  |  |  |  |
| Wet_Pond_Monroe_St_I1 | 2/28/1987 | 4/5/1988 | 30 |  |  |  |  |
| Harper Road | 6/17/1993 | 11/12/1993 | 7 |  |  |  |  |
| Lakeland Ave | 6/17/1993 | 4/12/1994 | 6 |  |  |  |  |
| Observatory Dr | 6/17/1993 | 8/12/1994 | 8 |  |  |  |  |
| Syene Road | 6/17/1993 | 9/22/1994 | 15 |  |  |  |  |
| West Towne | 6/17/1993 | 5/14/1994 | 7 |  |  |  |  |
| Annamark Road | 10/8/1993 | 5/14/1994 | 7 | 14 | 344 | 3/14/1980 | 9/22/1994 |

The first analyses compared the different land uses in EPA Rain Zone 1, as shown on Figure 9 box and whisker plots and Tables 24 and 25 multivariate analyses.

Figure 9. Box and Whisker Plots of NSQD, version 4.02, Total Phosphorus EPA Rain Zone 1 Concentrations Grouped by Land Use (Significant Land Use Groupings).

Table 24. Kruskal-Wallis One Way Analysis of Variance on Ranks

| Group | N | Missing | Median | 25% | 75% |
| --- | --- | --- | --- | --- | --- |
| Z1 TP OS | 78 | 0 | 0.19 | 0.11 | 0.33 |
| Z1 TP Res | 551 | 0 | 0.30 | 0.18 | 0.48 |
| Z1 TP instit | 13 | 0 | 0.15 | 0.090 | 0.24 |
| Z1 TP commer | 293 | 0 | 0.14 | 0.083 | 0.27 |
| Z1 TP free | 3 | 0 | 0.41 | 0.35 | 0.54 |
| Z1 TP indus | 106 | 0 | 0.21 | 0.12 | 0.38 |
| H = 146.660 with 5 degrees of freedom. (P = <0.001) |  |  |  |  |  |
| The differences in the median values among the treatment groups are greater than would be expected by chance; there is a statistically significant difference (P = <0.001) |  |  |  |  |  |

The Kruskal-Wallis tests indicated that at least one land use were different from the others. Therefore, multiple pairwise comparisons (Dunn’s test) were used, as summarized in Table 25 of p values for all pairs.

Table 25. Pairwise p Values for EPA Rain Zone 1 Total Phosphorus Concentrations by Land Use Groups

|  | open space | residential | institutional | commercial | freeways | industrial |
| --- | --- | --- | --- | --- | --- | --- |
| open space | X | <0.001 | 1.0 | 0.76 | 0.59 | 1.0 |
| residential | <0.001 | X | 0.015 | <0.001 | 1.0 | <0.001 |
| institutional | 1.0 | 0.015 | X | 1.0 | 0.23 | 1.0 |
| commercial | 0.76 | <0.001 | 1.0 | X | 0.18 | 0.015 |
| freeways | 0.59 | 1.0 | 0.23 | 0.18 | X | 0.95 |
| industrial | 1.0 | <0.001 | 1.0 | 0.015 | 0.95 | X |

Very few data were available for freeways, so they were not included in the final groupings. The significant land use groupings of EPA Rain Zone 1 total phosphorus data were therefore further evaluated for seasonal effects:

- Residential land uses

- Open space plus industrial land uses

- Institutional plus commercial land uses

It is possible that the last two groups could be further combined, leaving residential areas as the only separate land use for this area. The box and whisker plot for these three combined categories are also shown above with the plots for the individual groups.

The following analyses examine these three groups by season. The first is for the residential group of total phosphorus observations shown on Figure 10 and Tables 26 and 27.

Figure 10. Box and Whisker Plots of NSQD, version 4.02, EPA Rain Zone 1 Residential Land Use Total Phosphorus Concentrations Grouped by Season.

Table 26. EPA Rain Zone 1 Residential Area Total Phosphorus Comparisons using the Kruskal-Wallis One Way Analysis of Variance on Ranks Test

| Group | N | Missing | Median | 25% | 75% |
| --- | --- | --- | --- | --- | --- |
| Z1 res fall TP | 164 | 0 | 0.89 | 0.14 | 0.99 |
| Z1 res winter TP | 38 | 0 | 0.24 | 0.13 | 0.32 |
| Z1 res spring TP | 142 | 0 | 0.49 | 0.78 | 0.95 |
| Z1 res summer TP | 207 | 0 | 0.29 | 0.19 | 0.44 |
| H = 7.920 with 3 degrees of freedom. (P = 0.048) |  |  |  |  |  |
| The differences in the median values among the treatment groups are greater than would be expected by chance; there is a statistically significant difference (P = 0.048) |  |  |  |  |  |

Table 27. Pairwise p Values for EPA Rain Zone 1 Total Phosphorus Concentrations by Season Groups for Residential Land Use Areas

|  | Fall | Winter | Spring | Summer |
| --- | --- | --- | --- | --- |
| Fall | X | 1.0 | 0.19 | 1.0 |
| Winter | 1.0 | X | 0.10 | 1.0 |
| Spring | 0.19 | 0.10 | X | 0.41 |
| Summer | 1.0 | 1.0 | 0.41 | X |

Although the Kruskal-Wallis test indicates that at least one season was significantly different from the others (p = 0.048), the pairwise tests were not able to identify any significant differences. Therefore, the residential land use total phosphorus values in this region were not separated by seasonal difference.

The following analyses shown on Figure 11 and Tables 28 through 30 examine the combined group of institutional plus commercial land use total phosphorus data.

Figure 11. Box and Whisker Plots of NSQD, version 4.02, EPA Rain Zone 1 Institutional Plus Commercial Land Use Total Phosphorus Concentrations Grouped by Season.

Table 28. EPA Rain Zone 1 Institutional plus Commercial Area Total Phosphorus Comparisons using the Kruskal-Wallis One Way Analysis of Variance on Ranks Test by Seasons

| Group | N | Missing | Median | 25% | 75% |
| --- | --- | --- | --- | --- | --- |
| Z1 fall inst commer TP | 79 | 0 | 0.1 | 0.06 | 0.23 |
| Z1 winter inst commer TP | 33 | 0 | 0.13 | 0.095 | 0.45 |
| Z1 spring inst commer TP | 68 | 0 | 0.18 | 0.11 | 0.31 |
| Z1 summer inst commer TP | 126 | 0 | 0.14 | 0.078 | 0.23 |
| H = 12.164 with 3 degrees of freedom. (P = 0.007) |  |  |  |  |  |
| The differences in the median values among the treatment groups are greater than would be expected by chance; there is a statistically significant difference (P = 0.007) |  |  |  |  |  |

Table 29. Pairwise p Values for EPA Rain Zone 1 Total Phosphorus Concentrations by Season Groups for Institutional plus Commercial Land Use Areas

|  | Fall | Winter | Spring | Summer |
| --- | --- | --- | --- | --- |
| Fall | X | 1.0 | 0.005 | 0.082 |
| Winter | 1.0 | X | 0.68 | 1.0 |
| Spring | 0.005 | 0.68 | X | 1.0 |
| Summer | 0.082 | 1.0 | 1.0 | X |

The Kruskal-Wallis test indicated that at least one season was different from the others. The obvious differences shown are between spring and fall and possibly fall and summer. Further analyses investigated the seasons by grouping fall with winter and spring with summer, as also shown on the preceding box and whisker plot. As shown by the Mann-Whitney test (SigmaPlot 13), these two groups were significantly different, with the summer-spring group having larger total phosphorus concentrations compared to the winter-fall group.

Table 30. Mann-Whitney Rank Sum Test Comparing Institutional plus Commercial Total Phosphorus Values for Fall and Winter vs. Summer and Spring Seasonal Groups

| Group | N | Missing | Median | 25% | 75% |
| --- | --- | --- | --- | --- | --- |
| Z1 fall win inst commer TP | 112 | 0 | 0.12 | 0.07 | 0.23 |
| Z1 spring summ inst commer TP | 194 | 0 | 0.16 | 0.1 | 0.22 |
| Mann-Whitney U Statistic= 8602.500 |  |  |  |  |  |
| T = 14930.500 n(small)= 112 n(big)= 194 (P = 0.002) |  |  |  |  |  |
| The difference in the median values between the two groups is greater than would be expected by chance; there is a statistically significant difference (P = 0.002) |  |  |  |  |  |

The total phosphorus concentrations in the combined open space plus industrial land use group was also evaluated as shown on Figure 12 and Tables 31 through 33.

Figure 12. Box and Whisker Plots of NSQD, version 4.02, EPA Rain Zone 1 Open Space Plus Industrial Land Use Total Phosphorus Concentrations Grouped by Season.

Table 31. Open Space and Industrial Land Use Seasonal Analyses using the Kruskal-Wallis One Way Analysis of Variance on Ranks Test

| Group | N | Missing | Median | 25% | 75% |
| --- | --- | --- | --- | --- | --- |
| ZI fall opnspc indus TP | 63 | 0 | 0.14 | 0.087 | 0.28 |
| ZI winter opnspc indus TP | 22 | 0 | 0.89 | 0.38 | 0.58 |
| ZI spring opnspc indus TP | 36 | 0 | 0.95 | 0.097 | 0.89 |
| Z1 summer opnspc indus TP | 63 | 0 | 0.28 | 0.67 | 0.48 |
| H = 19.639 with 3 degrees of freedom. (P = <0.001) |  |  |  |  |  |
| The differences in the median values among the treatment groups are greater than would be expected by chance; there is a statistically significant difference (P = <0.001) |  |  |  |  |  |
| To isolate the group or groups that differ from the others use a multiple comparison procedure. |  |  |  |  |  |

The Kruskal Wallis test indicated that at least one seasonal data set was significantly different from the others. Table 32 shows all pairwise comparisons for these data.

Table 32. Pairwise p Values for EPA Rain Zone 1 Total Phosphorus Concentrations by Season Groups for Open Space plus Industrial Land Use Areas

|  | Fall | Winter | Spring | Summer |
| --- | --- | --- | --- | --- |
| Fall | X | 1.0 | 1.0 | <0.001 |
| Winter | 1.0 | X | 1.0 | 0.19 |
| Spring | 1.0 | 1.0 | X | 0.01 |
| Summer | <0.001 | 0.19 | 0.01 | X |

The major differences were between the fall and summer seasons, while the summer and spring seasons also indicated significant differences in total phosphorus concentrations, with the summer concentrations appearing to be larger than the other groups on the grouped box and whisker plot and the fall period being the lowest. Therefore, these data were grouped with fall, winter, and spring all combined, with summer left separate, as shown on the above plot. The Mann-Whitney analysis on Table 33 shows that these two data groups are significantly different (p<0.001), with the summer having the highest total phosphorus concentrations compared to the other group of remaining seasons.

Table 33. Mann-Whitney Rank Sum Test Comparing Open Space plus Industrial Total Phosphorus Values for Summer vs. Fall, Winter and Spring Seasonal Groups

| Group | N | Missing | Median | 25% | 75% |
| --- | --- | --- | --- | --- | --- |
| Z1 op spc indus summer TP | 63 | 0 | 0.28 | 0.67 | 0.48 |
| Z1 opspc indus fall wint sprin | 121 | 0 | 0.17 | 0.1 | 0.26 |
| Mann-Whitney U Statistic= 2330.500 |  |  |  |  |  |
| T = 7308.500 n(small)= 63 n(big)= 121 (P = <0.001) |  |  |  |  |  |
| The difference in the median values between the two groups is greater than would be expected by chance; there is a statistically significant difference (P = <0.001) |  |  |  |  |  |

# Appendix B: Seasonal, Land Use, and Location Values from NSQD ver 4.

| Residential |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | TSS (mg/L) | Fecal Coliform (colonies/100 mL) | Ammonia (mg/L) | N02+NO3 (mg/L as N) | Phosphorous Filtered as P (mg/L) | Phos part strength (mg/kg) | Phosphate Ortho as P (mg/L) |
| Residential overall: | count | 2352 | 520 | 774 | 1639 | 1076 | 964 | 137 |
|  | median | 57 | 7401.5 | 0.38 | 0.65 | 0.1795 | 2.700575 | 0.19 |
|  | stdev | 239.9165 | 385166.3 | 0.849786 | 1.708351 | 0.622860905 | 34.18694 | 0.266277177 |
|  | avg | 125.7131 | 82052 | 0.643535 | 0.987822 | 0.258188259 | 6.280238 | 0.263743796 |
|  | COV | 1.91 | 4.69 | 1.32 | 1.73 | 2.41 | 5.44 | 1.01 |
| Residential EPA 1: | count | 268 | 74 | 90 | 192 | 69 | 64 | 13 |
|  | median | 84 | 4100 | 0.486 | 0.609 | 0.132 | 2.66051 | 0.16 |
|  | stdev | 162.8292 | 740417.6 | 0.682915 | 0.551375 | 0.239827422 | 7.744697 | 0.114959692 |
|  | avg | 139.0049 | 176588.1 | 0.686111 | 0.753397 | 0.209115942 | 5.286272 | 0.155692308 |
|  | COV | 1.17 | 4.19 | 1.00 | 0.73 | 1.15 | 1.47 | 0.74 |
| Residential EPA 2: | count | 1156 | 189 | 477 | 946 | 470 | 434 | 0 |
|  | median | 43 | 2400 | 0.29 | 0.59 | 0.16 | 3.529412 |  |
|  | stdev | 136.0027 | 17245.37 | 0.42072 | 2.132229 | 0.185684955 | 47.33579 |  |
|  | avg | 87.05341 | 9347.945 | 0.410983 | 1.028579 | 0.213061702 | 7.89366 |  |
|  | COV | 1.56 | 1.84 | 1.02 | 2.07 | 0.87 | 6.00 |  |
| Residential EPA 3: | count | 121 | 28 | 3 | 53 | 78 | 58 | 0 |
|  | median | 23 | 2622.5 | 0.06 | 0.5 | 0.101 | 3.61724 |  |
|  | stdev | 172.0979 | 225415.8 | 0.078102 | 1.255479 | 2.176840914 | 46.6312 |  |
|  | avg | 83.82612 | 52270.39 | 0.1 | 0.929962 | 0.414505128 | 15.4764 |  |
|  | COV | 2.05 | 4.31 | 0.78 | 1.35 | 5.25 | 3.01 |  |
| Residential EPA 4: | count | 117 | 62 | 8 | 62 | 51 | 49 | 5 |
|  | median | 99 | 33500 | 0.92 | 0.812 | 0.28 | 1.8 | 0.05 |
|  | stdev | 726.8345 | 44364.97 | 0.5378 | 0.580641 | 0.182855134 | 7.540141 | 0.058906706 |
|  | avg | 405.0205 | 44291.58 | 1.08 | 0.894452 | 0.28 | 4.957651 | 0.082 |
|  | COV | 1.79 | 1.00 | 0.50 | 0.65 | 0.65 | 1.52 | 0.72 |
| Residential EPA 5: | count | 107 | 70 | 0 | 106 | 69 | 61 | 0 |
|  | median | 76 | 29000 |  | 0.7 | 0.27 | 2.5 |  |
|  | stdev | 106.3083 | 180308.7 |  | 0.785001 | 0.153769126 | 3.20452 |  |
|  | avg | 109.6449 | 106601.4 |  | 0.885915 | 0.290289855 | 3.083464 |  |
|  | COV | 0.97 | 1.69 |  | 0.89 | 0.53 | 1.04 |  |
| Residential EPA 6: | count | 145 | 39 | 120 | 31 | 137 | 113 | 20 |
|  | median | 61 | 90000 | 0.4605 | 1.1 | 0.23 | 1.028986 | 0.21 |
|  | stdev | 138.925 | 848892.8 | 0.915519 | 0.46028 | 0.269685049 | 6.473326 | 0.215306391 |
|  | avg | 106.1172 | 387869.2 | 0.831567 | 1.107742 | 0.321313869 | 2.956459 | 0.281 |
|  | COV | 1.31 | 2.19 | 1.10 | 0.42 | 0.84 | 2.19 | 0.77 |
| Residential EPA 7: | count | 158 | 47 | 25 | 53 | 18 | 17 | 24 |
|  | median | 82 | 1600 | 0.3 | 0.7 | 0.055 | 6.733333 | 0.1 |
|  | stdev | 90.35955 | 44838.94 | 1.132167 | 0.9235 | 0.572663348 | 20.8546 | 0.10357382 |
|  | avg | 98.27215 | 16965.7 | 0.62152 | 0.865472 | 0.346611111 | 13.65719 | 0.126666667 |
|  | COV | 0.92 | 2.64 | 1.82 | 1.07 | 1.65 | 1.53 | 0.82 |
| Residential EPA 8: | count | 7 | 5 | 7 | 1 | 0 | 0 | 5 |
|  | median | 53 | 4500 | 0.68 | 0.48 |  |  | 0.2079 |
|  | stdev | 113.036 | 12268.54 | 0.149423 |  |  |  | 0.18532472 |
|  | avg | 96.85714 | 8496 | 0.714429 | 0.48 |  |  | 0.27298 |
|  | COV | 1.17 | 1.44 | 0.21 |  |  |  | 0.68 |
| Residential EPA 9: | count | 273 | 6 | 44 | 195 | 184 | 168 | 70 |
|  | median | 111.91 | 41000 | 2.14 | 0.93 | 0.18 | 1.550209 | 0.235 |
|  | stdev | 294.0135 | 25838.91 | 1.610879 | 0.985009 | 0.22821524 | 2.441872 | 0.319491914 |
|  | avg | 208.5562 | 32373.33 | 2.523636 | 1.138518 | 0.251859602 | 2.351663 | 0.3382 |
|  | COV | 1.41 | 0.80 | 0.64 | 0.87 | 0.91 | 1.04 | 0.94 |

| Residential |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cadmium Filtered (ug/L) | Cadmium part strength (ug/kg) | Chromium Filtered (ug/L) | Chromium part strength (ug/kg) | Copper Filtered (ug/L) | Copper part strength (ug/kg) | Iron, Filtered as Fe (ug/L) | Iron part strength (ug/kg) | Lead Filtered After 1984 (ug/L) | Lead part strength (ug/kg) | Nickel Filtered (ug/L) | Nickel part strength (ug/kg) | Zinc Filtered (ug/L) | Zinc part strength (ug/kg) |
| Residential overall: | count | 8 | 4 | 13 | 9 | 152 | 114 | 40 | 30 | 42 | 25 | 23 | 14 | 150 | 113 |
|  | median | 1.095 | 9.166667 | 1.71 | 41.2371134 | 9.11 | 104.9572 | 235 | 6473.94 | 4.5 | 196.4286 | 4 | 26.95625 | 50 | 774.1935 |
|  | stdev | 1.043421 | 5.116644 | 4.2908092 | 46.0856344 | 20.7043 | 1608.292 | 1259.97 | 28765.99 | 27.66523 | 387.9438 | 4.992312 | 348.9624 | 119.4862 | 2536.878 |
|  | avg | 1.24875 | 9.848901 | 3.96538462 | 53.1609482 | 15.11072 | 497.7989 | 569.575 | 17617.83 | 14.38333 | 282.533 | 5.908261 | 127.5374 | 85.41467 | 1433.364 |
|  | COV | 0.84 | 0.52 | 1.08 | 0.87 | 1.37 | 3.23 | 2.21 | 1.63 | 1.92 | 1.37 | 0.84 | 2.74 | 1.40 | 1.77 |
| Residential EPA 1: | count | 0 | 0 | 0 | 0 | 11 | 7 | 0 | 0 | 1 | 1 | 0 | 0 | 11 | 6 |
|  | median |  |  |  |  | 1.4 | 152.6316 |  |  | 0.6 | 28.78788 |  |  | 6.5 | 144.6429 |
|  | stdev |  |  |  |  | 60.93195 | 3047.554 |  |  |  |  |  |  | 77.84323 | 1057.015 |
|  | avg |  |  |  |  | 35.20909 | 1670.952 |  |  | 0.6 | 28.78788 |  |  | 49.27273 | 763.1956 |
|  | COV |  |  |  |  | 1.73 | 1.82 |  |  |  |  |  |  | 1.58 | 1.38 |
| Residential EPA 2: | count | 4 | 1 | 4 | 4 | 33 | 26 | 0 | 0 | 15 | 10 | 0 | 0 | 43 | 33 |
|  | median | 0.75 | 10 | 1.47 | 39.4573643 | 7.12 | 193.75 |  |  | 2.5 | 183.8346 |  |  | 28 | 1166.667 |
|  | stdev | 0.4539 |  | 0.2957899 | 31.0875924 | 11.60161 | 1207.495 |  |  | 5.062702 | 280.6215 |  |  | 15.38174 | 2479.102 |
|  | avg | 0.7225 | 10 | 1.4375 | 39.8046052 | 10.89303 | 570.0175 |  |  | 5.166667 | 236.8436 |  |  | 30.2093 | 1827.367 |
|  | COV | 0.63 |  | 0.21 | 0.78 | 1.07 | 2.12 |  |  | 0.98 | 1.18 |  |  | 0.51 | 1.36 |
| Residential EPA 3: | count | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | median |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | stdev |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | avg |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | COV |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Residential EPA 4: | count | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | median |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | stdev |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | avg |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | COV |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Residential EPA 5: | count | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | median |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | stdev |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | avg |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | COV |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Residential EPA 6: | count | 3 | 2 | 4 | 0 | 87 | 61 | 40 | 30 | 14 | 3 | 12 | 6 | 67 | 47 |
|  | median | 1.5 | 6.364469 | 9.5 |  | 12.4 | 87.09677 | 235 | 6473.94 | 23.8 | 46.42857 | 9.1 | 40.51632 | 100 | 642.8571 |
|  | stdev | 1.274101 | 2.784395 | 3.73753484 |  | 13.697 | 1764.66 | 1259.97 | 28765.99 | 40.5738 | 43.41053 | 5.034648 | 526.7024 | 155.3589 | 3252.521 |
|  | avg | 2.133333 | 6.364469 | 9.475 |  | 16.36966 | 464.885 | 569.575 | 17617.83 | 35.64286 | 61.42447 | 9.090833 | 263.6459 | 145.6224 | 1528.927 |
|  | COV | 0.60 | 0.44 | 0.39 |  | 0.84 | 3.80 | 2.21 | 1.63 | 1.14 | 0.71 | 0.55 | 2.00 | 1.07 | 2.13 |
| Residential EPA 7: | count | 1 | 1 | 5 | 5 | 19 | 19 | 0 | 0 | 12 | 11 | 10 | 7 | 25 | 24 |
|  | median | 0.7 | 16.66667 | 1 | 41.2371134 | 5 | 62.5 |  |  | 1 | 216.4948 | 2 | 17.54386 | 40 | 626.3889 |
|  | stdev |  |  | 0.91214034 | 56.5847786 | 3.481992 | 96.04082 |  |  | 2.369311 | 499.5957 | 1.173788 | 15.66656 | 24.30535 | 852.9615 |
|  | avg | 0.7 | 16.66667 | 1.58 | 63.8460226 | 5.973684 | 94.60486 |  |  | 2.25 | 407.4389 | 2.6 | 25.73115 | 39.1 | 962.4502 |
|  | COV |  |  | 0.58 | 0.89 | 0.58 | 1.02 |  |  | 1.05 | 1.23 | 0.45 | 0.61 | 0.62 | 0.89 |
| Residential EPA 8: | count | 0 | 0 | 0 | 0 | 2 | 1 | 0 | 0 | 0 | 0 | 1 | 1 | 4 | 3 |
|  | median |  |  |  |  | 6.2 | 76.47059 |  |  |  |  | 0.8 | 23.52941 | 45 | 760.8696 |
|  | stdev |  |  |  |  | 5.374012 |  |  |  |  |  |  |  | 40.41761 | 149.9678 |
|  | avg |  |  |  |  | 6.2 | 76.47059 |  |  |  |  | 0.8 | 23.52941 | 59.25 | 709.8131 |
|  | COV |  |  |  |  | 0.87 |  |  |  |  |  |  |  | 0.68 | 0.21 |
| Residential EPA 9: | count | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | median |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | stdev |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | avg |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | COV |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Commercial |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | TSS (mg/L) | Fecal Coliform (colonies/100 mL) | Ammonia (mg/L) | N02+NO3 (mg/L as N) | Phosphorous Filtered as P (mg/L) | Phos part strength (mg/kg) | Phosphate Ortho as P (mg/L) |
| Commercial overall: | count | 1132 | 284 | 349 | 788 | 505 | 450 | 135 |
|  | median | 52 | 3250 | 0.48 | 0.67505 | 0.11 | 2.358289 | 0.12 |
|  | stdev | 212.0554 | 72449.34 | 1.016547 | 0.830286 | 0.282986179 | 7.90126 | 0.225377814 |
|  | avg | 119.827 | 24050.36 | 0.759639 | 0.864537 | 0.221625941 | 4.574666 | 0.196281481 |
|  | COV | 1.769681 | 3.012401 | 1.338198 | 0.960382 | 1.276863975 | 1.727177 | 1.148237787 |
| Commercial EPA 1: | count | 197 | 23 | 62 | 152 | 24 | 19 | 0 |
|  | median | 96.3 | 2050 | 0.37 | 0.6505 | 0.15 | 2.272727 |  |
|  | stdev | 166.5768 | 13125.46 | 0.256494 | 0.517555 | 0.062413709 | 24.53695 |  |
|  | avg | 142.9497 | 6774.435 | 0.434516 | 0.760941 | 0.142083333 | 10.47566 |  |
|  | COV | 1.17 | 1.94 | 0.59 | 0.68 | 0.44 | 2.34 |  |
| Commercial EPA 2: | count | 445 | 124 | 224 | 402 | 183 | 169 | 0 |
|  | median | 43 | 2574 | 0.485 | 0.68 | 0.11 | 3.333333 |  |
|  | stdev | 160.032 | 31778.35 | 0.686276 | 0.957528 | 0.210478673 | 5.761721 |  |
|  | avg | 87.37396 | 14929.29 | 0.647902 | 0.926199 | 0.192459016 | 4.661389 |  |
|  | COV | 1.83 | 2.13 | 1.06 | 1.03 | 1.09 | 1.24 |  |
| Commercial EPA 3: | count | 100 | 5 | 6 | 18 | 84 | 80 | 0 |
|  | median | 49.5 | 2000 | 0.105 | 0.2955 | 0.31 | 3.112903 |  |
|  | stdev | 124.1597 | 3066.268 | 0.115412 | 0.445728 | 0.416094581 | 6.352353 |  |
|  | avg | 91.3762 | 3220 | 0.15 | 0.443222 | 0.410775 | 6.089381 |  |
|  | COV | 1.36 | 0.95 | 0.77 | 1.01 | 1.01 | 1.04 |  |
| Commercial EPA 4: | count | 121 | 38 | 3 | 25 | 25 | 21 | 22 |
|  | median | 39 | 17500 | 0.4 | 0.75 | 0.11 | 2.5 | 0.105 |
|  | stdev | 329.136 | 107891.6 | 0.321455 | 0.615887 | 0.317131413 | 4.53291 | 0.184116587 |
|  | avg | 144.7793 | 53267.89 | 0.533333 | 0.9338 | 0.2484 | 4.85937 | 0.176818182 |
|  | COV | 2.27 | 2.03 | 0.60 | 0.66 | 1.28 | 0.93 | 1.04 |
| Commercial EPA 5: | count | 38 | 40 | 0 | 39 | 34 | 29 | 0 |
|  | median | 34.5 | 8050 |  | 0.52 | 0.05 | 1.945652 |  |
|  | stdev | 111.7848 | 111108 |  | 0.237948 | 0.03494596 | 3.228699 |  |
|  | avg | 70.71053 | 39723.83 |  | 0.550692 | 0.055147059 | 2.900022 |  |
|  | COV | 1.58 | 2.80 |  | 0.43 | 0.63 | 1.11 |  |
| Commercial EPA 6: | count | 34 | 14 | 30 | 33 | 37 | 19 | 22 |
|  | median | 70.5 | 5400 | 1.6 | 1.1 | 0.32 | 3.561644 | 0.29 |
|  | stdev | 202.1576 | 10764.66 | 2.288982 | 0.865809 | 0.308696037 | 16.65763 | 0.230106318 |
|  | avg | 141.7941 | 9264.286 | 2.185133 | 1.256061 | 0.383918919 | 10.00942 | 0.331818182 |
|  | COV | 1.43 | 1.16 | 1.05 | 0.69 | 0.80 | 1.66 | 0.69 |
| Commercial EPA 7: | count | 35 | 31 | 15 | 32 | 6 | 5 | 22 |
|  | median | 50 | 1900 | 0.5 | 0.25 | 0.0645 | 2.592593 | 0.1 |
|  | stdev | 94.42856 | 111415.2 | 0.998754 | 0.484821 | 0.616836445 | 1.407792 | 0.10597823 |
|  | avg | 87.8 | 28608.81 | 0.783333 | 0.4275 | 0.327 | 2.673141 | 0.128636364 |
|  | COV | 1.08 | 3.89 | 1.28 | 1.13 | 1.89 | 0.53 | 0.82 |
| Commercial EPA 9: | count | 162 | 9 | 9 | 87 | 112 | 108 | 69 |
|  | median | 80.75 | 15000 | 1.4 | 0.77 | 0.0645 | 1.378989 | 0.1 |
|  | stdev | 311.8032 | 27035.16 | 0.94408 | 0.87511 | 0.18483009 | 1.900298 | 0.249036061 |
|  | avg | 193.6091 | 19717 | 1.471111 | 0.980805 | 0.129767857 | 1.805045 | 0.18084058 |
|  | COV | 1.61 | 1.37 | 0.64 | 0.89 | 1.42 | 1.05 | 1.38 |

| Commercial |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cadmium Filtered (ug/L) | Cadmium part strength (ug/kg) | Chromium Filtered (ug/L) | Chromium part strength (ug/kg) | Copper Filtered (ug/L) | Copper part strength (ug/kg) | Iron, Filtered as Fe (ug/L) | Iron part strength (ug/kg) | Lead Filtered After 1984 (ug/L) | Lead part strength (ug/kg) | Nickel Filtered (ug/L) | Nickel part strength (ug/kg) | Zinc Filtered (ug/L) | Zinc part strength (ug/kg) |
| Commercial overall: | count | 13 | 6 | 11 | 11 | 52 | 44 | 5 | 5 | 35 | 24 | 11 | 8 | 78 | 69 |
|  | median | 0.36 | 13 | 2 | 64.516129 | 9.5 | 204.8387 | 320 | 3880.597 | 5 | 454.2857 | 3 | 25.32051 | 57 | 830.5085 |
|  | stdev | 17.13222 | 274.448 | 1.28262869 | 32.4265402 | 13.23395 | 2386.22 | 189.2881 | 16963.32 | 21.97588 | 572.8297 | 3.34664 | 20.81302 | 155.4889 | 8597.717 |
|  | avg | 6.381538 | 139.7745 | 2.16181818 | 58.3477464 | 12.43654 | 692.9323 | 384 | 11409.78 | 14.28857 | 586.1515 | 4 | 31.30596 | 105.6154 | 2779.344 |
|  | COV | 2.684654 | 1.963505 | 0.59331016 | 0.55574623 | 1.064118 | 3.443656 | 0.492938 | 1.486735 | 1.538004 | 0.977272 | 0.83666 | 0.664826 | 1.472218 | 3.093434 |
| Commercial EPA 1: | count | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | median |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | stdev |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | avg |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | COV |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Commercial EPA 2: | count | 1 | 1 | 2 | 2 | 14 | 13 | 0 | 0 | 6 | 5 | 0 | 0 | 17 | 16 |
|  | median | 0.2 | 20 | 2.89 | 33.074391 | 7.565 | 200 |  |  | 8 | 266.6667 |  |  | 129 | 3577.508 |
|  | stdev |  |  | 1.07480231 | 44.4653323 | 3.62763 | 293.7953 |  |  | 5.879342 | 414.6802 |  |  | 76.96203 | 3823.188 |
|  | avg | 0.2 | 20 | 2.89 | 33.074391 | 7.757143 | 291.1581 |  |  | 9.166667 | 518.4127 |  |  | 125.1765 | 4096.978 |
|  | COV |  |  | 0.37 | 1.34 | 0.47 | 1.01 |  |  | 0.64 | 0.80 |  |  | 0.61 | 0.93 |
| Commercial EPA 3: | count | 2 | 2 | 0 | 0 | 8 | 4 | 0 | 0 | 1 | 1 | 0 | 0 | 19 | 19 |
|  | median | 38 | 402.3297 |  |  | 14.5 | 227.2133 |  |  | 6 | 127.5917 |  |  | 9 | 293.6508 |
|  | stdev | 33.94113 | 411.8453 |  |  | 26.26649 | 252.9389 |  |  |  |  |  |  | 170.4913 | 1591.306 |
|  | avg | 38 | 402.3297 |  |  | 25.25 | 244.9042 |  |  | 6 | 127.5917 |  |  | 58.05263 | 750.6786 |
|  | COV | 0.89 | 1.02 |  |  | 1.04 | 1.03 |  |  |  |  |  |  | 2.94 | 2.12 |
| Commercial EPA 4: | count | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | median |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | stdev |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | avg |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | COV |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Commercial EPA 5: | count | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | median |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | stdev |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | avg |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | COV |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Commercial EPA 6: | count | 1 | 0 | 0 | 0 | 9 | 6 | 5 | 5 | 11 | 1 | 1 | 1 | 16 | 10 |
|  | median | 3 |  |  |  | 11 | 482.1429 | 320 | 3880.597 | 30 | 142.8571 | 10 | 75.14451 | 98.5 | 1010.007 |
|  | stdev |  |  |  |  | 5.101743 | 6356.368 | 189.2881 | 16963.32 | 30.1092 |  |  |  | 132.682 | 21841.64 |
|  | avg | 3 |  |  |  | 13.44444 | 3058.89 | 384 | 11409.78 | 34.81818 | 142.8571 | 10 | 75.14451 | 153.125 | 7890.639 |
|  | COV |  |  |  |  | 0.38 | 2.08 | 0.49 | 1.49 | 0.86 |  |  |  | 0.87 | 2.77 |
| Commercial EPA 7: | count | 9 | 3 | 9 | 9 | 21 | 21 | 0 | 0 | 17 | 17 | 10 | 7 | 26 | 24 |
|  | median | 0.3 | 5.882353 | 2 | 66.6666667 | 6 | 171.4286 |  |  | 2 | 500 | 2.5 | 25 | 54 | 1004.95 |
|  | stdev | 0.346186 | 2.215447 | 1.32287566 | 29.530215 | 9.982413 | 364.4195 |  |  | 4.08136 | 629.4334 | 2.836273 | 11.80331 | 188.9537 | 1530.223 |
|  | avg | 0.417778 | 4.662539 | 2 | 63.9640476 | 10.24286 | 351.0005 |  |  | 3.3 | 659.1249 | 3.4 | 25.04331 | 98.34615 | 1377.242 |
|  | COV | 0.83 | 0.48 | 0.66 | 0.46 | 0.97 | 1.04 |  |  | 1.24 | 0.95 | 0.83 | 0.47 | 1.92 | 1.11 |
| Commercial EPA 9: | count | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | median |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | stdev |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | avg |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | COV |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Institutional |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | TSS (mg/L) | Fecal Coliform (colonies/100 mL) | Ammonia (mg/L) | N02+NO3 (mg/L as N) | Phosphorous Filtered as P (mg/L) | Phos part strength (mg/kg) | Phosphate Ortho as P (mg/L) |
| Institutional overall: | count | 237 | 3 | 51 | 291 | 113 | 101 | 46 |
|  | median | 66.95 | 3400 | 0.292 | 0.18 | 0.12 | 1.057692 | 0.057999998 |
|  | stdev | 351.4658 | 1374.773 | 1.523763 | 0.496729 | 0.180140102 | 12.08872 | 0.248659639 |
|  | avg | 144.6702 | 3100 | 0.620647 | 0.401285 | 0.171185841 | 3.766353 | 0.112369565 |
|  | COV | 2.43 | 0.44 | 2.46 | 1.24 | 1.05 | 3.21 | 2.21 |
| institutional EPA1: | count | 13 | 3 | 1 | 12 | 5 | 4 | 0 |
|  | median | 76 | 3400 | 0.6 | 1.145 | 0.06 | 1.435786 |  |
|  | stdev | 56.64793 | 1374.773 |  | 0.456117 | 0.034351128 | 2.393685 |  |
|  | avg | 73.67846 | 3100 | 0.6 | 1.143333 | 0.054 | 2.567006 |  |
|  | COV | 0.77 | 0.44 |  | 0.40 | 0.64 | 0.93 |  |
| institutional EPA2: | count | 45 | 0 | 16 | 45 | 14 | 11 | 0 |
|  | median | 64.26 |  | 0.31 | 0.56 | 0.125 | 3.333333 |  |
|  | stdev | 83.24506 |  | 0.198759 | 0.465718 | 0.062092628 | 18.36242 |  |
|  | avg | 86.16711 |  | 0.37375 | 0.628667 | 0.126428571 | 11.45242 |  |
|  | COV | 0.97 |  | 0.53 | 0.74 | 0.49 | 1.60 |  |
| institutional EPA3: | count | 0 | 0 | 0 | 161 | 0 | 0 | 0 |
|  | median |  |  |  | 0.077 |  |  |  |
|  | stdev |  |  |  | 0.121092 |  |  |  |
|  | avg |  |  |  | 0.110584 |  |  |  |
|  | COV |  |  |  | 1.10 |  |  |  |
| institutional EPA4: | count | 36 | 0 | 0 | 0 | 0 | 0 | 1 |
|  | median | 50.5 |  |  |  |  |  | 0.08 |
|  | stdev | 37.44892 |  |  |  |  |  |  |
|  | avg | 62.51351 |  |  |  |  |  |  |
|  | COV | 0.60 |  |  |  |  |  |  |
| institutional EPA6: | count | 51 | 0 | 34 | 0 | 48 | 42 | 0 |
|  | median | 69 |  | 0.2885 |  | 0.1885 | 0.517394 |  |
|  | stdev | 90.06338 |  | 1.859134 |  | 0.217936938 | 1.373381 |  |
|  | avg | 94.21569 |  | 0.737441 |  | 0.264458333 | 1.074738 |  |
|  | COV | 0.96 |  | 2.52 |  | 0.82 | 1.28 |  |
| institutional EPA9: | count | 92 | 0 | 0 | 73 | 46 | 44 | 45 |
|  | median | 76 |  |  | 0.67 | 0.059999999 | 1.099271 | 0.057999998 |
|  | stdev | 543.5667 |  |  | 0.581564 | 0.115594871 | 15.37315 | 0.251421037 |
|  | avg | 242.7554 |  |  | 0.780274 | 0.100217392 | 4.523137 | 0.113088888 |
|  | COV | 2.24 |  |  | 0.75 | 1.15 | 3.40 | 2.22 |

| Institutional |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cadmium Filtered (ug/L) | Cadmium part strength (ug/kg) | Chromium Filtered (ug/L) | Chromium part strength (ug/kg) | Copper Filtered (ug/L) | Copper part strength (ug/kg) | Iron, Filtered as Fe (ug/L) | Iron part strength (ug/kg) | Lead Filtered After 1984 (ug/L) | Lead part strength (ug/kg) | Nickel Filtered (ug/L) | Nickel part strength (ug/kg) | Zinc Filtered (ug/L) | Zinc part strength (ug/kg) |
| Institutional overall: | count | 6 | 4 | 1 | 0 | 45 | 38 | 37 | 35 | 2 | 1 | 6 | 5 | 34 | 27 |
|  | median | 1.31 | 1.060049 | 11.2 |  | 10.4 | 58.17188 | 279 | 5591.837 | 15.85 | 25 | 6.44 | 20.625 | 93.45 | 362.3188 |
|  | stdev | 0.802363 | 5.547481 |  |  | 11.37885 | 969.4374 | 1086.359 | 20281.31 | 12.37437 |  | 2.749703 | 32.75424 | 54.10545 | 892.2192 |
|  | avg | 1.313333 | 3.554653 | 11.2 |  | 14.19133 | 293.9798 | 561.3014 | 12912.94 | 15.85 | 25 | 6.096667 | 34.17269 | 99.10588 | 726.3241 |
|  | COV | 0.61 | 1.56 |  |  | 0.80 | 3.30 | 1.94 | 1.57 | 0.78 |  | 0.45 | 0.96 | 0.55 | 1.23 |
| institutional EPA1: | count | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | median |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | stdev |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | avg |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | COV |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| institutional EPA2: | count | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | median |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | stdev |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | avg |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | COV |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| institutional EPA3: | count | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | median |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | stdev |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | avg |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | COV |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| institutional EPA4: | count | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | median |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | stdev |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | avg |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | COV |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| institutional EPA6: | count | 5 | 3 | 1 | 0 | 44 | 37 | 36 | 35 | 2 | 1 | 5 | 4 | 33 | 26 |
|  | median | 1.42 | 1.458333 | 11.2 |  | 10.6 | 60.60606 | 279.5 | 5591.837 | 15.85 | 25 | 6.48 | 35.04934 | 93.9 | 373.4671 |
|  | stdev | 0.602561 | 6.370448 |  |  | 11.43209 | 981.7136 | 1097.565 | 20281.31 | 12.37437 |  | 1.596459 | 32.60718 | 52.72728 | 901.6987 |
|  | avg | 1.556 | 4.51895 |  |  | 14.38886 | 301.4046 | 576.8889 | 12912.94 | 15.85 |  | 7.056 | 41.59454 | 101.7152 | 749.3106 |
|  | COV | 0.39 | 1.41 |  |  | 0.79 | 3.26 | 1.90 | 1.57 | 0.78 |  | 0.23 | 0.78 | 0.52 | 1.20 |
| institutional EPA9: | count | 1 | 1 | 0 | 0 | 1 | 1 | 1 | 0 | 0 | 0 | 1 | 1 | 1 | 1 |
|  | median | 0.1 | 0.661765 |  |  | 5.5 | 19.26471 | 0.15 |  |  |  | 1.3 | 4.485294 | 13 | 128.6765 |
|  | stdev |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | avg |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | COV |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Industrial |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | TSS (mg/L) | Fecal Coliform (colonies/100 mL) | Ammonia (mg/L) | N02+NO3 (mg/L as N) | Phosphorous Filtered as P (mg/L) | Phos part strength (mg/kg) | Phosphate Ortho as P (mg/L) |
| Industrial overall: | count | 695 | 348 | 298 | 560 | 370 | 336 | 70 |
|  | median | 75 | 2250 | 0.45 | 0.6555 | 0.095 | 1.939926 | 0.23 |
|  | stdev | 261.1604 | 279290.4 | 0.819019 | 0.803846 | 0.195829752 | 11.21182 | 0.218431307 |
|  | avg | 160.9249 | 46489.53 | 0.664812 | 0.873754 | 0.150882162 | 3.8457 | 0.273285714 |
|  | COV | 1.62 | 6.01 | 1.23 | 0.92 | 1.30 | 2.92 | 0.80 |
| Industrial EPA1: | count | 105 | 40 | 58 | 103 | 27 | 24 | 1 |
|  | median | 81 | 1350 | 0.52 | 0.564 | 0.04 | 2.438601 | 0.33 |
|  | stdev | 247.3206 | 568231 | 1.380276 | 0.401852 | 0.080105717 | 33.7589 |  |
|  | avg | 169.2276 | 97380.7 | 0.817121 | 0.661184 | 0.085185185 | 9.888561 | 0.33 |
|  | COV | 1.46 | 5.84 | 1.69 | 0.61 | 0.94 | 3.41 |  |
| Industrial EPA2: | count | 267 | 119 | 153 | 235 | 134 | 128 | 0 |
|  | median | 47 | 1600 | 0.35 | 0.63 | 0.1 | 2.440442 |  |
|  | stdev | 78.27748 | 35772.33 | 0.448945 | 0.679094 | 0.232216716 | 6.913522 |  |
|  | avg | 72.27341 | 14399.39 | 0.504392 | 0.808766 | 0.160402985 | 3.976191 |  |
|  | COV | 1.08 | 2.48 | 0.89 | 0.84 | 1.45 | 1.74 |  |
| Industrial EPA3: | count | 79 | 46 | 11 | 58 | 66 | 59 | 0 |
|  | median | 42 | 2050 | 0.15 | 0.415 | 0.07 | 1.428571 |  |
|  | stdev | 123.2876 | 10685.72 | 0.086865 | 1.275227 | 0.10511294 | 10.87578 |  |
|  | avg | 99.22658 | 6624.261 | 0.176364 | 0.840052 | 0.089506061 | 4.54471 |  |
|  | COV | 1.24 | 1.61 | 0.49 | 1.52 | 1.17 | 2.39 |  |
| Industrial EPA4: | count | 83 | 40 | 6 | 31 | 30 | 28 | 11 |
|  | median | 82 | 1700 | 0.745 | 0.727 | 0.13 | 0.899919 | 0.09 |
|  | stdev | 228.1171 | 355826.1 | 0.593487 | 0.49164 | 0.110776258 | 1.369947 | 0.172104619 |
|  | avg | 161.7952 | 83426.78 | 0.846667 | 0.820903 | 0.151 | 1.422713 | 0.18 |
|  | COV | 1.41 | 4.27 | 0.70 | 0.60 | 0.73 | 0.96 | 0.96 |
| Industrial EPA5: | count | 50 | 53 | 0 | 53 | 53 | 46 | 0 |
|  | median | 157 | 9700 |  | 0.651 | 0.06 | 0.97619 |  |
|  | stdev | 362.5224 | 350412 |  | 0.288527 | 0.081902434 | 1.279433 |  |
|  | avg | 226.96 | 87560.19 |  | 0.679981 | 0.089622642 | 1.28565 |  |
|  | COV | 1.60 | 4.00 |  | 0.42 | 0.91 | 1.00 |  |
| Industrial EPA6: | count | 68 | 20 | 56 | 62 | 55 | 46 | 39 |
|  | median | 311 | 5600 | 0.765 | 1.7 | 0.2 | 2.124324 | 0.28 |
|  | stdev | 471.8263 | 357841.1 | 0.850493 | 0.996633 | 0.256583353 | 5.648586 | 0.21644097 |
|  | avg | 474.3088 | 103595 | 1.012839 | 1.844032 | 0.287272727 | 3.712379 | 0.341794872 |
|  | COV | 0.99 | 3.45 | 0.84 | 0.54 | 0.89 | 1.52 | 0.63 |
| Industrial EPA7: | count | 24 | 27 | 11 | 15 | 2 | 2 | 19 |
|  | median | 114.5 | 600 | 0.5 | 0.29 | 0.0875 | 2.926142 | 0.13 |
|  | stdev | 222.3713 | 15742.3 | 0.496533 | 0.169397 | 0.072831998 | 1.480443 | 0.208332281 |
|  | avg | 182.7083 | 6103.852 | 0.636364 | 0.293333 | 0.0875 | 2.926142 | 0.183684211 |
|  | COV | 1.22 | 2.58 | 0.78 | 0.58 | 0.83 | 0.51 | 1.13 |
| Industrial EPA9: | count | 19 | 3 | 3 | 3 | 3 | 3 | 0 |
|  | median | 130 | 8100 | 0.86 | 0.9 | 0.13 | 0.698276 |  |
|  | stdev | 313.013 | 22162.13 | 0.628517 | 0.378594 | 0.285832119 | 0.62758 |  |
|  | avg | 290.6842 | 16806.67 | 0.936667 | 0.733333 | 0.29 | 0.713941 |  |
|  | COV | 1.08 | 1.32 | 0.67 | 0.52 | 0.99 | 0.88 |  |

| Industrial |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cadmium Filtered (ug/L) | Cadmium part strength (ug/kg) | Chromium Filtered (ug/L) | Chromium part strength (ug/kg) | Copper Filtered (ug/L) | Copper part strength (ug/kg) | Iron, Filtered as Fe (ug/L) | Iron part strength (ug/kg) | Lead Filtered After 1984 (ug/L) | Lead part strength (ug/kg) | Nickel Filtered (ug/L) | Nickel part strength (ug/kg) | Zinc Filtered (ug/L) | Zinc part strength (ug/kg) |
| Industrial overall: | count | 23 | 17 | 20 | 17 | 43 | 37 | 1 | 1 | 28 | 18 | 22 | 18 | 47 | 41 |
|  | median | 0.6 | 6.428571 | 3 | 97.3451327 | 8 | 187.5 | 120 | 2142.857 | 5 | 208.5131 | 6.5 | 54.37666 | 115 | 1078.571 |
|  | stdev | 1.376696 | 6.984716 | 2.82229022 | 330.373769 | 7.948686 | 193.673 |  |  | 34.74557 | 184.088 | 19.05818 | 34.11522 | 1051.013 | 885.8111 |
|  | avg | 1.247391 | 8.21695 | 3.868 | 252.435766 | 10.90698 | 250.4806 | 120 | 2142.857 | 22.44286 | 250.236 | 13.5 | 57.21058 | 302.4043 | 1214.315 |
|  | COV | 1.10 | 0.85 | 0.73 | 1.31 | 0.73 | 0.77 |  |  | 1.55 | 0.74 | 1.41 | 0.60 | 3.48 | 0.73 |
| Industrial EPA1: | count | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | median |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | stdev |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | avg |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | COV |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Industrial EPA2: | count | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | median |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | stdev |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | avg |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | COV |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Industrial EPA3: | count | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | median |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | stdev |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | avg |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | COV |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Industrial EPA4: | count | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | median |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | stdev |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | avg |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | COV |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Industrial EPA5: | count | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | median |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | stdev |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | avg |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | COV |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Industrial EPA6: | count | 8 | 6 | 9 | 7 | 23 | 18 | 1 | 1 | 15 | 6 | 9 | 7 | 27 | 22 |
|  | median | 0.245 | 4.559524 | 2 | 72.2222222 | 10 | 171.4286 | 120 | 2142.857 | 24 | 130.9524 | 2 | 67.85714 | 59 | 1010.119 |
|  | stdev | 0.64127 | 2.326042 | 0.99608121 | 361.644875 | 8.461959 | 252.7441 | #DIV/0! | #DIV/0! | 41.0252 | 301.178 | 3.370625 | 40.26684 | 117.2582 | 978.4108 |
|  | avg | 0.47375 | 4.449735 | 2.59555556 | 223.980726 | 12.82609 | 267.8127 | 120 | 2142.857 | 39.06667 | 255.167 | 3.888889 | 76.68007 | 95.59259 | 1214.847 |
|  | COV | 1.35 | 0.52 | 0.38 | 1.61 | 0.66 | 0.94 | #DIV/0! | #DIV/0! | 1.05 | 1.18 | 0.87 | 0.53 | 1.23 | 0.81 |
| Industrial EPA7: | count | 15 | 11 | 11 | 10 | 20 | 19 | 0 | 0 | 13 | 12 | 13 | 11 | 20 | 19 |
|  | median | 0.8 | 10.60606 | 4 | 125.008444 | 7 | 216.2162 |  |  | 2 | 239.0056 | 11 | 42.01681 | 228 | 1125 |
|  | stdev | 1.499428 | 7.886254 | 3.41919714 | 325.233564 | 6.867927 | 118.755 |  |  | 2.953399 | 105.4614 | 22.60106 | 23.90595 | 1585.155 | 791.7419 |
|  | avg | 1.66 | 10.27179 | 4.90909091 | 272.354295 | 8.7 | 234.0607 |  |  | 3.261538 | 247.7705 | 20.15385 | 44.8209 | 581.6 | 1213.7 |
|  | COV | 0.90 | 0.77 | 0.70 | 1.19 | 0.79 | 0.51 |  |  | 0.91 | 0.43 | 1.12 | 0.53 | 2.73 | 0.65 |
| Industrial EPA9: | count | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | median |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | stdev |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | avg |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | COV |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Freeways |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | TSS (mg/L) | Fecal Coliform (colonies/100 mL) | Ammonia (mg/L) | N02+NO3 (mg/L as N) | Phosphorous Filtered as P (mg/L) | Phos part strength (mg/kg) | Phosphate Ortho as P (mg/L) |
| Freeways overall: | count | 266 | 67 | 76 | 114 | 40 | 38 | 103 |
|  | median | 74.5 | 2000 | 1.045 | 1.18 | 0.1355 | 2.06235 | 0.09 |
|  | stdev | 335.9506 | 22719.15 | 2.152921 | 1.45647 | 1.243959734 | 9.927388 | 0.12495179 |
|  | avg | 140.3939 | 8553.403 | 1.652974 | 1.528947 | 0.4482725 | 5.268375 | 0.128592233 |
|  | COV | 2.39 | 2.66 | 1.30 | 0.95 | 2.78 | 1.88 | 0.97 |
| Freeways EPA 2: | count | 123 | 18 | 7 | 81 | 5 | 4 | 0 |
|  | median | 62.7 | 2000 | 0.85 | 1.37 | 0.094 | 0.719828 |  |
|  | stdev | 155.2013 | 37209.19 | 0.468468 | 1.488158 | 0.142856571 | 7.228919 |  |
|  | avg | 113.4598 | 11338.22 | 0.885714 | 1.770864 | 0.146 | 4.162656 |  |
|  | COV | 1.37 | 3.28 | 0.53 | 0.84 | 0.98 | 1.74 |  |
| Freeways EPA 3: | count | 13 | 0 | 0 | 0 | 14 | 13 | 0 |
|  | median | 24.6 |  |  |  | 0.0346 | 3.684211 |  |
|  | stdev | 51.52832 |  |  |  | 0.073381705 | 15.31415 |  |
|  | avg | 36.20385 |  |  |  | 0.055707143 | 9.065032 |  |
|  | COV | 1.42 |  |  |  | 1.32 | 1.69 |  |
| Freeways EPA 6: | count | 104 | 26 | 56 | 0 | 21 | 21 | 81 |
|  | median | 97 | 1700 | 1.39 |  | 0.197 | 1.644266 | 0.09 |
|  | stdev | 503.3709 | 16364.95 | 2.428435 |  | 1.662014214 | 4.398798 | 0.13321036 |
|  | avg | 182.64 | 7945.385 | 1.962071 |  | 0.781952381 | 3.128677 | 0.148333333 |
|  | COV | 2.76 | 2.06 | 1.24 |  | 2.13 | 1.41 | 0.90 |
| Freeways EPA 7: | count | 26 | 23 | 13 | 24 | 0 | 0 | 22 |
|  | median | 108.5 | 2200 | 0.6 | 0.28 |  |  | 0.05 |
|  | stdev | 129.7533 | 12852.19 | 0.356712 | 0.632223 |  |  | 0.034594122 |
|  | avg | 150.9231 | 7061.304 | 0.734615 | 0.5125 |  |  | 0.055909091 |
|  | COV | 0.86 | 1.82 | 0.49 | 1.23 |  |  | 0.62 |

| Freeways |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cadmium Filtered (ug/L) | Cadmium part strength (ug/kg) | Chromium Filtered (ug/L) | Chromium part strength (ug/kg) | Copper Filtered (ug/L) | Copper part strength (ug/kg) | Iron, Filtered as Fe (ug/L) | Iron part strength (ug/kg) | Lead Filtered After 1984 (ug/L) | Lead part strength (ug/kg) | Nickel Filtered (ug/L) | Nickel part strength (ug/kg) | Zinc Filtered (ug/L) | Zinc part strength (ug/kg) |
| Freeways overall: | count | 30 | 21 | 79 | 46 | 129 | 90 | 4 | 4 | 63 | 59 | 64 | 61 | 104 | 86 |
|  | median | 0.6825 | 4.265302 | 2.3 | 73.7142857 | 10.9 | 191.5175 | 335 | 25428.57 | 1.8 | 295.2381 | 4 | 48.49922 | 51 | 1127.232 |
|  | stdev | 0.938684 | 4.260111 | 1.98884538 | 61.4111403 | 30.73732 | 177.8078 | 355.1995 | 13787.37 | 6.525037 | 517.2064 | 12.02657 | 114.9702 | 236.5392 | 792.8158 |
|  | avg | 0.912233 | 5.81226 | 2.8245443 | 84.7311084 | 20.47519 | 245.4424 | 435 | 24678.3 | 3.961111 | 458.7864 | 8.711203 | 73.97693 | 127.4691 | 1311.708 |
|  | COV | 1.03 | 0.73 | 0.70 | 0.72 | 1.50 | 0.72 | 0.82 | 0.56 | 1.65 | 1.13 | 1.38 | 1.55 | 1.86 | 0.60 |
| Freeways EPA 2: | count | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | median |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | stdev |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | avg |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | COV |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Freeways EPA 3: | count | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | median |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | stdev |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | avg |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | COV |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Freeways EPA 6: | count | 23 | 14 | 72 | 39 | 104 | 66 | 4 | 4 | 45 | 41 | 56 | 53 | 79 | 62 |
|  | median | 0.76 | 5.127373 | 2.3 | 79.026131 | 11.6 | 238.0122 | 335 | 25428.57 | 1.86 | 270 | 4.15 | 50.8 | 50 | 1189.441 |
|  | stdev | 1.055152 | 4.293667 | 2.00805778 | 65.6585847 | 33.43132 | 196.5305 | 355.1995 | 13787.37 | 4.736534 | 585.9229 | 12.63656 | 122.1757 | 264.9733 | 823.7368 |
|  | avg | 1.002913 | 5.977834 | 2.80748611 | 88.7995863 | 23.22404 | 269.7022 | 435 | 24678.3 | 3.776667 | 487.6525 | 9.134232 | 79.84386 | 143.4403 | 1415.825 |
|  | COV | 1.05 | 0.72 | 0.72 | 0.74 | 1.44 | 0.73 | 0.82 | 0.56 | 1.25 | 1.20 | 1.38 | 1.53 | 1.85 | 0.58 |
| Freeways EPA 7: | count | 7 | 7 | 7 | 7 | 25 | 24 | 0 | 0 | 18 | 18 | 8 | 8 | 25 | 24 |
|  | median | 0.6 | 3.030303 | 3 | 66.6666667 | 6 | 169.9234 |  |  | 1.15 | 315.7336 | 3.5 | 31.37066 | 51 | 914.4921 |
|  | stdev | 0.20354 | 4.512151 | 1.91485422 | 16.5715834 | 8.560958 | 82.76483 |  |  | 9.843574 | 313.5131 | 5.970164 | 20.63226 | 91.89079 | 646.9959 |
|  | avg | 0.614286 | 5.481113 | 3 | 62.0638743 | 9.04 | 178.7278 |  |  | 4.422222 | 393.0358 | 5.75 | 35.1085 | 77 | 1042.739 |
|  | COV | 0.33 | 0.82 | 0.64 | 0.27 | 0.95 | 0.46 |  |  | 2.23 | 0.80 | 1.04 | 0.59 | 1.19 | 0.62 |

| Open Space |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | TSS (mg/L) | Fecal Coliform (colonies/100 mL) | Ammonia (mg/L) | N02+NO3 (mg/L as N) | Phosphorous Filtered as P (mg/L) | Phos part strength (mg/kg) | Phosphate Ortho as P (mg/L) |
| open space overall: | count | 152 | 81 | 39 | 77 | 88 | 73 | 11 |
|  | median | 44.5 | 2000 | 0.179 | 0.59 | 0.08 | 0.754717 | 0.14 |
|  | stdev | 836.777 | 334714.3 | 1.227771 | 0.769529 | 0.273686874 | 6.634844 | 0.052414432 |
|  | avg | 261.9481 | 64756.12 | 0.791872 | 0.849792 | 0.175977273 | 2.902958 | 0.145454545 |
|  | COV | 3.19 | 5.17 | 1.55 | 0.91 | 1.56 | 2.29 | 0.36 |
| open space EPA 1: | count | 5 | 6 | 4 | 5 | 0 | 0 | 4 |
|  | median | 24 | 2150 | 0.165 | 0.23 |  |  | 0.16 |
|  | stdev | 14.89295 | 24384.23 | 0.061644 | 0.137477 |  |  | 0.039475731 |
|  | avg | 26.6 | 14277.33 | 0.17 | 0.26 |  |  | 0.1725 |
|  | COV | 0.56 | 1.71 | 0.36 | 0.53 |  |  | 0.23 |
| open space EPA 2: | count | 32 | 8 | 4 | 26 | 18 | 14 | 0 |
|  | median | 22 | 4700 | 0.245 | 1.27 | 0.06 | 1.867978 |  |
|  | stdev | 41.85876 | 9184.459 | 0.236132 | 1.01837 | 0.179677452 | 10.04815 |  |
|  | avg | 39.28438 | 9202.75 | 0.2975 | 1.172692 | 0.163888889 | 5.500018 |  |
|  | COV | 1.07 | 1.00 | 0.79 | 0.87 | 1.10 | 1.83 |  |
| open space EPA 4: | count | 18 | 14 | 2 | 17 | 17 | 15 | 0 |
|  | median | 300 | 14600 | 1.65 | 0.49 | 0.16 | 0.714286 |  |
|  | stdev | 310.0112 | 18324.83 | 0.212132 | 0.744813 | 0.142103421 | 8.770024 |  |
|  | avg | 369.6111 | 17927.86 | 1.65 | 0.782941 | 0.199411765 | 3.180136 |  |
|  | COV | 0.84 | 1.02 | 0.13 | 0.95 | 0.71 | 2.76 |  |
| open space EPA 5: | count | 17 | 20 | 0 | 20 | 14 | 11 | 0 |
|  | median | 118 | 1850 |  | 0.63 | 0.08 | 0.728346 |  |
|  | stdev | 506.7405 | 86684.88 |  | 0.436523 | 0.291757916 | 6.323719 |  |
|  | avg | 285.2353 | 26352.5 |  | 0.7367 | 0.199285714 | 3.203558 |  |
|  | COV | 1.78 | 3.29 |  | 0.59 | 1.46 | 1.97 |  |
| open space EPA 6: | count | 73 | 33 | 22 | 2 | 32 | 26 | 0 |
|  | median | 20 | 500 | 0.143 | 0.995 | 0.07 | 0.459496 |  |
|  | stdev | 1160.734 | 517475.8 | 0.419843 | 0.572756 | 0.380467285 | 2.962338 |  |
|  | avg | 330.7808 | 130543 | 0.251045 | 0.995 | 0.16925 | 1.795811 |  |
|  | COV | 3.51 | 3.96 | 1.67 | 0.58 | 2.25 | 1.65 |  |
| open space EPA 9: | count | 7 | 0 | 7 | 7 | 7 | 7 | 7 |
|  | median | 257.13 |  | 3.1 | 0.56 | 0.15 | 0.775929 | 0.12 |
|  | stdev | 248.8972 |  | 1.430453 | 0.309023 | 0.054116277 | 0.16367 | 0.055075705 |
|  | avg | 396.7157 |  | 2.884286 | 0.515714 | 0.134285714 | 0.754771 | 0.13 |
|  | COV | 0.63 |  | 0.50 | 0.60 | 0.40 | 0.22 | 0.42 |

| Open Space |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cadmium Filtered (ug/L) | Cadmium part strength (ug/kg) | Chromium Filtered (ug/L) | Chromium part strength (ug/kg) | Copper Filtered (ug/L) | Copper part strength (ug/kg) | Iron, Filtered as Fe (ug/L) | Iron part strength (ug/kg) | Lead Filtered After 1984 (ug/L) | Lead part strength (ug/kg) | Nickel Filtered (ug/L) | Nickel part strength (ug/kg) | Zinc Filtered (ug/L) | Zinc part strength (ug/kg) |
| open space overall: | count | 0 | 0 | 1 | 1 | 7 | 5 | 18 | 16 | 0 | 0 | 0 | 0 | 3 | 1 |
|  | median |  |  | 3.4 | 61.5384615 | 1.7 | 645 | 255 | 2130.573 |  |  |  |  | 103 | 101.8201 |
|  | stdev |  |  |  |  | 5.221117 | 274.0273 | 995.6863 | 7900.497 |  |  |  |  | 39.15354 |  |
|  | avg |  |  | 3.4 | 61.5384615 | 3.684286 | 497.8135 | 503.2778 | 5523.364 |  |  |  |  | 116 | 101.8201 |
|  | COV |  |  |  |  | 1.42 | 0.55 | 1.98 | 1.43 |  |  |  |  | 0.34 |  |
| open space EPA 1: | count | 0 | 0 | 0 | 0 | 2 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 |
|  | median |  |  |  |  | 0.895 | 645 |  |  |  |  |  |  |  |  |
|  | stdev |  |  |  |  | 1.138442 |  |  |  |  |  |  |  |  |  |
|  | avg |  |  |  |  | 0.895 |  |  |  |  |  |  |  |  |  |
|  | COV |  |  |  |  | 1.27 |  |  |  |  |  |  |  |  |  |
| open space EPA 2: | count | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | median |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | stdev |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | avg |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | COV |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| open space EPA 4: | count | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | median |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | stdev |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | avg |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | COV |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| open space EPA 5: | count | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | median |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | stdev |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | avg |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | COV |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| open space EPA 6: | count | 0 | 0 | 1 | 1 | 5 | 4 | 18 | 16 | 0 | 0 | 0 | 0 | 2 | 1 |
|  | median |  |  | 3.4 | 61.5384615 | 1.8 | 528.4014 | 255 | 2130.573 |  |  |  |  | 94 | 101.8201 |
|  | stdev |  |  |  |  | 5.926213 | 301.8189 | 995.6863 | 7900.497 |  |  |  |  | 12.72792 |  |
|  | avg |  |  |  |  | 4.8 | 461.0168 | 503.2778 | 5523.364 |  |  |  |  | 94 |  |
|  | COV |  |  |  |  | 1.23 | 0.65 | 1.98 | 1.43 |  |  |  |  | 0.14 |  |
| open space EPA 9: | count | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | median |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | stdev |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | avg |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | COV |  |  |  |  |  |  |  |  |  |  |  |  |  |  |