R. Pitt, J. Voorhees

8 December 2022

Urban Tree Rainfall Interception Measurements

Version 8: Winter, Spring, Summer, and Fall Seasons

Contents

General Description	1

Example Rain Data	3

Tree Canopy Runoff Interception Equations	4

Algorithm Summary	5

References	8

# General Description

Tree canopy runoff interception is a source area feature in WinSLAMM.  Unlike typical WinSLAMM stormwater control practices, you can only compare the results of a model run with and without tree canopy interception by running two separate models – one with and one without tree canopy interception.

Please note that there is also a possibility for double counting some of these benefits.  For example, calibrated stormwater models rely on monitored outfall flow measurements of existing areas.  These areas have varying amounts of trees through their landscapes.  Adding additional interception to these calibrated models can result in improper estimates of runoff.  However, if new trees are planted in an area, interception benefits may increase.  The following figures are examples of monitored medium density residential areas used in WinSLAMM calibrations showing the contrast of mature trees in older areas and the few young trees in new developments.  These residential areas were separated based on age of development to account for the differences in vegetation during the model calibration process.  However, the outfall monitored runoff characteristics did not indicate any differences between the old and new developments, beyond which was explained by differences in directly connected impervious area types and other land surface areas.

| Residential area with few isolated trees | Older medium density residential area with more mature trees |
| --- | --- |

If a tree is located in a pervious area of the watershed (over lawns or other non-paved areas), interception may not affect outfall runoff quantities much; any un-intercepted rainfall (throughfall) is likely to be infiltrated with or without the trees.  However, trees likely maintain good soil characteristics and minimize compaction, which would improve the infiltration of rainfall.  The largest hydrological benefit of urban trees would be when directly connected impervious areas (such as roofs, walkways, parking areas, and streets) are heavily covered by an overstory of trees.  If tree-covered impervious areas are directly connected to the drainage system, these benefits would be the greatest, but if the tree-covered impervious areas drain to pervious areas (such as disconnected roofs or walks surrounded by lawns), the benefits would be lower.  Obviously, trees add substantially to the quality of life in urban areas, but nuisance conditions and increase public works leaf removal activities may occur.

This algorithm description report includes the data plots and resulting equations used to describe throughfall under urban trees (canopy interception) to quantify some of these hydrologic benefits for inclusion in WinSLAMM.  It also lists the references that provide the data for the equations and shows how the tree canopy data should be entered into WinSLAMM.

The experiments used to develop the equations were conducted to comprehensively examine canopy interception by direct measurements of throughfall under isolated or low density stands of mature urban deciduous and evergreen trees in the Southeast of the US.  (Bob, what reference should go here?) These measurements resulted in throughfall data for 55 (oak) to 75 (pine) rains over all seasons to determine statistically significant relationships for use in the WinSLAMM urban stormwater quality model.

# Example Rain Data

The following plot is the accumulative rainfall at the background location (surrounded by grass) vs. the accumulative rainfall measured under the pine and oak trees for rainfall between December 7 and 11, 2018:

It is obvious that the measured rain under the pines had little difference compared to the background rainfall, while the oak had a substantial reduction.  The total rain for this event was about 3.32 inches. The steepest portion of the accumulated rain curve indicated about 2.1 inches over 7.25 hours, for a fairly constant rain intensity of about 0.29 inches per hour.

This rain interception measurement test was supplemented with many more rains through January 24, 2020.  In the Birmingham area, about 50 rains occur per year having >0.1 inches of rain (and about 100 rains >0.01 inches).  This initial rain was one of the largest expected, but we usually receive a few rains between 2- and 5-inches total per year.  These large amounts of data enabled the significant factors affection interception of urban trees to be identified and quantified.  The factors examined by factorial analyses included:  tree type, season (tree canopy coverage), rain intensity and depth, along with wind speed.

A 23 full factorial analysis was conducted and found that all three factors (season, tree type, and rain category) were significant when determining the throughfall (rain depth under the trees vs. rain depth at the grass location).  The tree type (oak vs. pine) had the greatest effects on the throughfall, followed by the rain depth, while the seasonal differences had only marginally significant effects.

# Tree Canopy Runoff Interception Equations

The following scatterplots used in WinSLAMM are for the total rain depths under the pine tree and the oak tree plotted against the rain depths at the grass gage, for the winter, spring, and summer seasons.  The plots and ANOVA statistical tests were conducted on log10 transformed rain depth data, indicating highly significant regressions.  The pine data for winter, spring, and summer periods did not result in significant constant (intercept) terms, so those regressions only have a slope coefficient term, while the oak data (and the fall pine data) had both significant intercept and slope coefficients.  The scatterplots show the fitted regression lines along with the actual data.  The residual analyses indicated satisfactory patterns (example shown below for winter pine observations).

| Winter pine log10 total rain (in.) = 1.07 Winter grass log10 total rain (in.) | Winter oak log10 total rain (in.) = - 0.203 + 1.17 Winter grass log10 total rain (in.) |
| --- | --- |
| Spring pine log10 total rain (in.) = 1.26 Spring grass log10 total rain (in.) | Spring oak log10 total rain (in.) = - 0.260 + 1.06 Spring grass log10 total rain (in.) |
| Summer pine log10 total rain (in.) = 1.51 Summer grass log10 total rain (in.) | Summer oak log10 total rain (in.) = - 0.215 + 1.17 Summer grass log10 total rain (in.) |
| Fall pine log10 total rain (in.) = -0.0871 +1.255 fall grass log10 total rain (in.) |  |

# Algorithm Summary

Literature reviews have concluded that the interacting mechanisms of urban trees affecting urban hydrology are poorly understood.  Past canopy throughfall measurements of urban trees have identified important differences between tree types. Projected runoff volume reductions due to extensive use of urban trees have been found to be about 10 to 20%.  Field studies have also concluded that stemflow is usually a small portion of the total tree runoff yield to runoff (usually <10% of the canopy throughfall).  Soil characteristics under urban trees are also expected to affect understory runoff yields, with trees expected to improve soil structure (decreased compaction and increased organic matter).

Highly significant regression equations relating rain depth and throughfall were developed for conifer and deciduous trees for the different seasons for implementation in WinSLAMM. As noted previously, tree interception effects on throughfall in stormwater management is only relevant for newly planted trees that shade directly connected impervious areas. Counting the benefits of existing trees in a calibrated model likely would result in double-counting the benefits. Also, the benefits of new trees shading uncompacted soils during small and intermediate rains are likely small as the throughfall would likely be almost completely infiltrated, as would the total rainfall for these areas. During large rains, the canopy interception fraction is much reduced, also resulting in minimal differences in runoff compared to uncompacted soil areas having no trees. WinSLAMM was therefore modified to directly calculate the benefits of trees over directly connected impervious areas, as shown on the following figure.

# Tree Canopy Data Entry into WinSLAMM

The user can enter tree canopy data, which is the percent of the source area with a deciduous and/or a coniferous tree canopy, for all source areas that are directly connected or that drain to a directly connected area.  You cannot enter tree canopy information for pervious areas because these areas already include some tree canopy cover as part of the runoff coefficients developed for these source areas, as described earlier.

Below is a conceptualization of a set of land uses.  The strip commercial land use has tree areas that overhang the parking area that are outlined in red.  The percentage of these overhang areas would be, for this example, the value, for each tree type, that is entered into the source area form to represent the tree canopy coverage over an impervious area.

Tree canopy interception shading over directly connected impervious areas (shown in red outline).

The following screen shot of a paved parking area in WinSLAMM shows how the tree canopy shading values are entered for directly connected areas.

Paved parking area information input screen.

The following plots show modeled cumulative runoff for one year of rains for Birmingham, AL, from a one-acre paved parking area having varying amounts of shading by tree canopies.  With 100% shading, the deciduous trees may provide about 35% reductions in runoff from paved areas.  The benefits are linear, with half this maximum benefit with 50% canopy shading, for example.  A similar plot for conifers would show much smaller benefits, especially for the early months of the year. The maximum benefit of shading of impervious areas by conifer trees are about half of the canopy interception benefit of the oak tree.

There relatively low runoff reduction values are in contrast with rural forested areas where the runoff amounts of heavily wooded areas (both conifers and deciduous trees) is very small.  These major forest benefits are mostly associated with the forest duff (thick layers of partially and completely decomposed organic material) beneath the trees and large infiltration rates.  In urban areas (especially for thinly planted or isolated trees, if relatively young and with common leaf removal by homeowners), the benefits of trees on underlying soils is important, but much reduced compared to thick stands of mature trees having deep layers of organic material covering the soil.  Duff has no effect on paved areas, although it may build up near the trunk in tree planter boxes or other small areas.  Therefore, the main effect of urban trees on urban hydrology is the limited canopy interception amounts.

# References

Alves, P.L., K.T.M. Formiga, and M.A.B. Traldi. “Rainfall interception capacity of tree species used in urban afforestation.” Urban Ecosystems. 21:697–706. 2018.

Asadian, Y. and M. Weiler. “A new approach in measuring rainfall interception by urban trees in coastal British Columbia.” Water Qual. Res. J. Can. Volume 44, No. 1, 16-25. 2009

Baptista, M.D., S. J. Livesley, E.G. Parmehr, M. Neave, and M. Amati. “Variation in leaf area density drives the rainfall storage capacity of individual urban tree species.” Hydrological Processes. 32: 3729–3740. 2018.

Bean, R. and R. Pitt “Advancing stormwater beneficial uses: ET mapping in urban areas.” Water Environmental and Water Resources Congress 2012. ASCE-EWRI. Congress Proceedings. Albuquerque, NM, May 20-25, 2020.    http://unix.eng.ua.edu/~rpitt/Publications/6_Urban_Hydrology/ET%20in%20urban%20areas%20Bean_Pitt_EWRI2012_1.pdf

Berland, A., S.A. Shiflett, W.D. Shuster, A.S. Garmestani, H.C. Goddardc, D.L. Herrmann, and M.E. Hopton. “The role of trees in urban stormwater management.” Landscape and Urban Planning. 162, 167–177. 2017.

Gonzalez-Sosa, E., I. Braud, R.B. Piña, C.A.M. Loza, N.M.R. Salinas, and C.V. Chavez. “A methodology to quantify ecohydrological services of street trees.” Ecohydrology & Hydrobiology. 17, 190–206. 2017.

Guevara-Escobar, A., E. González-Sosa, C. Véliz-Chávez, Ventura-Ramos, and M. Ramos-Salinas. “Rainfall interception and distribution patterns of gross precipitation around an isolated Ficus benjamina tree in an urban area.” Journal of Hydrology. 333, 532– 541. 2007.

Kermavnar, J. and U. Vilhar. “Canopy precipitation interception in urban forests in relation to stand structure.” Urban Ecosyst. 20:1373–1387. 2007.

Kirnbauer, M.C., B.W. Baetz, and W.A. Kenney. “Estimating the stormwater attenuation benefits derived from planting four monoculture species of deciduous trees on vacant and underutilized urban land parcels.” Urban Forestry & Urban Greening. 12, 401–407. 2013.

Kuehler, E. J. Hathaway, and A. Tirpak. “Quantifying the benefits of urban forest systems as a component of the green infrastructure stormwater treatment network.” Ecohydrology.10: e1813. 2017.

Livesley, S.J., B. Baudinette, and D. Glover. “Rainfall interception and stem flow by eucalypt street trees–The impacts of canopy density and bark type.” Urban Forestry & Urban Greening. 13, 192–197. 2014.

Livesley, S.J., E. G. McPherson, and C. Calfapietra. “The urban forest and ecosystem services: Impacts on urban water, heat, and pollution cycles at the tree, street, and city scale.” Journal of Environmental Quality. January 11, 2016

Nytch, C.J., E.J. Meléndez-Ackerman, M. Pérez, and J.R. Ortiz-Zayas. “Rainfall interception by six urban trees in San Juan, Puerto Rico.” Urban Ecosystems. 22:103–115. 2019.

Rahmana, M.A., A. Moserb, M. Andersona, C. Zhangb, T. Rötzerb, and S. Pauleita. “Comparing the infiltration potentials of soils beneath the canopies of two contrasting urban tree species.” Urban Forestry & Urban Greening. 38, 22–32. 2019.

Schooling, J. T. and D. E. Carlyle-Moses. “The influence of rainfall depth class and deciduous tree traits on stemflow production in an urban park.” Urban Ecosyst. 18:1261–1284. 2015.

Smets, V., W. Akkermans, B. Verbeiren, M. Hermy, and B. Somers. “Ex-situ estimation of interception storage capacity of small urban plant species.” Journal of Hydrology. 572, 869–883. 2019.

Van Stan II, J.T., D.F. Levia Jr., and R.B. Jenkins. “Forest canopy interception loss across temporal scales: Implications for urban greening initiatives.” The Professional Geographer, 67:1, 41-51. 2015.

Xiao, Q., E.G. McPherson, S.L. Ustin, M.E. Grismer, and J.R. Simpson. “Winter rainfall interception by two mature open-grown trees in Davis, California.” Hydrological Processes. 14, 763-784. 2000.

Zabret, K., J. Rakovec, and M. Šraj. “Influence of meteorological variables on rainfall partitioning for deciduous and coniferous tree species in urban area.” Journal of Hydrology. 558, 29–41. 2018.