R. Pitt

December 13, 2022

Stormwater Particulate Strengths by Particle Size

and WinSLAMM Calculations

Contents

Summary	2

Introduction	3

Pollutant Strengths for Urban Area Particulate Samples	4

Particulate Pollutant Strengths for Stormwater Source Area Sheetflow Samples	7

Particulate Pollutant Strengths for Sediment Samples Collected from Inlet Sumps	9

Particulate Pollutant Strengths for Stormwater Outfall Samples	14

Particulate Pollutant Strengths for Sediment Samples Collected from Small Urban Creeks	24

Analyses of Particulate Pollutant Strengths by Particle Size	37

Specific Gravity	40

COD, TOC, and Nutrients	42

Heavy Metals	47

PAHs	63

Chlordane and PCBs	78

Recent SERDP Monitoring of Particulate Strengths of Metals, PAHs, and PFAS Compounds	79

Heavy Metals	80

PAHs	82

PFAS	89

Example WinSLAMM Calculations	90

Conclusions	95

Specific Gravity and Volatile Solids	95

COD and Nutrients	95

Heavy Metals	96

PAHs	96

Overall Particulate Strength Summary Tables	96

References Cited and Bibliography	102

Appendix A: Characteristics of Stormwater PAHs and PPCPs Affecting their Portioning to Particulates	107

# Summary

Currently, WinSLAMM routes many particle sizes from source areas, through stormwater controls, and along the drainage system to the outfall. However, only a lumped particulate strength value is used for the sum of all particle sizes to determine the stormwater concentrations and loads. As part of the current SERDP project, WinSLAMM is being expanded to incorporate particulate strength values for the particle sizes already being considered in the model, instead of using a single bulk value for all sizes for a specific contaminant.

This whitepaper summarizes available particulate strength data by different particle size ranges for several constituents, from past research by Pitt’s research group at the University of Alabama, along with the first phase SERDP (2018) project data collected at Paleta Creek, in San Diego, and also includes summaries of the second phase SERDP (2022) project at San Diego, Puget Sound, WA, and Lubbock, TX. These data were normalized to obtain ratio factors for each particle size range that are to be multiplied with the calibrated lumped particle values to obtain particulate strengths for each range. This is in leu of using specific particle size range particulate strengths for each land use and source area in the model, as that detailed data are not available. These normalized factors are applied to the available lumped particulate strength values currently used in the model.

Particulate strength data presented here are used as an example of the factors to be used in the model. As with all stormwater characteristics, there are uncertainties associated with the parameters. For example, the coefficients of variation (COV, or the ratio of the standard deviation to the mean) range from about 0.6 to 1.3. These COV values can be optionally used by a model user in the Monte Carlo portion of the model to better represent the typical variability in stormwater characteristics. Also, locally collected data are preferred for a site rather than using these factors summarized here.

The data summarized in this whitepaper are associated with samples collected from sheetflows at source areas (mainly paved parking areas and roof runoff, plus mixed flows at stormdrain inlets), from sediment samples collected at sumps at stormdrain inlets (perhaps the most straight-forward way of collecting these data), from bulk stormwater outfall samples (most of the data), and from sediments from small urban creeks. The data are further separated into constituent categories: specific gravity and volatile solids; COD, TOC, and nutrients; heavy metals; PAHs; and chlordane and total PCBs. The particulate strength values from the research reports are presented, along with the resulting normalized factors.

There are some obvious trends in the values (and factors) when compared to particle size, with a few examples listed below:

- Specific gravity is high for small particles and decreases for large particles (and with increased organic material).

- Mixed residential and commercial area TOC values are high for small particles but decrease for large particles. In contrast, industrial area TOC values have the opposite trend, indicating the likely presence of oily debris as part of the large industrial particles.

- Many of the heavy metals behave similarly to the TOC trends, with high concentrations associated with the small particles, with lower values for large particles, except for industrial samples indicating higher values for larger sizes.

- Most of the PAHs have slightly higher values for small particles, then decrease for larger particles, except for the largest particles (including large organic material and oily debris) with very high PAH values.

- Total PCBs have high values for small particles and low values for the larger particles.

The factors for the constituents and particle size ranges can be organized as a matrix as a new parameter file, possibly linked to pollutant concentration parameter files. It may also be worthwhile to include a new solids and pollutant category for floatable material at this time, along with seasonal factors to reflect (at least) phosphorus particulate strength (and maybe even PSD) changes with time (in later model updates). An additional white paper addresses seasonal trends.

# Introduction

Knowing the distribution of pollutants associated with different sized stormwater particles allows more accurate determinations of their sources, transport, and control. This summary will be used to modify WinSLAMM by expanding the particulate strength characteristics by particle size. WinSLAMM currently tracks particulates based on particle size distributions but only uses a single overall particulate strength value for all sizes. Stormwater controls usually preferentially remove the largest particles, resulting in changes in bulk particle strength values after treatment. Therefore, using particulate strength values for different particle sizes will result in more accurate calculations assisting stormwater management decisions and knowledge of the fates of these particulates.

This memo reviews stormwater pollutant strength data for different particle sizes to support these enhanced WinSLAMM calculations as part of the current SERDP project. Most of these data are from past master’s theses and Ph.D. dissertations (Bathi 2008, Cai 2013, Eppakayala 2015, Goodson 2013, Khambhammettu 2005, and Morquecho 2005) from the University of Alabama, along with past reviews (Pitt, et al. 2005a and 2005b) and the first phase of this SERDP project (SERDP 2018). These data are supplemented by additional data obtained during the second SERDP project phase.

Pollutant strengths are the contaminant concentrations associated with the particulate matter in the stormwater. Particulate strengths are determined by calculating the pollutant concentration only associated with the particulates (measured as TSS or SSC, depending on how the sample was collected and analyzed) in the runoff water. Particulate strengths are calculated by the following equation:

total conc.-filterable conc.particulate solids conc.

As an example, if the total copper concentration was 50 µg/L, the filterable (“dissolved”) copper concentration was 10 µg/L, and the TSS concentration was 150 mg/L, the particulate strength for this sample would be:

50 µgCuL-10µgCuL150 mg/L =0.26 µgCumg solids = 260 µg Cu/g solids =

260 mg Cu/kg solids (also = 260 ppm, the usual units for soil analyses)

This value is therefore the pollutant concentration associated with the particulate matter in the runoff sample. This calculation can be used based on stormwater samples divided into separate particle sizes.

It is also possible to calculate particulate strength data using particulate samples, such as sediment collected from stormwater inlets, creek sediments, sediments from sumps in stormwater controls, and even soil samples (including vacuumed particulates from different paved areas). The particulate samples are sieved before the chemical analyses to obtain data for different particle size ranges. It is more difficult to obtain data for very small particles as the smallest sieves available are about 20 μm. The membrane filters used for separating stormwater particulates are generally 0.45 μm (as used for most TSS vs. TDS analyses), while larger and smaller specialized membrane filters are also available. However, it is more difficult to obtain data for large particles from water samples due to their low abundance. The following summaries therefore combine data from water samples which emphasize small particles and from sediment data which emphasize large particles.

# Pollutant Strengths for Urban Area Particulate Samples

Pitt, et al. (2005a and 2005b) summarized particulate strength data from a number of prior research studies. These studies collected dry soil samples from various urban surfaces for gravimetric, particle size, and quality analyses. In general, they were vacuumed from hard surfaces in very specific patterns in order to determine their accumulation rates and quality.

The data summarized in Tables 1 and 2 are average particulate strength values for different pollutants, land uses, and source areas. Generally, each sample comprised 12 to 40 subsamples for each collection. Collections were usually made several times a week for up to three years from 5 to 10 areas per project. Therefore, each project typically included data from hundreds to thousands of samples. Normally, each sample was separated into several particle sizes using standard sieves. Each sample fraction was retained, usually in small plastic bags. Seasonal composites were then made of all similar sized samples for each source area for the chemical analyses. The data shown in this section are averages of the chemical analyses for the smallest particle sizes (generally <125 μm) for all samples representing each land use for each project. The smallest particle sizes are of the most interest here as they are the particles that are most mobile in stormwater drainage systems and affect receiving waters.

These data show the variations in chemical quality between particles from different land uses and source areas. Typically, the particulate strengths increase as the use of an area becomes more intensive, but the variations are slight for different locations. Seasonal variations may also be evident for nutrients that are affected by changes in leaf drop and accumulations in the drainage system.

Table 1.  Summary of observed street dirt mean chemical quality for small particles (mg constituent/kg solids), collected by dry vacuuming (Pitt, et al. 2005a).

| Constituent | Residential | Commercial | Industrial |
| --- | --- | --- | --- |
| P | 620 (4) 540 (6) 1,100 (5) 710 (1) 810 (3) | 400 (6) 1,500 (5) 910 (1) | 670 (4) |
| TKN | 1,030 (4) 3,000 (6) 290 (5) 2,630 (3) 3,000 (2) | 1,100 (6) 340 (5) 4,300 (2) | 560 (4) |
| COD | 100,000 (4) 150,000 (6) 180,000 (5) 280,000 (1) 180,000 (3) 170,000 (2) | 110,000 (6) 250,000 (5) 340,000 (1) 210,000 (2) | 65,000 (4) |
| Cu | 162 (4) 110 (6) 420 (2) | 130 (6) 220 (2) | 360 (4) |
| Pb | 1,010 (4) 1,800 (6) 530 (5) 1,200 (1) 1,650 (3) 3,500 (2) | 3,500 (6) 2,600 (5) 2,400 (1) 7,500 (2) | 900 (4) |
| Zn | 460 (4) 260 (5) 325 (3) 680 (2) | 750 (5) 1,200 (2) | 500 (4) |
| Cd | <3 (5) 4 (2) | 5 (5) 5 (2) |  |
| Cr | 42 (4) 31 (5) 170 (2) | 65 (5) 180 (2) | 70 (4) |

References:

(1) Bannerman, et al. 1983 (Milwaukee, WI)  <31m

(2) Pitt 1979  (San Jose, CA)  <45 m

(3) Pitt 1985  (Bellevue, WA)  <63 m

(4) Pitt and McLean 1986 (Toronto, Ontario)  <125 m

(5) Pitt and Sutherland 1982  (Reno/Sparks, NV)  <63 m

(6) Terstriep, et al. 1982 (Champaign/Urbana, IL)  <63 m

Increasing concentrations of heavy metals with decreasing particle sizes was also evident, for those studies that included particle size information. However, phosphorus concentrations typically increased with increasing particle sizes associated with increased organic material with the large particles. Only the particulate quality of the smallest particle sizes are shown on these tables because they best represent the particles that are washed off during rains.

Table 2.  Summary of observed particulate quality for other source areas (means for <125 m particles) (mg constituent/kg solids), sheetflow samples during rains (Pitt and McLean 1986).

|  | P | TKN | COD | Cu | Pb | Zn | Cr |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Residential/Commercial Land Uses Roofs Paved parking Unpaved driveways Paved driveways Dirt footpath Paved sidewalk Garden soil Road shoulder | 1,500 600 400 550 360 1,100 1,300 870 | 5,700 790 850 2,750 760 3,620 1,950 720 | 240,000 78,000 50,000 250,000 25,000 146,000 70,000 35,000 | 130 145 45 170 15 44 30 35 | 980 630 160 900 38 1,200 50 230 | 1,900 420 170 800 50 430 120 120 | 77 47 20 70 25 32 35 25 |
| Industrial Land Uses Paved parking Unpaved parking/storage Paved footpath Bare ground | 770 620 890 700 | 1,060 700 1,900 1,700 | 130,000 110,000 120,000 70,000 | 1,110 1,120 280 91 | 650 2,050 460 135 | 930 1,120 1,300 270 | 98 62 63 38 |

Source:  Pitt and McLean  1986  (Toronto, Ontario)

The above data are not separated by particle size but do indicate the overall range of pollutant strengths for different source areas (lead is high compared to current samples). The following sets of data show the particulate strengths by particle size which will be used for the normalizing calculations in this whitepaper.

## Particulate Pollutant Strengths for Stormwater Source Area Sheetflow Samples

Morquecho (2005) collected a number of sheetflow samples from residential and commercial areas in Birmingham and Tuscaloosa, AL, for pollutant associations with particle size as part of her Ph.D. dissertation research at the University of Alabama. In contrast to most other researchers, she found that less than 50% of Zn, Cu, Cd, or Pb were associated with the particulate fraction of the stormwater, with most of these constituents associated with the filtered stormwater fraction. Table 3 summarizes particulate strengths by particle size for selected stormwater pollutants.

Table 3. Average pollutant strengths for different particle sizes for roof runoff samples (N=5) (Morquecho 2005)

|  | Total phosphorus |  | COD |  | Zinc |  | Copper |  | Lead |  | Cadmium |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | mg/kg SS | COV | mg/kg SS | COV | mg/kg SS | COV | mg/kg SS | COV | mg/kg SS | COV | mg/kg SS | COV |
| >250 μm | 4,400 | 0.77 | 300,000 | 0.91 | 2,800 | 1.15 | 150 | 0.28 | 630 | 0.35 | 13 | 1.24 |
| 106-250 μm | 1,400 | na | 510,000 | 0.95 | 580 | 1.38 | 6.2 | na | 53 | 0.83 | 1.9 | na |
| 45-106 μm | 1,400 | 1.30 | 520,000 | 0.43 | na | na | 1,900 | na | 200 | 1.22 | na | na |
| 10-45 μm | 3,000 | 0.56 | 840 | 1.18 | 5,589 | 1.41 | 310 | na | 150 | 1.40 | 4.6 | na |
| 2-10 μm | na | na | 370,000 | na | 2,000 | 0.40 | na | na | 120 | na | 5.4 | 1.03 |
| 1-2 μm | 5,200 | na | 630,000 | 0.50 | na | na | na | na | 390 | na | 22 | 0.68 |
| 0.45-1 μm | 6,800 | 1.69 | 1,212 | 0.85 | 45,000 | 1.12 | 4,900 | na | 5,100 | 0.70 | 87 | 1.13 |

Note: na=too few detectable observations for calculation

Table 4 lists the average percentage of the heavy metals analyzed that occurred as either ionic or bound forms for the filtered samples.  Most of the zinc, cadmium and lead were not present in free ionic forms, but were bound to colloids or organic matter whose bonds could be broken by exposure to UV light.  Only copper occurred in mostly ionic forms.  These results differ from previous results from this laboratory (Johnson, et al. 2003), in which only cadmium was mostly particulate bound (70%) and about 50% of copper was in ionic forms. The other metals were mostly found in their ionic forms.  Results from these types of tests can be highly variable due to low metals concentrations in the filterable fractions.

Table 4.  Average percentage of metals occurring as ionic or bound forms for last four samples (metals measured by ICP-MS) (Morquecho 2005)

|  | Average % ionic | Average % bound |
| --- | --- | --- |
| Zinc | 15 | 85 |
| Copper | 70 | 30 |
| Cadmium | 10 | 90 |
| Lead | 12 | 88 |

## Particulate Pollutant Strengths for Sediment Samples Collected from Inlet Sumps

Table 5 lists average particulate strength results from sediment collected from stormwater inlets and catchbasins in Bellevue, WA (Pitt 1985). These values are comparable to recent data, with the expected exception for lead. Current lead sediment concentration values are about 1/10th of these older lead values, while the other particulate strengths shown on this table are similar to more current data.

Table 5. Chemical Quality of Bellevue, WA, Inlet Structure Sediment (mg constituent/kg total solids) (Pitt 1985)

| Particle Size (µm) | COD | TKN | TP | Pb* | Zn |
| --- | --- | --- | --- | --- | --- |
| <63 | 160,000 | 2,900 | 880 | 1,200 | 400 |
| 61-125 | 130,000 | 2,100 | 690 | 870 | 320 |
| 125-250 | 92,000 | 1,500 | 630 | 620 | 200 |
| 250-500 | 100,000 | 1,600 | 610 | 560 | 200 |
| 500-1,000 | 140,000 | 1,600 | 550 | 540 | 200 |
| 1,000-2,000 | 250,000 | 2,600 | 930 | 540 | 230 |
| 2,000-6,350 | 270,000 | 2,500 | 1,100 | 480 | 190 |
| >6,350 | 240,000 | 2,100 | 760 | 290 | 150 |

* these lead values are much higher than are found for current samples due to the decreased use of leaded gasoline since 1981 when these samples were collected.

Khambhammettu (2005) collected sediment samples from the UpFlo Filter sump that was located in the City Hall parking lot in Tuscaloosa, AL, as part of his MSCE thesis research at the University of Alabama. Table 6 is a summary of the particulate strengths by particle size. Except for lead, these particulate strengths are similar to those noted above for the Bellevue, WA (Pitt 1985) samples.

Table 6. Average Particulate Strengths of Sediment Collected from UpFlo Filter Sump (Khambhammettu 2005)

| Sediment Size Range (µm) | COD (mg/kg) | P (mg/kg) | Ca (mg/kg) | Mg (mg/kg) | Fe (mg/kg) | Cu (mg/kg) | Cr (mg/kg) | Pb (mg/kg) | Zn (mg/kg) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| <75 | 233,000 | 3,580 | 117 | 15,600 | 6,050 | 190 | 21.2 | 80.4 | 1,340 |
| 75-150 | 129,000 | 1,620 | 177 | 25,300 | 4,960 | 99.8 | 17.4 | 70.3 | 958 |
| 150-250 | 35,500 | 511 | 158 | 22,900 | 3,010 | 48.2 | 8.0 | 34.9 | 501 |
| 250-425 | 60,100 | 315 | 134 | 18,600 | 2,790 | 33.6 | 6.7 | 28.4 | 539 |
| 425-850 | 45,000 | 496 | 146 | 19,700 | 2,290 | 22.1 | 3.7 | 23.7 | 270 |
| 850-2,000 | 29,200 | 854 | 312 | 44,700 | 4,050 | 27.8 | 6.9 | 25.1 | 414 |
| 2,000-4,750 | 143,000 | 1,400 | 452 | 65,000 | 4,430 | 54.9 | 10.5 | 27.8 | 450 |
| >4,750 | 251,000 | 1,700 | 134 | 8,390 | 7,000 | 48.7 | 9.3 | 59.6 | 564 |

Cai (2013) conducted performance monitoring of the UpFlo Filter in Tuscaloosa, AL, as part of his MSCE thesis research at the University of Alabama.  The filter was installed in a small paved parking lot at a river-side park, with moderate parking activity (fishers and dog walkers), with some adjacent landscaping. At the end of the monitoring period, grab samples of sump sediment were collected, dried, and analyzed by particle size (nutrients and heavy metals along with solids characteristics and specific gravity). Table 7 shows the analysis results for nutrients, while Table 8 shows the analysis results for heavy metals. Total sulfite and total sulfide were analyzed to evaluate the potential for anaerobic conditions in the sediment and binding of heavy metals. These were both undetected for all samples. Total sulfate, total phosphorus, and all the heavy metal constituents tended to be higher on the organic leaves and on the very small particles.

Table 7. Nutrient Content of Sump Sediment Samples (Cai 2013)

|  | Total N (mg/kg) | Total P (mg/kg) | Total Sulfide (mg/kg) | Total Sulfate (mg/kg) | Total Sulfite (mg/kg) |
| --- | --- | --- | --- | --- | --- |
| Leaves | 1790 | 127 | BDL | 1260 | BDL |
| Sticks | 2600 | 115 | BDL | 522 | BDL |
| >2800 | 2740 | 131 | BDL | 540 | BDL |
| 1400 - 2800 | 2660 | 312 | BDL | 545 | BDL |
| 710-1400 | 1930 | 161 | BDL | 691 | BDL |
| 355-710 | 812 | 303 | BDL | 574 | BDL |
| 180-355 | 588 | 335 | BDL | 362 | BDL |
| 75-180 | 1900 | 350 | BDL | 949 | BDL |
| 45-75 | 1710 | 815 | BDL | 1780 | BDL |
| <45 (Pan) | 2740 | 670 | BDL | 2310 | BDL |

Table 8. Heavy Metal Content of Sump Sediment Samples

|  | Cadmium (mg/kg) | Chromium (mg/kg) | Lead (mg/kg) | Copper (mg/kg) | Zinc (mg/kg) |
| --- | --- | --- | --- | --- | --- |
| Leaves | 2.1 | 24 | 4.3 | 9 | 28 |
| Sticks | BDL | 14 | 2.1 | 5 | 46 |
| >2800 | BDL | 6 | 1.9 | 3 | BDL |
| 1400 - 2800 | 0.8 | 60 | 6.1 | 10 | 31 |
| 710-1400 | 0.8 | 160 | 15.1 | 18 | 56 |
| 355-710 | 0.7 | 97 | 15.6 | 15 | 71 |
| 180-355 | BDL | 85 | 14.7 | 25 | 71 |
| 75-180 | 11.0 | 81 | 35.5 | 29 | 76 |
| 45-75 | 2.0 | 135 | 60.2 | 41 | 198 |
| <45 (Pan) | 2.4 | 157 | 51.6 | 48 | 245 |

Table 9 shows the volatile solids and specific gravity values of these samples, separated by particle size.

Table 9. Solids Characteristics of Sump Sediment Samples (2)

| Sieve size range (um) | Specific Gravity (g/cc) | Volatile Solids (%) |
| --- | --- | --- |
| Decomposing leaves | 2.28 | 93.2 |
| Sticks | 0.84 | 81.2 |
| >2800 | 0.66 | 70.9 |
| 1400 - 2800 | 1.15 | 57.8 |
| 710-1400 | 1.43 | 42.7 |
| 355-710 | 2.56 | 26.1 |
| 180-355 | 2.76 | 19.4 |
| 75-180 | 2.97 | 20.6 |
| 45-75 | 3.30 | 25.7 |
| <45 (Pan) | 3.46 | 26.0 |

Except for the decomposing leaves, specific gravity decreases as the volatile solids content increases; larger particle sizes have lower specific gravity and greater volatile solids as they contain larger amounts of light-weight organic debris. Figure 1 is a scatterplot showing the close relationship of these two constituents supporting the likelihood that the low specific gravity values for the large particles are due to the increased amount of organic matter in those larger particle sizes. However, the decomposing leaves in the sediment have a very large volatile solids content, but also a high specific gravity. “Fresher” leaves would have a lower specific gravity and be associated with the floatable portion of the stormwater solids.

Figure 1. Specific gravity vs. volatile solids relationship.

## Particulate Pollutant Strengths for Stormwater Outfall Samples

The first phase of this SERDP (2018) research included stormwater sampling in the Paleta Creek watershed, located in San Diego and National City, CA. The upper area of the watershed is comprised mainly of residential areas with some commercial activity. The creek then flows through Naval Base San Diego that has mostly industrial attributes. Tables 10 through 17 summarize the metal and organic pollutant strengths of particulates in four separate size ranges (0.45 to 5 μm, 5 to 20 μm, 20 to 63 μm, and >63 μm). These particle sizes represent potential near shore sedimentation effects (>63 μm) to long-range transport (0.45 to 5 μm) after discharge from the creek.

As expected, particulates from the NBSD (industrial) stormwater have greater pollutant strengths than the particulates from the residential/commercial area. In addition, the pollutant strengths increase dramatically with larger particles for the NBSD samples, likely due to relatively large particulates of metals and oily matter. The residential area particulate strengths are high for both the finest and the largest particles, with intermediate size particles having lower pollutant strengths.

Table 10. Particulate Strengths for Heavy Metals at Paleta Creek (SERDP 2018)

|  | Naval Base San Diego (Industrial Outfalls) |  |  |  |  |  |  | Paleta Creek (mixed urban mostly residential some commercial) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NBSD outfall at Paunack and Division Streets (1st event) | NBSD outfall at Paunack and Division Streets (2nd event) | NBSD outfall north of railroad crossing (1st event) | NBSD outfall #23 (1ST event) | NBSD outfall #33 (1st event) | NBSD outfall #33 (2nd event) | Industrial average (COV) | Paleta Creek at Main Street (1st event) | Paleta Creek at Main Street (2nd event) | Paleta Creek at Cummings Road 1st event) | Paleta Creek at Cummings Road (2nd event) | Residential/Commercial average (COV) |
| Mercury (μg/kg) |  |  |  |  |  |  |  |  |  |  |  |  |
| Particulate (0.45 -5 μm) | 1,606 | 310 | na* | na | nd | 75.7 | 664 (1.01) | nd | na | nd | 46.6 | 47 |
| Particulate (5-20 μm) | 138 | 189 | 483 | nd** | 781 | 237 | 366 (0.65) | 115 | 101 | 147 | 250 | 153 (0.44) |
| Particulate (20-63 μm) | 4,804 | 1,859 | na | 31.0 | nd | 728 | 1,856 (0.98) | 113 | 75.0 | 47.3 | 124 | 90 (0.39) |
| Particulate (> 63 μm) | 10,048 | 1,795 | 2,800 | 91.7 | 22,143 | 384 | 6,210 (1.27) | 933 | 42.1 | 433 | 115 | 381 (1.06) |
| Lead (mg/kg) |  |  |  |  |  |  |  |  |  |  |  |  |
| Particulate (0.45 -5 μm) | 396 | nd | na* | na | 0.1 | 4.1 | 133 (1.39) | 485 | na | 24.7 | 42.4 | 184 (1.42) |
| Particulate (5-20 μm) | 14.1 | 9.9 | 103 | 173 | 78.1 | 98.3 | 79 (0.70) | 78.0 | 67.5 | 112 | 116 | 93 (0.26) |
| Particulate (20-63 μm) | nd** | nd | na | 12.9 | nd | 401 | 207 (0.94) | 44.1 | 15.2 | 20.3 | 98.6 | 45 (0.86) |
| Particulate (> 63 μm) | 278 | 143 | 68.3 | 213 | 1262 | 577 | 424 (0.96) | 563 | 37.4 | 192 | 47.5 | 210 (1.17) |
| Cadmium (mg/kg) |  |  |  |  |  |  |  |  |  |  |  |  |
| Particulate (0.45 -5 μm) | nd** | nd | na* | na | nd | nd | all nd | nd | na | nd | nd | all nd |
| Particulate (5-20 μm) | nd | nd | 0.2 | 0.1 | 1.4 | nd | 0.6 (1.04) | 1.8 | nd | nd | nd | 1.8 |
| Particulate (20-63 μm) | nd | nd | na | 2.1 | na* | nd | 2.1 | 1.5 | 0.21 | nd | 2.3 | 1.3 (0.79) |
| Particulate (> 63 μm) | 21.6 | nd | 29.1 | 3.2 | 165 | nd | 54.7 (1.18) | 10.8 | 0.43 | 6.4 | 12.3 | 7.5 (0.71) |
| Zinc (mg/kg) |  |  |  |  |  |  |  |  |  |  |  |  |
| Particulate (0.45 -5 μm) | 2,667 | 2,995 | na* | na | 454 | nd** | 2,039 (0.55) | 2,169 | na | 414 | 1,343 | 1,309 (0.67) |
| Particulate (5-20 μm) | 146 | nd | 546 | 42.6 | nd | 173 | 227 (0.84) | 543 | 595 | 265 | 502 | 476 (0.31) |
| Particulate (20-63 μm) | 1,610 | nd | na | 1,098 | 2,039 | nd | 1,582 (0.24) | 659 | 274. | 161 | 523 | 404 (0.56) |
| Particulate (> 63 μm) | 4,046 | 13,086 | 8,797 | 2,505 | 3,875 | 29,021 | 10,222 (0.89) | 4,917 | 375 | 1,657 | 3,054 | 2,501 (0.78) |
| Nickel (mg/kg) |  |  |  |  |  |  |  |  |  |  |  |  |
| Particulate (0.45 -5 μm) | 34.8 | nd | na* | na | nd | nd | 35 | 175 | na | 95.3 | 0 | 90 (0.97) |
| Particulate (5-20 μm) | nd** | nd | nd | 14.5 | 13.6 | nd | 14 (0.03) | 22.1 | 33.4 | nd | 56.9 | 37 (0.47) |
| Particulate (20-63 μm) | 356 | nd | na | 36.0 | nd | nd | 196 (0.82) | 37.8 | 3.9 | nd | nd | 21 (1.15) |
| Particulate (> 63 μm) | 526 | nd | 597 | 10.2 | 1,080 | nd | 553 (0.69) | 195 | 14.4 | 78.4 | nd | 96 (0.95) |
| Copper (mg/kg) |  |  |  |  |  |  |  |  |  |  |  |  |
| Particulate (0.45 -5 μm) | 436 | 1,638 | na | na | 1.2 | 278 | 588 (1.06) | 752 | na | 226 | nd | 489 (0.76) |
| Particulate (5-20 μm) | 28.5 | 12.0 | nd | 55.7 | 76.0 | 94.3 | 53 (0.56) | 116 | 122 | nd | 189 | 142 (0.28) |
| Particulate (20-63 μm) | 608 | 714 | na | 128 | nd | 286 | 434 (0.55) | 136 | 30.9 | 47.8 | 78.3 | 73 (0.63) |
| Particulate (> 63 μm) | 505 | nd | 146 | 96.6 | 8,751 | 1,081 | 2,116 (1.58) | 773 | 62.1 | 278 | 164 | 319 (0.99) |
| Silver (mg/kg) |  |  |  |  |  |  |  |  |  |  |  |  |
| Particulate (0.45 -5 μm) |  | nd |  |  |  | nd | all nd |  | na* |  | nd | all nd |
| Particulate (5-20 μm) |  | nd |  |  |  | nd | all nd |  | 0.4 |  | 1.4 | 0.9 (0.79) |
| Particulate (20-63 μm) |  | nd |  |  |  | nd | all nd |  | 0.1 |  | nd | 0.1 |
| Particulate (> 63 μm) |  | nd |  |  |  | nd | all nd |  | 0.1 |  | nd | 0.1 |
| Arsenic (mg/kg) |  |  |  |  |  |  |  |  |  |  |  |  |
| Particulate (0.45 -5 μm) | nd |  | na* | na | nd |  | all nd | 50.4 | na | 25.6 |  | 38 (0.46) |
| Particulate (5-20 μm) | 5.2 |  | 1.3 | 6.2 | 16.9 |  | 7 (0.78) | 16.9 |  | nd |  | 17 |
| Particulate (20-63 μm) | 134 | nd | na | 655 | nd | nd | 395 (0.66) | 18.0 | 6.3 | 7.3 | nd | 11 (0.62) |
| Particulate (> 63 μm) | 128 | nd | 1,418 | nd | 1,111 | nd | 886 (0.62) | 544 | 4.3 | 559 | 22.4 | 282 (1.10) |
| TOC (%) |  |  |  |  |  |  |  |  |  |  |  |  |
| Particulate (0.45 -5 μm) |  |  | na* | na | 8.8 | nd | 8.8 |  | na | 40.5 | nd | 40.5 |
| Particulate (5-20 μm) | 2.4 |  | 6.7 | 15.7 | 2.7 | 2.0 | 5.9 (0.88) | 2.0 | nd | 4.9 | 9.1 | 5.3 (0.67) |
| Particulate (20-63 μm) | nd |  | na | 19.9 | 4.4 | 15.2 | 13.2 (0.49) | 10.9 | 18.1 | 6.2 | 2.6 | 9.5 (0.71) |
| Particulate (> 63 μm) | 47.2 |  |  | 23.6 | 10.0 | 21.6 | 25.6 (0.53) | 19.8 | 2.2 | 21.3 | 16.2 | 14.9 (0.59) |

* SSC was not detected; particulate strength could not be calculated and assumed to be zero

** not detected; particulate strength could not be calculated and assumed to be zero

Blanks are missing data

Table 11. Chlordane (ug/kg) All Flows Combined (SERDP 2018)

| Size range | 0.7-2.7 μm | 2.7-20 μm | 20-63 μm | > 63 μm |
| --- | --- | --- | --- | --- |
|  | 453 | nd | nd | nd |
|  | 13.0 | 27.5 | 549 | nd |
|  | 42.9 | 30.8 | 139 | 41.5 |
|  | nd | 37.6 | 71.8 | nd |
|  | nd | 3.58 | 19.7 | 81.0 |
|  | nd | 21.0 | nd | NA |
|  | 9.1 | 15.1 | 8.7 | nd |
|  | 5.1 | 2.6 | nd | nd |
|  | 26.8 | 2.8 | 286 | nd |
| average | 61.1 | 15.7 | 119 | 15.3 |
| median | 9.1 | 15.1 | 19.7 | nd |
| COV | 2.4 | 0.90 | 1.57 | 1.98 |
| min | nd | nd | nd | nd |
| max | 453 | 37.6 | 549 | 81.0 |
| count | 9 | 9 | 9 | 8 |

Table 12. Total PCBs (ug/kg) All Flows Combined (SERDP 2018)

| Size range | 0.7-2.7 μm | 2.7-20 μm | 20-63 μm | > 63 μm |
| --- | --- | --- | --- | --- |
|  | nd | 3.3 | 240 | nd |
|  | 492 | 46.2 | 21.2 | 113 |
|  | 23.0 | nd | 43.9 | 22.3 |
|  | 3.2 | 273 | nd | nd |
|  | 49.0 | 30.9 | 211 | 128 |
|  | 1,270 | 72.5 | nd | nd |
|  | nd | 20.0 | 39.0 | nd |
|  | 418 | 2.7 | 126 | nd |
|  |  | 258 | 268 | nd |
|  |  | 4.3 | 174 | nd |
|  |  | 22.3 | 519 | nd |
|  |  | 30.2 | nd |  |
|  |  | 33.7 | 5.8 |  |
|  |  | 102 | 287 |  |
|  |  | 19.5 | nd |  |
| avg | 282 | 61.2 | 129 | 23.9 |
| median | 36.0 | 30.2 | 43.9 | 0.0 |
| COV | 1.58 | 1.43 | 1.18 | 2.02 |
| count | 8 | 15 | 15 | 11 |
| min | nd | nd | nd | nd |
| max | 1,270 | 273 | 519 | 128 |

Table 13. Paleta Creek Stormwater PAH Pollutant Strengths (SERDP 2018)

|  |  | Naphthalene (ug/kg) |  |  |  |  | Fluorene (ug/kg) |  |  |  |  | Acenaphthene (ug/kg) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 0.7-2.7 μm | 2.7-20 μm | 20-63 μm | > 63 μm |  | 0.7-2.7 μm | 2.7-20 μm | 20-63 μm | > 63 μm |  | 0.7-2.7 μm | 2.7-20 μm | 20-63 μm | > 63 μm |
| C2W event 1 | resid | nd | 98 | nd | 392 |  | nd | 36 | nd | nd |  | nd | 30 | nd | nd |
| C2W event 2 | resid | 310 | nd | 86 | 15 |  | nd | 896 | 1,576 | nd |  | nd | nd | 9 | 82 |
| O4W event 1 | NBSD | 25 | nd | 27 | nd |  | 20 | nd | nd | nd |  | 6 | nd | 99 | nd |
| O3W event 1 | NBSD | nd | nd | nd | 16,756 |  | nd | nd | 37 | 137 |  | nd | nd | 254 | 628 |
| O1W event 1 | NBSD | nd | 47 | nd | 529 |  | nd | nd | 20 | 474 |  | nd | nd | 57 | 1,255 |
| O2W event 1 | NBSD | nd | 2 | nd | 117 |  | nd | nd | nd | nd |  | nd | 2 | nd | 7,204 |
| O4W event2 | NBSD | nd | 61 | 274 | nd |  | nd | 3 | nd | nd |  | nd | 12 | nd | 672 |
| O2W event2 | NBSD | nd | 10 | nd | 5,837 |  | nd | 30 | nd | nd |  | nd | 59 | 319 | nd |
| C1W event 1 | mixed flow | nd | 16 | 4 | 182 |  | nd | 9 | nd | 30 |  | 149 | 10 | nd | 453 |
| C1W event 2 | mixed flow | nd | 10 | nd | 192 |  | nd | nd | nd | 169 |  | nd | nd | nd | 450 |
| A1W event 2 | creek mouth | nd | 17 | 68 | 145 |  | nd | nd | nd | 224 |  | nd | 12 | nd | 912 |
| A2W event 1 | creek mouth | nd | nd | nd | nd |  | nd | nd | nd | nd |  | 65 | nd | 208 | nd |
| A3W event 1 | creek mouth | nd | nd | nd | 4,388 |  | 573 | nd | nd | 1,400 |  | 3,420 | nd | nd | 996 |
| A1W event 2 | creek mouth | nd | 5 | nd | 223 |  | nd | 21 | nd | 6 |  | 2 | 13 | nd | 297 |
| A2W event 2 | creek mouth | nd | 8 | 451 | nd |  | nd | nd | nd | nd |  | nd | 10 | nd | nd |
| count |  | 2 | 10 | 6 | 11 |  | 2 | 6 | 3 | 7 |  | 5 | 8 | 6 | 10 |
| min |  | 25 | 2 | 4 | 15 |  | 20 | 3 | 20 | 6 |  | 2 | 2 | 9 | 82 |
| max |  | 310 | 98 | 451 | 16,756 |  | 573 | 896 | 1,576 | 1,400 |  | 3,420 | 59 | 319 | 7,204 |
| average |  | 167 | 27 | 152 | 2,616 |  | 296 | 166 | 544 | 348 |  | 728 | 18 | 158 | 1,295 |
| median |  | 167 | 13 | 77 | 223 |  | 296 | 26 | 37 | 169 |  | 65 | 12 | 153 | 650 |
| stdev |  | 202 | 31 | 175 | 5,093 |  | 391 | 358 | 893 | 489 |  | 1,506 | 18 | 121 | 2,105 |
| COV |  | 1.21 | 1.15 | 1.15 | 1.95 |  | 1.32 | 2.16 | 1.64 | 1.40 |  | 2.07 | 0.98 | 0.77 | 1.63 |

Table 14. Paleta Creek Stormwater PAH Pollutant Strengths (continued) (SERDP 2018)

|  |  | Phenanthrene (ug/kg) |  |  |  |  | Anthracene (ug/kg) |  |  |  |  | Fluoranthene (ug/kg) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 0.7-2.7 μm | 2.7-20 μm | 20-63 μm | > 63 μm |  | 0.7-2.7 μm | 2.7-20 μm | 20-63 μm | > 63 μm |  | 0.7-2.7 μm | 2.7-20 μm | 20-63 μm | > 63 μm |
| C2W event 1 | resid | nd | 261 | 86 | nd |  | nd | nd | 212 | nd |  | 1,587 | 382 | 1,170 | 8,308 |
| C2W event 2 | resid | nd | nd | nd | 541 |  | nd | nd | nd | nd |  | nd | 349 | 708 | 113 |
| O4W event 1 | NBSD | 46 | nd | 121 | nd |  | 7 | nd | nd | nd |  | nd | 186 | 1,468 | nd |
| O3W event 1 | NBSD | nd | nd | 109 | 19,198 |  | nd | nd | 493 | 18,310 |  | nd | 386 | 1,403 | 137,838 |
| O1W event 1 | NBSD | nd | 360 | 296 | 2,126 |  | nd | 32 | 48 | nd |  | nd | 810 | 579 | 1,710 |
| O2W event 1 | NBSD | nd | 14 | 7 | 10,098 |  | nd | 5 | 420 | 986 |  | nd | 95 | nd | 105,346 |
| O4W event2 | NBSD | nd | 11 | nd | 997 |  | nd | nd | nd | 1,191 |  | nd | 23 | nd | 1,395 |
| O2W event2 | NBSD | nd | nd | nd | 2,490 |  | nd | nd | nd | 135 |  | nd | 35 | 438 | 1,249 |
| C1W event 1 | mixed flow | nd | 94 | 275 | 1,416 |  | 30 | 20 | 170 | 118 |  | nd | 645 | 3,646 | nd |
| C1W event 2 | mixed flow | nd | nd | 357 | 443 |  | nd | 5 | 14 | nd |  | 715 | 134 | 709 | nd |
| A1W event 2 | creek mouth | nd | 406 | 829 | 5,758 |  | nd | 48 | 79 | 1,141 |  | nd | 1,187 | 2,270 | 16,768 |
| A2W event 1 | creek mouth | nd | 17 | nd | nd |  | nd | 4 | nd | nd |  | 95 | 171 | 3,444 | nd |
| A3W event 1 | creek mouth | nd | nd | nd | 4,971 |  | nd | 36 | 185 | nd |  | 7,636 | 142 | 839 | nd |
| A1W event 2 | creek mouth | 16 | 44 | nd | 3,070 |  | 42 | nd | nd | nd |  | 9 | 32 | 678 | 4,062 |
| A2W event 2 | creek mouth | nd | 20 | nd | nd |  | nd | 18 | nd | nd |  | nd | 46 | nd | nd |
| count |  | 2 | 9 | 8 | 11 |  | 3 | 8 | 8 | 6 |  | 5 | 15 | 12 | 9 |
| min |  | 16 | 11 | 7 | 443 |  | 7 | 4 | 14 | 118 |  | 9 | 23 | 438 | 113 |
| max |  | 46 | 406 | 829 | 19,198 |  | 42 | 48 | 493 | 18,310 |  | 7,636 | 1,187 | 3,646 | 137,838 |
| average |  | 31 | 136 | 260 | 4,646 |  | 26 | 21 | 203 | 3,647 |  | 2,009 | 308 | 1,446 | 30,754 |
| median |  | 31 | 44 | 198 | 2,490 |  | 30 | 19 | 177 | 1,064 |  | 715 | 171 | 1,004 | 4,062 |
| stdev |  | 21 | 161 | 260 | 5,609 |  | 18 | 16 | 172 | 7,200 |  | 3,209 | 337 | 1,104 | 52,390 |
| COV |  | 0.69 | 1.18 | 1.00 | 1.21 |  | 0.68 | 0.78 | 0.85 | 1.97 |  | 1.60 | 1.09 | 0.76 | 1.70 |

Table 15. Paleta Creek Stormwater PAH Pollutant Strengths (continued) (SERDP 2018)

|  |  | Pyrene (ug/kg) |  |  |  |  | Chrysene (ug/kg) |  |  |  |  | Benzo[a]anthracene (ug/kg) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 0.7-2.7 μm | 2.7-20 μm | 20-63 μm | > 63 μm |  | 0.7-2.7 μm | 2.7-20 μm | 20-63 μm | > 63 μm |  | 0.7-2.7 μm | 2.7-20 μm | 20-63 μm | > 63 μm |
| C2W event 1 | resid | 505 | 511 | 685 | 2,314 |  | 165 | 133 | 397 | nd |  | 90 | 95 | 401 | 294 |
| C2W event 2 | resid | nd | 479 | 267 | 162 |  | nd | 183 | 105 | 48 |  | nd | 93 | 41 | 52 |
| O4W event 1 | NBSD | 20 | 114 | 65 | nd |  | 9 | 34 | 329 | nd |  | nd | 22 | 193 | nd |
| O3W event 1 | NBSD | nd | 230 | nd | 33,846 |  | nd | 91 | 230 | 6,838 |  | nd | 68 | 159 | 9,652 |
| O1W event 1 | NBSD | nd | 433 | 163 | 687 |  | nd | 265 | 170 | 893 |  | nd | 266 | 175 | 1,468 |
| O2W event 1 | NBSD | nd | 111 | nd | 67,884 |  | nd | 44 | nd | 20,094 |  | 50 | 32 | nd | 15,189 |
| O4W event2 | NBSD | nd | 5 | 507 | 279 |  | nd | nd | 559 | 750 |  | nd | nd | 276 | 377 |
| O2W event2 | NBSD | nd | 30 | 299 | 3,950 |  | nd | 18 | 124 | 2,926 |  | nd | 10 | 103 | 1,002 |
| C1W event 1 | mixed flow | 129 | 372 | 1,163 | 2,043 |  | 152 | 172 | 343 | 1,176 |  | 69 | 126 | 465 | 246 |
| C1W event 2 | mixed flow | 488 | 159 | 601 | 648 |  | 74 | 60 | 233 | 658 |  | nd | 39 | 167 | 431 |
| A1W event 2 | creek mouth | nd | 934 | 307 | 19,374 |  | nd | 459 | 150 | 8,843 |  | nd | 490 | nd | 6,350 |
| A2W event 1 | creek mouth | 115 | 145 | 320 | nd |  | 49 | 65 | 249 | nd |  | 37 | 44 | 22 | nd |
| A3W event 1 | creek mouth | 3,336 | 58 | 217 | nd |  | 1,546 | 6 | 99 | 11 |  | 1,166 | 24 | 24 | 362 |
| A1W event 2 | creek mouth | nd | 24 | 494 | 3,731 |  | 5 | 12 | 183 | 1,418 |  | 3 | 8 | 113 | 766 |
| A2W event 2 | creek mouth | nd | 33 | 162 | nd |  | nd | 15 | 58 | nd |  | nd | 13 | 106 | nd |
| count |  | 6 | 15 | 13 | 11 |  | 7 | 14 | 14 | 11 |  | 6 | 14 | 13 | 12 |
| min |  | 20 | 5 | 65 | 162 |  | 5 | 6 | 58 | 11 |  | 3 | 8 | 22 | 52 |
| max |  | 3,336 | 934 | 1,163 | 67,884 |  | 1,546 | 459 | 559 | 20,094 |  | 1,166 | 490 | 465 | 15,189 |
| average |  | 765 | 243 | 404 | 12,265 |  | 286 | 111 | 231 | 3,969 |  | 236 | 95 | 173 | 3,016 |
| median |  | 308 | 145 | 307 | 2,314 |  | 74 | 63 | 207 | 1,176 |  | 59 | 42 | 159 | 599 |
| stdev |  | 1,276 | 259 | 292 | 21,247 |  | 559 | 127 | 137 | 6,076 |  | 457 | 133 | 136 | 4,856 |
| COV |  | 1.67 | 1.07 | 0.72 | 1.73 |  | 1.96 | 1.14 | 0.59 | 1.53 |  | 1.94 | 1.40 | 0.79 | 1.61 |

Table 16. Paleta Creek Stormwater PAH Pollutant Strengths (continued) (SERDP 2018)

|  |  | Benzo[b]fluoranthene (ug/kg) |  |  |  |  | Benzo[k]fluoranthene (ug/kg) |  |  |  |  | Benzo[a]pyrene (ug/kg) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 0.7-2.7 μm | 2.7-20 μm | 20-63 μm | > 63 μm |  | 0.7-2.7 μm | 2.7-20 μm | 20-63 μm | > 63 μm |  | 0.7-2.7 μm | 2.7-20 μm | 20-63 μm | > 63 μm |
| C2W event 1 | resid | 300 | 183 | 997 | 2,945 |  | 83 | 68 | 344 | nd |  | 148 | 117 | 592 | nd |
| C2W event 2 | resid | nd | 280 | 182 | 123 |  | nd | 76 | 54 | 24 |  | nd | 164 | 120 | 33 |
| O4W event 1 | NBSD | 19 | 78 | 145 | nd |  | 2 | 21 | 21 | nd |  | 0.2 | 39.3 | 45.0 | nd |
| O3W event 1 | NBSD | nd | 204 | 833 | 22,596 |  | nd | 86 | 229 | 5,641 |  | nd | 184 | 546 | 10,907 |
| O1W event 1 | NBSD | nd | 492 | 508 | 2,514 |  | nd | 151 | 90 | 462 |  | nd | 330 | 261 | 1,687 |
| O2W event 1 | NBSD | nd | 74 | nd | 30,990 |  | nd | 31 | nd | 13,632 |  | nd | 31 | nd | 14,676 |
| O4W event2 | NBSD | nd | nd | 477 | 778 |  | nd | nd | 60 | 48 |  | nd | nd | 248 | nd |
| O2W event2 | NBSD | nd | 58 | 584 | 1,663 |  | nd | 20 | 156 | 972 |  | nd | 20 | 220 | 1,287 |
| C1W event 1 | mixed flow | 192 | 344 | 1,347 | 210 |  | 58 | 119 | 430 | 29 |  | 85 | 184 | 624 | 309 |
| C1W event 2 | mixed flow | 242 | 125 | 624 | 39 |  | 46 | 41 | 214 | 54 |  | 190 | 72 | 398 | 110 |
| A1W event 2 | creek mouth | nd | 1,105 | nd | 12,385 |  | nd | 412 | 594 | 2,032 |  | nd | 662 | 1,022 | 3,851 |
| A2W event 1 | creek mouth | 161 | 147 | nd | nd |  | 51 | 51 | nd | nd |  | 72 | 69 | nd | nd |
| A3W event 1 | creek mouth | 3,770 | 72 | 104 | nd |  | 1,333 | 25 | 25 | nd |  | 2,190 | 39 | 18 | 570 |
| A1W event 2 | creek mouth | 3 | 19 | 312 | 1,554 |  | 1 | 5 | 106 | 496 |  | 3 | 8 | 198 | 969 |
| A2W event 2 | creek mouth | nd | 31 | 357 | nd |  | nd | 9 | 109 | nd |  | nd | 14 | 153 | nd |
| count |  | 7 | 14 | 12 | 11 |  | 7 | 14 | 13 | 10 |  | 7 | 14 | 13 | 10 |
| min |  | 3 | 19 | 104 | 39 |  | 1 | 5 | 21 | 24 |  | 0.2 | 8 | 18 | 33 |
| max |  | 3,770 | 1,105 | 1,347 | 30,990 |  | 1,333 | 412 | 594 | 13,632 |  | 2,190 | 662 | 1,022 | 14,676 |
| average |  | 669 | 229 | 539 | 6,891 |  | 225 | 80 | 187 | 2,339 |  | 384 | 138 | 342 | 3,440 |
| median |  | 192 | 136 | 493 | 1,663 |  | 51 | 46 | 109 | 479 |  | 85 | 70 | 248 | 1,128 |
| stdev |  | 1,371 | 285 | 372 | 10,597 |  | 489 | 105 | 174 | 4,327 |  | 799 | 176 | 286 | 5,129 |
| COV |  | 2.05 | 1.24 | 0.69 | 1.54 |  | 2.18 | 1.31 | 0.93 | 1.85 |  | 2.08 | 1.28 | 0.84 | 1.49 |

Table 17. Paleta Creek Stormwater PAH Pollutant Strengths (continued) (SERDP 2018)

|  |  | Dibenzo[a,h]anthracene (ug/kg) |  |  |  |  | Benzo[ghi]perylene+Indeno[1,2,3-cd]pyrene (ug/kg) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 0.7-2.7 μm | 2.7-20 μm | 20-63 μm | > 63 μm |  | 0.7-2.7 μm | 2.7-20 μm | 20-63 μm | > 63 μm |
| C2W event 1 | resid | 34.0 | 52.7 | 137.7 | nd |  | 460 | 327 | 1,598 | nd |
| C2W event 2 | resid | nd | 42.1 | 133.1 | nd |  | nd | 608 | 476 | 41 |
| O4W event 1 | NBSD | nd | 11.7 | 52.6 | nd |  | 26 | 116 | 174 | nd |
| O3W event 1 | NBSD | nd | 37.0 | 54.8 | 6018.2 |  | nd | 204 | 682 | 6,011 |
| O1W event 1 | NBSD | nd | 119.7 | nd | 33.2 |  | nd | 779 | 383 | 1,644 |
| O2W event 1 | NBSD | 2.8 | 6.4 | nd | 6254.6 |  | 4 | 19 | nd | 9,990 |
| O4W event2 | NBSD | nd | nd | nd | 1585.4 |  | nd | 2 | 442 | 578 |
| O2W event2 | NBSD | nd | 7.5 | 19.1 | 448.4 |  | nd | 33 | 402 | 1,638 |
| C1W event 1 | mixed flow | 44.6 | 63.6 | 34.3 | 147.2 |  | 233 | 363 | 792 | 931 |
| C1W event 2 | mixed flow | 45.6 | 19.3 | 74.1 | 31.2 |  | 293 | 193 | 1,098 | 183 |
| A1W event 2 | creek mouth | nd | 193.8 | 53.5 | 1828.9 |  | nd | 1,081 | 1,297 | 6,901 |
| A2W event 1 | creek mouth | 18.4 | 21.2 | 77.6 | nd |  | 132 | 107 | nd | nd |
| A3W event 1 | creek mouth | 782.1 | 18.0 | 54.7 | nd |  | 3,569 | 45 | nd | 1,095 |
| A1W event 2 | creek mouth | 4.5 | 2.5 | 166.3 | nd |  | nd | 16 | 750 | 2,512 |
| A2W event 2 | creek mouth | nd | 5.4 | nd | nd |  | nd | 34 | 290 | nd |
| count |  | 7 | 14 | 11 | 8 |  | 7 | 15 | 12 | 11 |
| min |  | 2.8 | 3 | 19 | 31 |  | 3.8 | 2 | 174 | 41 |
| max |  | 782 | 194 | 166 | 6,255 |  | 3,569 | 1,081 | 1,598 | 9,990 |
| average |  | 133 | 43 | 78 | 2,043 |  | 674 | 262 | 699 | 2,866 |
| median |  | 34 | 20 | 55 | 1,017 |  | 233 | 116 | 579 | 1,638 |
| stdev |  | 287 | 54 | 47 | 2,619 |  | 1,286 | 323 | 435 | 3,276 |
| COV |  | 2.15 | 1.25 | 0.60 | 1.28 |  | 1.91 | 1.24 | 0.62 | 1.14 |

Morquecho (2005) also collected mixed stormwater flow samples for particulate strengths from flows entering inlets (Table 18) in addition to similar samples from roof runoff shown earlier.

Table 18. Summary table showing average pollutant associations for different particle sizes for storm drain outlet samples (N=10, rounded to two significant figures)

|  | Total phosphorus |  | COD |  | Zinc |  | Copper |  | Lead |  | Cadmium |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | mg/kg SS | COV | mg/kg SS | COV | mg/kg SS | COV | mg/kg SS | COV | mg/kg SS | COV | mg/kg SS | COV |
| >250 μm | 5,900 | 1.00 | 1,100,000 | 1.49 | 800 | 0.55 | 120 | 0.55 | 140 | 0.57 | 9.4 | 0.35 |
| 106-250 μm | 1,900 | 1.45 | 780,000 | 1.33 | 52 | 0.68 | 20 | 0.52 | 30 | 0.54 | 8.3 | 1.29 |
| 45-106 μm | 2,300 | 0.77 | 540,000 | 1.57 | na | na | 21 | 0.90 | 200 | 1.24 | na | na |
| 10-45 μm | 3,400 | 0.73 | 720,000 | 0.49 | 990 | na | 130 | 1.06 | 150 | 1.22 | 9.1 | 0.67 |
| 2-10 μm | 2,800 | 0.69 | 340,000 | na | 1,700 | 0.71 | 150 | 0.69 | 75 | 1.10 | 500 | 1.41 |
| 1-2 μm | 8,000 | 0.78 | 670,000 | na | 540 | 1.17 | 320 | 1.08 | 600 | 1.24 | 440 | 1.33 |
| 0.45-1 μm | 8,200 | 0.88 | 1,400,000 | na | 1,600 | 1.05 | 430 | 0.91 | 430 | 0.71 | 110 | 0.80 |

Note: na=too few detectable observations for standard deviation and therefore COV calculations.

## Particulate Pollutant Strengths for Sediment Samples Collected from Small Urban Creeks

Bathi (2008) collected and analyzed urban creek sediments in Tuscaloosa, AL, for PAHs as part of his Ph.D. dissertation research at the University of Alabama. Sediment samples were collected from three different creeks in and around Tuscaloosa and Northport, AL (Cribb’s Mill Creek, Carroll’s Creek, and Hunter Creek). The PAH concentrations were found to be strongly associated with particulate matter. The fugacity level I partitioning calculations were performed for the PAHs in a hypothetical environmental system. This modeling approach indicated that except for the low molecular weight PAHs (naphthalene, fluorene, phenanthrene, and anthracene), all the other studied PAHs were predominantly portioned with the sediment phase. The model predictions also indicated that the PAHs with Log (KOW) or Log (KOC) values greater than about 4.5 were mostly partitioned with the sediment phase, compared to other phases. The high molecular weight PAHs had a greater portion associated with the particulates than did the low molecular weight PAHs.

Tables 19 through 57 show the observed PAH particulate strength values for the different particles sizes for each sample collected at each creek location. Bathi (2008) found that overall, all characteristics studied showed similar trends, with the smaller and larger particles found to have relatively higher values compared to the intermediate sized particles. Cluster analyses of the PAH concentrations for the different particle sizes showed that for most cases examined, the large organic matter (LOM) fraction was found to be a statistically separate sample category (having much higher concentrations) from all other sizes.

Table 19.  Observed Concentrations of Naphthalene at Cribbs Mill Creek

| Size Range (µm) | Concentration (µg/kg), Sample Number |  |  |  |  | Mean Concentration (µg/kg) | Standard Deviation |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 3/28 | 8/18 | 9/9 | 9/26 | 10/9 |  |  |
| < 45 | 189 | 92 | 1045 | 568 | 283 | 436 | 384 |
| 45 – 90 | 101 | 59 | 466 | 486 | 216 | 266 | 200 |
| 90 – 180 | 59 | 101 | 683 | 681 | 281 | 361 | 305 |
| 180 – 355 | 62 | 61 | 176 | 252 | 125 | 135 | 81 |
| 355 – 710 | 37 | 87 | 362 | 103 | 75 | 133 | 130 |
| 710 – 1400 | 156 | 113 | 8126 | 824 | 365 | 1917 | 3483 |
| 1400 – 2800 | 102 | 87 | 7973 | 1866 | 685 | 2143 | 3338 |
| > 2800 (w/o LOM) | 84 | 99 | 153 | 259 | 147 | 149 | 69 |
| > 2800 (LOM) | 1057 | 1895 | 4584 | 2015 | 1656 | 2241 | 1361 |

w/o LOM = with LOM removed     LOM = Large organic matter

Table 20.  Observed Concentrations of Fluorene at Cribbs Mill Creek

| Size Range (µm) | Concentration (µg/kg), Sample Number |  |  |  |  | Mean Concentration (µg/kg) | Standard Deviation |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 3/28 | 8/18 | 9/9 | 9/26 | 10/9 |  |  |
| < 45 | 56 | 89 | 26 | 685 | 256 | 223 | 273 |
| 45 - 90 | 67 | 35 | 54 | 212 | 111 | 96 | 71 |
| 90 - 180 | 25 | 81 | 567 | 329 | 307 | 262 | 217 |
| 180 - 355 | 59 | 52 | 15 | 521 | 199 | 169 | 209 |
| 355 - 710 | 48 | 41 | 541 | 105 | 231 | 193 | 209 |
| 710 - 1400 | 61 | 58 | 458 | 149 | 223 | 190 | 165 |
| 1400 - 2800 | 108 | 101 | 389 | 565 | 354 | 303 | 198 |
| > 2800 (w/o LOM) | 49 | 251 | 159 | 255 | 154 | 174 | 85 |
| > 2800 (LOM) | 1903 | 2057 | 3289 | 1204 | 2132 | 2117 | 751 |

w/o LOM = with LOM removed     LOM = Large organic matter

Table 21.  Observed Concentrations of Phenanthrene at Cribbs Mill Creek

| Size Range (µm) | Concentration (µg/kg), Sample Number |  |  |  |  | Mean Concentration (µg/kg) | Standard Deviation |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 3/28 | 8/18 | 9/9 | 9/26 | 10/9 |  |  |
| < 45 | 256 | 98 | 27 | 189 | 105 | 135 | 89 |
| 45 – 90 | 206 | 40 | 86 | 78 | 68 | 96 | 64 |
| 90 – 180 | 167 | ND | 53 | 167 | 110 | 124 | 55 |
| 180 – 355 | ND | 52 | 12 | 111 | 58 | 58 | 41 |
| 355 – 710 | ND | 28 | 56 | 97 | 60 | 60 | 28 |
| 710 – 1400 | 392 | 42 | 41 | 166 | 83 | 145 | 147 |
| 1400 – 2800 | 941 | 15 | 276 | 247 | 179 | 332 | 355 |
| > 2800 (w/o LOM) | 124 | 611 | 18 | 98 | 242 | 219 | 234 |
| > 2800 (LOM) | 1980 | 2304 | 5821 | 953 | 3026 | 2817 | 1838 |

w/o LOM = with LOM removed     LOM = Large organic matter

Table 22.  Observed Concentrations of Anthracene at Cribbs Mill Creek

| Size Range (µm) | Concentration (µg/kg), Sample Number |  |  |  |  | Mean Concentration (µg/kg) | Standard Deviation |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 3/28 | 8/18 | 9/9 | 9/26 | 10/9 |  |  |
| < 45 | 223 | 125 | 178 | 593 | 298 | 283 | 184 |
| 45 - 90 | 193 | 65 | 309 | 402 | 259 | 246 | 126 |
| 90 - 180 | 53 | 20 | 105 | 442 | 189 | 162 | 169 |
| 180 - 355 | ND | 33 | 18 | 202 | 84 | 84 | 83 |
| 355 - 710 | 132 | 31 | 262 | 684 | 326 | 287 | 250 |
| 710 - 1400 | 603 | 73 | 832 | 525 | 477 | 502 | 276 |
| 1400 - 2800 | 2480 | 53 | 1035 | 632 | 573 | 955 | 921 |
| > 2800 (w/o LOM) | 321 | 126 | 69 | 429 | 208 | 231 | 146 |
| > 2800 (LOM) | 2540 | 4215 | 853 | 1621 | 2230 | 2292 | 1254 |

w/o LOM = with LOM removed     LOM = Large organic matter

Table 23.  Observed Concentrations of Fluoranthene at Cribbs Mill Creek

| Size Range (µm) | Concentration (µg/kg), Sample Number |  |  |  |  | Mean Concentration (µg/kg) | Standard Deviation |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 3/28 | 8/18 | 9/9 | 9/26 | 10/9 |  |  |
| < 45 | 332 | 290 | 44 | 302 | 222 | 238 | 116 |
| 45 – 90 | 441 | 399 | 67 | 325 | 302 | 307 | 145 |
| 90 – 180 | 245 | 243 | 43 | 208 | 177 | 183 | 83 |
| 180 – 355 | 141 | 159 | 43 | 127 | 114 | 117 | 44 |
| 355 – 710 | 138 | 160 | 133 | 139 | 143 | 143 | 10 |
| 710 – 1400 | 398 | 278 | 149 | 301 | 275 | 280 | 89 |
| 1400 – 2800 | 366 | 269 | 291 | 542 | 308 | 355 | 110 |
| > 2800 (w/o LOM) | 240 | 105 | 20 | 211 | 122 | 140 | 88 |
| > 2800 (LOM) | 2240 | 1092 | 3059 | 1092 | 2238 | 1944 | 847 |

w/o LOM = with LOM removed     LOM = Large organic matter

Table 24. Observed Concentrations of Pyrene at Cribbs Mill Creek

| Size Range (µm) | Concentration (µg/kg), Sample Number |  |  |  |  | Mean Concentration (µg/kg) | Standard Deviation |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 3/28 | 8/18 | 9/9 | 9/26 | 10/9 |  |  |
| < 45 | 313 | 363 | 66 | 562 | 330 | 327 | 177 |
| 45 - 90 | 405 | 226 | 116 | 369 | 237 | 271 | 117 |
| 90 - 180 | 178 | 95 | 71 | 543 | 236 | 225 | 190 |
| 180 - 355 | 117 | 60 | 99 | 215 | 125 | 123 | 57 |
| 355 - 710 | 98 | 69 | 40 | 289 | 133 | 126 | 98 |
| 710 - 1400 | 272 | 90 | 214 | 312 | 205 | 219 | 84 |
| 1400 - 2800 | 261 | 159 | 111 | 386 | 219 | 227 | 105 |
| > 2800 (w/o LOM) | 527 | 321 | 50 | 198 | 190 | 257 | 179 |
| > 2800 (LOM) | 2240 | 2654 | 922 | 2923 | 2166 | 2181 | 769 |

w/o LOM = with LOM removed     LOM = Large organic matter

Table 25.  Observed Concentrations of Benzo(a)anthracene at Cribbs Mill Creek

| Size Range (µm) | Concentration (µg/kg), Sample Number |  |  |  |  | Mean Concentration (µg/kg) | Standard Deviation |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 3/28 | 8/18 | 9/9 | 9/26 | 10/9 |  |  |
| < 45 | 270 | 89 | 179 | 689 | 350 | 315 | 231 |
| 45 – 90 | 321 | 53 | 205 | 522 | 299 | 280 | 172 |
| 90 – 180 | 179 | 12 | 330 | 755 | 315 | 318 | 276 |
| 180 – 355 | 93 | 40 | 185 | 629 | 254 | 240 | 233 |
| 355 – 710 | 97 | 15 | 206 | 511 | 208 | 207 | 188 |
| 710 – 1400 | 185 | 22 | 419 | 173 | 127 | 185 | 146 |
| 1400 – 2800 | 171 | 25 | 933 | 393 | 196 | 344 | 355 |
| > 2800 (w/o LOM) | 350 | 72 | 29 | 218 | 1547 | 443 | 630 |
| > 2800 (LOM) | 4350 | 2537 | 1260 | 3202 | 3313 | 2932 | 1138 |

w/o LOM = with LOM removed          LOM = Large organic matter

Table 26.  Observed Concentrations of Chrysene at Cribbs Mill Creek

| Size Range (µm) | Concentration (µg/kg), Sample Number |  |  |  |  | Mean Concentration (µg/kg) | Standard Deviation |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 3/28 | 8/18 | 9/9 | 9/26 | 10/9 |  |  |
| < 45 | 418 | 108 | 96 | 568 | 361 | 310 | 205 |
| 45 - 90 | 507 | 43 | 147 | 922 | 525 | 429 | 349 |
| 90 - 180 | 307 | 30 | 249 | 664 | 407 | 331 | 231 |
| 180 - 355 | 162 | 49 | 142 | 356 | 220 | 186 | 113 |
| 355 - 710 | 158 | 37 | 176 | 383 | 239 | 199 | 126 |
| 710 - 1400 | 350 | 42 | 424 | 142 | 305 | 253 | 157 |
| 1400 - 2800 | 313 | 56 | 833 | 136 | 427 | 353 | 305 |
| > 2800 (w/o LOM) | 310 | 134 | 15 | 142 | 156 | 151 | 105 |
| > 2800 (LOM) | 3010 | 2936 | 3016 | 1353 | 1788 | 2420 | 792 |

w/o LOM = with LOM removed     LOM = Large organic matter

Table 27.  Observed Concentrations of Benzo(b)fluoranthene at Cribbs Mill Creek

| Size Range (µm) | Concentration (µg/kg), Sample Number |  |  |  |  | Mean Concentration (µg/kg) | Standard Deviation |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 3/28 | 8/18 | 9/9 | 9/26 | 10/9 |  |  |
| < 45 | 1010 | 686 | 216 | 496 | 574 | 597 | 289 |
| 45 – 90 | 105 | 119 | 205 | 562 | 291 | 257 | 187 |
| 90 – 180 | 593 | 62 | 252 | 268 | 371 | 309 | 194 |
| 180 – 355 | 2600 | 626 | 108 | 252 | 987 | 915 | 1002 |
| 355 – 710 | 320 | 103 | 167 | 322 | 270 | 236 | 98 |
| 710 – 1400 | 741 | 414 | 613 | 468 | 607 | 569 | 129 |
| 1400 – 2800 | 735 | 114 | 1323 | 236 | 765 | 635 | 483 |
| > 2800 (w/o LOM) | 532 | 455 | 36 | 163 | 1530 | 543 | 588 |
| > 2800 (LOM) | 4390 | 3522 | 1040 | 1633 | 1869 | 2491 | 1405 |

w/o LOM = with LOM removed     LOM = Large organic matter

Table 28.  Observed Concentrations of Benzo(a)pyrene at Cribbs Mill Creek

| Size Range (µm) | Concentration (µg/kg), Sample Number |  |  |  |  | Mean Concentration (µg/kg) | Standard Deviation |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 3/28 | 8/18 | 9/9 | 9/26 | 10/9 |  |  |
| < 45 | 665 | 693 | 212 | 924 | 610 | 621 | 258 |
| 45 – 90 | 882 | 510 | 324 | 598 | 663 | 596 | 205 |
| 90 – 180 | 512 | 515 | 749 | 995 | 674 | 689 | 199 |
| 180 – 355 | 1970 | 289 | 233 | 570 | 943 | 801 | 711 |
| 355 – 710 | 159 | 664 | 242 | 430 | 445 | 388 | 197 |
| 710 – 1400 | 269 | 549 | 2086 | 435 | 1023 | 872 | 734 |
| 1400 – 2800 | 345 | 910 | 8132 | 294 | 3112 | 2559 | 3321 |
| > 2800 (w/o LOM) | 850 | 687 | 857 | 239 | 594 | 645 | 253 |
| > 2800 (LOM) | 4650 | 2109 | 3013 | 1099 | 2074 | 2589 | 1336 |

w/o LOM = with LOM removed     LOM = Large organic matter

Table 29.  Observed Concentrations of Indeno(1,2,3-cd)pyrene at Cribbs Mill Creek

| Size Range (µm) | Concentration (µg/kg), Sample Number |  |  |  |  | Mean Concentration (µg/kg) | Standard Deviation |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 3/28 | 8/18 | 9/9 | 9/26 | 10/9 |  |  |
| < 45 | 732 | 562 | 420 | 854 | 668 | 647 | 165 |
| 45 – 90 | 281 | 393 | 992 | 470 | 581 | 543 | 273 |
| 90 – 180 | 374 | 312 | 1752 | 2406 | 1511 | 1271 | 908 |
| 180 - 355 | 131 | 130 | 588 | 427 | 382 | 332 | 199 |
| 355 - 710 | 219 | 356 | 786 | 360 | 455 | 435 | 213 |
| 710 - 1400 | 269 | 287 | 1308 | 376 | 651 | 578 | 436 |
| 1400 - 2800 | 261 | 550 | 3563 | 283 | 1369 | 1205 | 1393 |
| > 2800 (w/o LOM) | 534 | 144 | 98 | 211 | 610 | 320 | 235 |
| > 2800 (LOM) | 1520 | 956 | 3563 | 1956 | 2717 | 2142 | 1022 |

w/o LOM = with LOM removed     LOM = Large organic matter

Table 30.  Observed Concentrations of Dibenz(a,h)anthracene at Cribbs Mill Creek

| Size Range (µm) | Concentration (µg/kg), Sample Number |  |  |  |  | Mean Concentration (µg/kg) | Standard Deviation |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 3/28 | 8/18 | 9/9 | 9/26 | 10/9 |  |  |
| < 45 | 114 | 413 | 901 | 953 | 656 | 607 | 350 |
| 45 – 90 | 40 | 358 | 1266 | 562 | 623 | 570 | 451 |
| 90 – 180 | 50 | 412 | 1460 | 1051 | 854 | 765 | 550 |
| 180 – 355 | 23 | 257 | 623 | 623 | 423 | 390 | 256 |
| 355 – 710 | 32 | 301 | 715 | 699 | 482 | 446 | 287 |
| 710 – 1400 | ND | 227 | 1344 | 1398 | 1371 | 1085 | 572 |
| 1400 - 2800 | 559 | 448 | 4941 | 351 | 1950 | 1650 | 1952 |
| > 2800 (w/o LOM) | 720 | 148 | 68 | 492 | 427 | 371 | 265 |
| > 2800 (LOM) | 720 | 3022 | 1533 | 2533 | 1964 | 1954 | 891 |

w/o LOM = with LOM removed     LOM = Large organic matter

Table 31.  Observed Concentrations of Benzo(ghi)perylene at Cribbs Mill Creek

| Size Range (µm) | Concentration (µg/kg), Sample Number |  |  |  |  | Mean Concentration (µg/kg) | Standard Deviation |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 3/28 | 8/18 | 9/9 | 9/26 | 10/9 |  |  |
| < 45 | 548 | 167 | 990 | 453 | 389 | 509 | 303 |
| 45 – 90 | 153 | 86 | 474 | 622 | 287 | 324 | 223 |
| 90 – 180 | 236 | 153 | 1346 | 269 | 219 | 444 | 506 |
| 180 – 355 | 84 | 129 | 151 | 182 | 132 | 136 | 36 |
| 355 – 710 | 135 | 69 | ND | 193 | 132 | 132 | 50 |
| 710 – 1400 | 190 | 62 | 386 | 1997 | 750 | 677 | 782 |
| 1400 – 2800 | 313 | 107 | 1027 | 683 | 368 | 499 | 360 |
| > 2800 (w/o LOM) | 588 | 147 | 562 | 365 | 500 | 433 | 181 |
| > 2800 (LOM) | 988 | 1238 | 798 | 3521 | 2658 | 1841 | 1190 |

Table 32.  Observed Concentrations of Naphthalene at Hunter Creek

| Size Range (µm) | Concentration (µg/kg), Sample Number |  |  |  |  | Mean Concentration (µg/kg) | Standard Deviation |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8/18 | 9/9 | 9/26 | 10/9 | 10/21 |  |  |
| < 45 | 11 | 178 | 192 | 74 | 195 | 130 | 83 |
| 45 – 90 | 27 | 98 | 186 | 109 | 59 | 96 | 60 |
| 90 - 180 | 21 | 172 | 87 | 4 | 68 | 70 | 66 |
| 180 - 355 | 15 | 281 | 57 | 9 | 26 | 78 | 115 |
| 355 - 710 | 27 | 63 | 254 | 94 | 44 | 97 | 92 |
| 710 - 1400 | 31 | 20 | 157 | 236 | 519 | 193 | 204 |
| 1400 - 2800 | 78 | 102 | 551 | 351 | 106 | 238 | 207 |
| > 2800 (w/o LOM) | 82 | 5 | 72 | 168 | 147 | 95 | 65 |
| > 2800 (LOM) | 1723 | 2150 | 5413 | 8667 | 2311 | 4053 | 2967 |

w/o LOM = with LOM removed     LOM = Large organic matter

Table 33.  Observed Concentrations of Fluorene at Hunter Creek

| Size Range (µm) | Concentration (µg/kg), Sample Number |  |  |  |  | Mean Concentration (µg/kg) | Standard Deviation |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8/18 | 9/9 | 9/26 | 10/9 | 10/21 |  |  |
| < 45 | 107 | 542 | 93 | 198 | 1090 | 406 | 424 |
| 45 - 90 | 103 | 367 | 292 | 264 | 521 | 310 | 153 |
| 90 - 180 | 160 | 562 | 123 | 34 | 417 | 259 | 221 |
| 180 - 355 | 80 | 53 | 40 | 32 | 197 | 80 | 68 |
| 355 - 710 | 114 | 63 | NA | 14 | 245 | 109 | 99 |
| 710 - 1400 | 20 | 305 | 60 | 38 | 407 | 166 | 178 |
| 1400 - 2800 | 178 | 410 | 616 | 515 | 183 | 380 | 196 |
| > 2800 (w/o LOM) | 79 | 89 | 214 | 126 | 217 | 145 | 67 |
| > 2800 (LOM) | 2900 | 1435 | 986 | 297 | 1075 | 1339 | 965 |

w/o LOM = with LOM removed     LOM = Large organic matter

Table 34.  Observed Concentrations of Phenanthrene at Hunter Creek

| Size Range (µm) | Concentration (µg/kg), Sample Number |  |  |  |  | Mean Concentration (µg/kg) | Standard Deviation |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8/18 | 9/9 | 9/26 | 10/9 | 10/21 |  |  |
| < 45 | 772 | 529 | 351 | 249 | 926 | 566 | 283 |
| 45 - 90 | 395 | 372 | 351 | 259 | 799 | 435 | 210 |
| 90 - 180 | 34 | 503 | 59 | 39 | 457 | 218 | 239 |
| 180 - 355 | 131 | 238 | ND | 9 | 241 | 155 | 110 |
| 355 - 710 | 248 | 162 | 55 | 27 | 260 | 150 | 107 |
| 710 - 1400 | 42 | 333 | 105 | 46 | 422 | 190 | 176 |
| 1400 - 2800 | 107 | 412 | 146 | 106 | 101 | 174 | 134 |
| > 2800 (w/o LOM) | 137 | 34 | 128 | 153 | 322 | 155 | 104 |
| > 2800 (LOM) | 2748 | 874 | 1400 | 925 | 479 | 1285 | 881 |

w/o LOM = with LOM removed     LOM = Large organic matter

Table 35.  Observed Concentrations of Anthracene at Hunter Creek

| Size Range (µm) | Concentration (µg/kg), Sample Number |  |  |  |  | Mean Concentration (µg/kg) | Standard Deviation |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8/18 | 9/9 | 9/26 | 10/9 | 10/21 |  |  |
| < 45 | 58 | 708 | 125 | 320 | 1599 | 562 | 632 |
| 45 - 90 | 54 | 543 | 651 | 515 | 949 | 542 | 323 |
| 90 - 180 | 424 | 712 | 52 | 612 | 570 | 474 | 257 |
| 180 - 355 | 16 | 432 | 86 | ND | 456 | 248 | 229 |
| 355 - 710 | 55 | 342 | 19 | 136 | 306 | 172 | 146 |
| 710 - 1400 | 132 | 900 | 27 | ND | 637 | 424 | 414 |
| 1400 - 2800 | 83 | 309 | 214 | 152 | 566 | 265 | 188 |
| > 2800 (w/o LOM) | 103 | 200 | 313 | 178 | 613 | 281 | 200 |
| > 2800 (LOM) | 3502 | 2189 | 3141 | 2150 | 1594 | 2515 | 783 |

w/o LOM = with LOM removed     LOM = Large organic matter

Table 36.  Observed Concentrations of Fluranthene at Hunter Creek

| Size Range (µm) | Concentration (µg/kg), Sample Number |  |  |  |  | Mean Concentration (µg/kg) | Standard Deviation |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8/18 | 9/9 | 9/26 | 10/9 | 10/21 |  |  |
| < 45 | 701 | 1256 | 1324 | 2935 | 2066 | 1656 | 864 |
| 45 – 90 | 1385 | 582 | 1521 | 2605 | 1352 | 1489 | 724 |
| 90 - 180 | 409 | 1122 | 846 | 323 | 1109 | 762 | 379 |
| 180 - 355 | 701 | 712 | 124 | 56 | 561 | 431 | 318 |
| 355 - 710 | 1294 | 633 | 201 | 79 | 506 | 543 | 476 |
| 710 - 1400 | 917 | 462 | 248 | 69 | 469 | 433 | 317 |
| 1400 - 2800 | 517 | 612 | 82 | 56 | 171 | 287 | 259 |
| > 2800 (w/o LOM) | 712 | 303 | 60 | 142 | 321 | 308 | 251 |
| > 2800 (LOM) | 1893 | 650 | 1543 | 620 | 638 | 1069 | 605 |

w/o LOM = with LOM removed     LOM = Large organic matter

Table 37.  Observed Concentrations of Pyrene at Hunter Creek

| Size Range (µm) | Concentration (µg/kg), Sample Number |  |  |  |  | Mean Concentration (µg/kg) | Standard Deviation |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8/18 | 9/9 | 9/26 | 10/9 | 10/21 |  |  |
| < 45 | 1382 | 958 | 1324 | 1582 | 2495 | 1548 | 575 |
| 45 - 90 | 647 | 1022 | 1241 | 1770 | 1261 | 1188 | 408 |
| 90 - 180 | 2097 | 957 | 286 | 81 | 1154 | 915 | 798 |
| 180 - 355 | 340 | 563 | 58 | 25 | 522 | 302 | 252 |
| 355 - 710 | 1636 | 218 | 86 | 20 | 480 | 488 | 665 |
| 710 - 1400 | 330 | 271 | 124 | 33 | 612 | 274 | 222 |
| 1400 - 2800 | 216 | 328 | 119 | 31 | 403 | 220 | 151 |
| > 2800 (w/o LOM) | 270 | 74 | 161 | 41 | 129 | 135 | 89 |
| > 2800 (LOM) | 2987 | 1594 | 1875 | 748 | 1298 | 1700 | 832 |

Table 38.  Observed Concentrations of Benzo(a)anthracene at Hunter Creek

| Size Range (µm) | Concentration (µg/kg), Sample Number |  |  |  |  | Mean Concentration (µg/kg) | Standard Deviation |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8/18 | 9/9 | 9/26 | 10/9 | 10/21 |  |  |
| < 45 | 80 | 722 | 1622 | 1208 | 1826 | 1092 | 706 |
| 45 – 90 | 85 | 428 | 508 | 1411 | 1879 | 862 | 750 |
| 90 – 180 | 184 | 801 | ND | 11 | 677 | 418 | 380 |
| 180 – 355 | 263 | 390 | 39 | 22 | 291 | 201 | 163 |
| 355 – 710 | 862 | 287 | 27 | 175 | 460 | 362 | 321 |
| 710 – 1400 | 22 | 352 | 43 | 156 | 296 | 174 | 148 |
| 1400 – 2800 | 62 | 628 | 99 | 167 | 502 | 292 | 256 |
| > 2800 (w/o LOM) | 109 | 21 | 107 | 159 | 263 | 132 | 89 |
| > 2800 (LOM) | 2613 | 1180 | 2415 | 729 | 1526 | 1693 | 804 |

w/o LOM = with LOM removed     LOM = Large organic matter

Table 39.  Observed Concentrations of Chrysene at Hunter Creek

| Size Range (µm) | Concentration (µg/kg), Sample Number |  |  |  |  | Mean Concentration (µg/kg) | Standard Deviation |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8/18 | 9/9 | 9/26 | 10/9 | 10/21 |  |  |
| < 45 | 210 | 1257 | 924 | 1591 | 2095 | 1215 | 709 |
| 45 - 90 | 186 | 1033 | 2354 | 1815 | 681 | 1214 | 871 |
| 90 - 180 | 518 | 1303 | 58 | 226 | 820 | 585 | 495 |
| 180 - 355 | 596 | 632 | 95 | 34 | 381 | 348 | 276 |
| 355 - 710 | 1037 | 589 | 51 | 190 | 505 | 475 | 384 |
| 710 - 1400 | 40 | 169 | 315 | 151 | 269 | 189 | 108 |
| 1400 - 2800 | 108 | 633 | 84 | 268 | 469 | 312 | 236 |
| > 2800 (w/o LOM) | 88 | 41 | 47 | 142 | 338 | 131 | 123 |
| > 2800 (LOM) | 1823 | 553 | 985 | 1462 | 1259 | 1217 | 480 |

w/o LOM = with LOM removed     LOM = Large organic matter

Table 40.  Observed Concentrations of Benzo(b)flourantrene at Hunter Creek

| Size Range (µm) | Concentration (µg/kg), Sample Number |  |  |  |  | Mean Concentration (µg/kg) | Standard Deviation |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8/18 | 9/9 | 9/26 | 10/9 | 10/21 |  |  |
| < 45 | 109 | 1551 | 859 | 1307 | 1418 | 1049 | 586 |
| 45 – 90 | 69 | 875 | 1102 | 1626 | 1883 | 1111 | 708 |
| 90 – 180 | 77 | 1505 | 856 | 168 | 479 | 617 | 582 |
| 180 – 355 | 113 | 521 | 124 | 22 | 466 | 249 | 227 |
| 355 – 710 | 50 | 313 | 514 | 111 | 479 | 294 | 210 |
| 710 – 1400 | 204 | 285 | 413 | 62 | 671 | 327 | 231 |
| 1400 – 2800 | 152 | 154 | 241 | 64 | 835 | 289 | 311 |
| > 2800 (w/o LOM) | 204 | 81 | 77 | 244 | 637 | 249 | 229 |
| > 2800 (LOM) | 1524 | 870 | 745 | 940 | 4642 | 1744 | 1647 |

w/o LOM = with LOM removed     LOM = Large organic matter

Table 41.  Observed Concentrations of Benzo(a)pyrene at Hunter Creek

| Size Range (µm) | Concentration (µg/kg), Sample Number |  |  |  |  | Mean Concentration (µg/kg) | Standard Deviation |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8/18 | 9/9 | 9/26 | 10/9 | 10/21 |  |  |
| < 45 | 675 | 2331 | 5421 | 7803 | 1514 | 3549 | 2979 |
| 45 - 90 | 401 | 782 | 2015 | 12311 | 2935 | 3689 | 4923 |
| 90 - 180 | 495 | 894 | 206 | 650 | 381 | 525 | 262 |
| 180 - 355 | 410 | 901 | 26 | 214 | 347 | 380 | 326 |
| 355 - 710 | 284 | 452 | NA | 45 | 524 | 326 | 213 |
| 710 - 1400 | 585 | 182 | 285 | 127 | 772 | 390 | 277 |
| 1400 - 2800 | 936 | 544 | 142 | 70 | 1055 | 550 | 448 |
| > 2800 (w/o LOM) | 431 | 279 | 144 | 100 | 544 | 300 | 188 |
| > 2800 (LOM) | 1023 | 1501 | 953 | 610 | 7556 | 2329 | 2939 |

w/o LOM = with LOM removed     LOM = Large organic matter

Table 42.  Observed Concentrations of Indeno(1,2,3-cd)pyrene at Hunter Creek

| Size Range (µm) | Concentration (µg/kg), Sample Number |  |  |  |  | Mean Concentration (µg/kg) | Standard Deviation |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8/18 | 9/9 | 9/26 | 10/9 | 10/21 |  |  |
| < 45 | 420 | 255 | 1622 | 1935 | 493 | 945 | 773 |
| 45 – 90 | 276 | 125 | 1985 | 1570 | 627 | 917 | 820 |
| 90 – 180 | 234 | 972 | 125 | 256 | 109 | 339 | 359 |
| 180 – 355 | 184 | 322 | ND | 128 | 342 | 244 | 105 |
| 355 – 710 | 344 | 302 | 514 | 158 | 531 | 370 | 156 |
| 710 – 1400 | 492 | 80 | 100 | 216 | 3020 | 782 | 1262 |
| 1400 – 2800 | 626 | 403 | 317 | 209 | 1751 | 661 | 628 |
| > 2800 (w/o LOM) | 126 | 57 | 315 | 1723 | 624 | 569 | 682 |
| > 2800 (LOM) | 1203 | 588 | 542 | 2403 | 1505 | 1248 | 764 |

w/o LOM = with LOM removed     LOM = Large organic matter

Table 43.  Observed Concentrations of Dibenz(a,h)anthracene at Hunter Creek

| Size Range (µm) | Concentration (µg/kg), Sample Number |  |  |  |  | Mean Concentration (µg/kg) | Standard Deviation |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8/18 | 9/9 | 9/26 | 10/9 | 10/21 |  |  |
| < 45 | 333 | 1962 | 4876 | 6448 | 433 | 2810 | 2739 |
| 45 – 90 | 217 | 918 | 2541 | 3630 | 925 | 1646 | 1399 |
| 90 – 180 | 183 | 1264 | 259 | 533 | 108 | 469 | 472 |
| 180 – 355 | 138 | 726 | 154 | 30 | 432 | 296 | 283 |
| 355 – 710 | 299 | 523 | 214 | 128 | 713 | 375 | 239 |
| 710 – 1400 | 412 | 431 | 512 | 206 | 939 | 500 | 270 |
| 1400 – 2800 | 488 | 625 | 112 | 220 | 1549 | 599 | 569 |
| > 2800 (w/o LOM) | 312 | 69 | 309 | 356 | 524 | 314 | 163 |
| > 2800 (LOM) | 1724 | 1385 | 679 | 802 | 864 | 1091 | 445 |

Table 44.  Observed Concentrations of Benzo(ghi)perylene at Hunter Creek

| Size Range (µm) | Concentration (µg/kg), Sample Number |  |  |  |  | Mean Concentration (µg/kg) | Standard Deviation |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8/18 | 9/9 | 9/26 | 10/9 | 10/21 |  |  |
| < 45 | 96 | 1390 | 2016 | 2247 | 894 | 1329 | 870 |
| 45 – 90 | 62 | 978 | 525 | 803 | 1683 | 810 | 598 |
| 90 – 180 | 59 | 1165 | 558 | ND | 176 | 490 | 498 |
| 180 – 355 | 43 | 782 | 57 | ND | 617 | 375 | 381 |
| 355 – 710 | 49 | 328 | 224 | 172 | 364 | 228 | 126 |
| 710 – 1400 | 67 | 734 | 219 | 173 | 1318 | 502 | 524 |
| 1400 – 2800 | 124 | 523 | 242 | 117 | 972 | 396 | 362 |
| > 2800 (w/o LOM) | 53 | 147 | 836 | 162 | 646 | 369 | 349 |
| > 2800 (LOM) | 1073 | 4222 | 1400 | 2092 | 7102 | 3178 | 2513 |

w/o LOM = with LOM removed     LOM = Large organic matter

Table 45.  Observed Concentrations of Naphthalene at Carroll’s Creek

| Size Range (µm) | Concentration (µg/kg), Sample Number |  |  |  |  | Mean Concentration (µg/kg) | Standard Deviation |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8/18 | 9/9 | 9/26 | 10/9 | 10/21 |  |  |
| < 45 | 22 | 561 | 95 | 102 | 217 | 199 | 214 |
| 45 - 90 | 81 | 425 | 157 | 30 | 165 | 171 | 152 |
| 90 - 180 | 163 | 11 | 5 | 15 | 102 | 59 | 71 |
| 180 - 355 | 15 | 157 | 31 | 55 | 93 | 70 | 57 |
| 355 - 710 | 101 | 65 | 464 | 27 | 58 | 143 | 181 |
| 710 - 1400 | 99 | 31 | 782 | 16 | 379 | 261 | 326 |
| 1400 - 2800 | 107 | 759 | 102 | 215 | 286 | 294 | 271 |
| > 2800 (w/o LOM) | 27 | 183 | 36 | 169 | 231 | 129 | 92 |
| > 2800 (LOM) | 1702 | 672 | 1819 | 3029 | 863 | 1617 | 936 |

w/o LOM = with LOM removed     LOM = Large organic matter

Table 46.  Observed Concentrations of Fluorene at Carroll’s Creek

| Size Range (µm) | Concentration (µg/kg), Sample Number |  |  |  |  | Mean Concentration (µg/kg) | Standard Deviation |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8/18 | 9/9 | 9/26 | 10/9 | 10/21 |  |  |
| < 45 | 27 | 175 | 228 | 169 | 121 | 144 | 76 |
| 45 - 90 | 184 | 113 | 147 | 266 | 102 | 163 | 66 |
| 90 - 180 | 252 | ND | 32 | 99 | 162 | 137 | 94 |
| 180 - 355 | 78 | 31 | 101 | 301 | 121 | 126 | 103 |
| 355 - 710 | 207 | 11 | 68 | 186 | 69 | 108 | 84 |
| 710 - 1400 | 313 | ND | 307 | 141 | 216 | 244 | 82 |
| 1400 - 2800 | 224 | 119 | 152 | 187 | 301 | 197 | 70 |
| > 2800 (w/o LOM) | 364 | 723 | 210 | 180 | 169 | 329 | 234 |
| > 2800 (LOM) | 1263 | 3621 | 687 | 2106 | 1623 | 1860 | 1113 |

w/o LOM = with LOM removed     LOM = Large organic matter

Table 47.  Observed Concentrations of Phenanthrene at Carroll’s Creek

| Size Range (µm) | Concentration (µg/kg), Sample Number |  |  |  |  | Mean Concentration (µg/kg) | Standard Deviation |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8/18 | 9/9 | 9/26 | 10/9 | 10/21 |  |  |
| < 45 | 7 | 63 | 8 | 260 | 125 | 92 | 105 |
| 45 - 90 | 30 | 64 | 4 | 236 | 96 | 86 | 91 |
| 90 - 180 | 44 | 37 | ND | 69 | 81 | 58 | 21 |
| 180 - 355 | 17 | 45 | 3 | 241 | 38 | 69 | 98 |
| 355 - 710 | 82 | 23 | 178 | 216 | 54 | 111 | 83 |
| 710 - 1400 | 38 | 49 | 33 | 84 | 81 | 57 | 24 |
| 1400 - 2800 | 92 | 138 | 9 | 124 | 72 | 87 | 51 |
| > 2800 (w/o LOM) | 56 | 172 | 166 | 466 | 98 | 192 | 161 |
| > 2800 (LOM) | 1253 | 3802 | 627 | 1732 | 2183 | 1920 | 1200 |

w/o LOM = with LOM removed     LOM = Large organic matter

Table 48.  Observed Concentrations of Anthracene at Carroll’s Creek

| Size Range (µm) | Concentration (µg/kg), Sample Number |  |  |  |  | Mean Concentration (µg/kg) | Standard Deviation |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8/18 | 9/9 | 9/26 | 10/9 | 10/21 |  |  |
| < 45 | 113 | 104 | 88 | 524 | 261 | 218 | 185 |
| 45 - 90 | 115 | 55 | 44 | 69 | 99 | 77 | 30 |
| 90 - 180 | 39 | ND | ND | 105 | 64 | 69 | 33 |
| 180 - 355 | 102 | 77 | 50 | 290 | 132 | 130 | 94 |
| 355 - 710 | 62 | 22 | 53 | 241 | 59 | 87 | 88 |
| 710 - 1400 | 67 | ND | 26 | 317 | 144 | 138 | 128 |
| 1400 - 2800 | 127 | 231 | 128 | 564 | 231 | 256 | 180 |
| > 2800 (w/o LOM) | 48 | 113 | 163 | 284 | 109 | 144 | 88 |
| > 2800 (LOM) | 1430 | 2987 | 627 | 1027 | 3724 | 1959 | 1332 |

w/o LOM = with LOM removed     LOM = Large organic matter

Table 49.  Observed Concentrations of Fluoranthene at Carroll’s Creek

| Size Range (µm) | Concentration (µg/kg), Sample Number |  |  |  |  | Mean Concentration (µg/kg) | Standard Deviation |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8/18 | 9/9 | 9/26 | 10/9 | 10/21 |  |  |
| < 45 | 60 | 37 | 66 | 154 | 106 | 85 | 46 |
| 45 - 90 | 41 | 39 | 28 | 104 | 182 | 79 | 65 |
| 90 - 180 | 59 | 63 | 34 | 88 | 213 | 91 | 71 |
| 180 - 355 | 18 | 15 | 31 | 137 | 99 | 60 | 55 |
| 355 - 710 | 43 | 21 | 53 | 100 | 71 | 58 | 30 |
| 710 - 1400 | 29 | 27 | 61 | 50 | 162 | 66 | 56 |
| 1400 - 2800 | 93 | 48 | 43 | 99 | 63 | 69 | 26 |
| > 2800 (w/o LOM) | 103 | 69 | 61 | 285 | 121 | 128 | 91 |
| > 2800 (LOM) | 1026 | 624 | 2102 | 3281 | 712 | 1549 | 1133 |

w/o LOM = with LOM removed     LOM = Large organic matter

Table 50.  Observed Concentrations of Pyrene at Carroll’s Creek

| Size Range (µm) | Concentration (µg/kg), Sample Number |  |  |  |  | Mean Concentration (µg/kg) | Standard Deviation |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8/18 | 9/9 | 9/26 | 10/9 | 10/21 |  |  |
| < 45 | 21 | 43 | 65 | 211 | 79 | 84 | 75 |
| 45 - 90 | 10 | 16 | 67 | 279 | 120 | 99 | 110 |
| 90 - 180 | 19 | 21 | 60 | 237 | 153 | 98 | 95 |
| 180 - 355 | 31 | 32 | 60 | 249 | 130 | 101 | 92 |
| 355 - 710 | 17 | 101 | 82 | 231 | 103 | 107 | 78 |
| 710 - 1400 | 31 | 87 | 49 | 184 | 301 | 130 | 112 |
| 1400 - 2800 | 51 | 73 | 51 | 202 | 278 | 131 | 104 |
| > 2800 (w/o LOM) | 41 | 71 | 121 | 207 | 182 | 125 | 71 |
| > 2800 (LOM) | 3120 | 872 | 3604 | 2892 | 927 | 2283 | 1289 |

w/o LOM = with LOM removed     LOM = Large organic matter

Table 51.  Observed Concentrations of Benzo(a)anthracene at Carroll’s Creek

| Size Range (µm) | Concentration (µg/kg), Sample Number |  |  |  |  | Mean Concentration (µg/kg) | Standard Deviation |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8/18 | 9/9 | 9/26 | 10/9 | 10/21 |  |  |
| < 45 | 33 | 179 | 42 | 131 | 99 | 97 | 61 |
| 45 – 90 | 16 | 203 | 21 | 47 | 132 | 84 | 81 |
| 90 – 180 | 48 | 22 | 10 | 63 | 219 | 72 | 84 |
| 180 – 355 | 43 | 66 | 16 | 110 | 98 | 67 | 39 |
| 355 – 710 | 13 | 116 | 152 | 128 | 112 | 104 | 53 |
| 710 – 1400 | 65 | 41 | 99 | 124 | 382 | 142 | 138 |
| 1400 – 2800 | 99 | 234 | 23 | 223 | 312 | 178 | 115 |
| > 2800 (w/o LOM) | 194 | 452 | 224 | 323 | 110 | 261 | 132 |
| > 2800 (LOM) | 1782 | 2039 | 589 | 3026 | 1902 | 1868 | 868 |

w/o LOM = with LOM removed     LOM = Large organic matter

Table 52.  Observed Concentrations of Chrysene at Carroll’s Creek

| Size Range (µm) | Concentration (µg/kg), Sample Number |  |  |  |  | Mean Concentration (µg/kg) | Standard Deviation |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8/18 | 9/9 | 9/26 | 10/9 | 10/21 |  |  |
| < 45 | 72 | 155 | 34 | 631 | 342 | 247 | 246 |
| 45 - 90 | 28 | 67 | 19 | 279 | 423 | 163 | 180 |
| 90 - 180 | 41 | 13 | 12 | 553 | 240 | 172 | 234 |
| 180 - 355 | 68 | 46 | 12 | 104 | 132 | 72 | 47 |
| 355 - 710 | 22 | 101 | 131 | 268 | 200 | 144 | 94 |
| 710 - 1400 | 89 | ND | 24 | 160 | 178 | 113 | 71 |
| 1400 - 2800 | 48 | 196 | 21 | 538 | 312 | 223 | 212 |
| > 2800 (w/o LOM) | 89 | 362 | 60 | 429 | 214 | 231 | 163 |
| > 2800 (LOM) | 1862 | 2973 | 894 | 969 | 2262 | 1792 | 881 |

w/o LOM = with LOM removed     matter     LOM = Large organic matter

Table 53.  Observed Concentrations of Benzo(b)fluoranthene at Carroll’s Creek

| Size Range (µm) | Concentration (µg/kg), Sample Number |  |  |  |  | Mean Concentration (µg/kg) | Standard Deviation |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8/18 | 9/9 | 9/26 | 10/9 | 10/21 |  |  |
| < 45 | 20 | 134 | 27 | 343 | 217 | 148 | 136 |
| 45 – 90 | 23 | 84 | 20 | 573 | 217 | 184 | 232 |
| 90 – 180 | 322 | 23 | 179 | 125 | 98 | 150 | 111 |
| 180 – 355 | 16 | 43 | 20 | 38 | 92 | 42 | 30 |
| 355 – 710 | 44 | 124 | 62 | 213 | 318 | 152 | 114 |
| 710 – 1400 | 21 | 25 | 18 | 61 | 129 | 51 | 47 |
| 1400 – 2800 | 162 | 206 | 6 | 314 | 313 | 200 | 127 |
| > 2800 (w/o LOM) | 179 | 178 | 90 | 315 | 216 | 196 | 81 |
| > 2800 (LOM) | 1027 | 2712 | 638 | 4023 | 3102 | 2301 | 1429 |

w/o LOM = with LOM removed     LOM = Large organic matter

Table 54.  Observed Concentrations of Benzo(a)pyrene at Carroll’s Creek

| Size Range (µm) | Concentration (µg/kg), Sample Number |  |  |  |  | Mean Concentration (µg/kg) | Standard Deviation |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8/18 | 9/9 | 9/26 | 10/9 | 10/21 |  |  |
| < 45 | 473 | 180 | 93 | 231 | 289 | 253 | 142 |
| 45 - 90 | 431 | 82 | 72 | 536 | 313 | 287 | 207 |
| 90 - 180 | 1049 | 14 | 1951 | 228 | 612 | 771 | 769 |
| 180 - 355 | 303 | 39 | 104 | 31 | 123 | 120 | 110 |
| 355 - 710 | 481 | ND | 54 | 635 | 146 | 329 | 274 |
| 710 - 1400 | 340 | ND | 3 | 240 | 128 | 178 | 145 |
| 1400 - 2800 | 637 | 194 | 6 | 189 | 214 | 248 | 233 |
| > 2800 (w/o LOM) | 263 | 205 | 127 | 295 | 261 | 230 | 66 |
| > 2800 (LOM) | 1526 | 3027 | 3627 | 453 | 1729 | 2073 | 1262 |

w/o LOM = with LOM removed     LOM = Large organic matter

Table 55.  Observed Concentrations of Indeno(1,2,3-cd)pyrene at Carroll’s Creek

| Size Range (µm) | Concentration (µg/kg), Sample Number |  |  |  |  | Mean Concentration (µg/kg) | Standard Deviation |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8/18 | 9/9 | 9/26 | 10/9 | 10/21 |  |  |
| < 45 | 245 | 216 | 1494 | 1302 | 587 | 769 | 596 |
| 45 – 90 | 159 | ND | 739 | 672 | 321 | 473 | 278 |
| 90 – 180 | 821 | ND | 2167 | 1555 | 587 | 1282 | 719 |
| 180 – 355 | 118 | 76 | 603 | 33 | 152 | 197 | 232 |
| 355 – 710 | 265 | 211 | 341 | 32 | 101 | 190 | 125 |
| 710 – 1400 | 114 | ND | 120 | 614 | 421 | 317 | 244 |
| 1400 – 2800 | 158 | 184 | 43 | 562 | 305 | 251 | 198 |
| > 2800 (w/o LOM) | 312 | 237 | 102 | 43 | 211 | 181 | 108 |
| > 2800 (LOM) | 1672 | 672 | 2134 | 3273 | 1903 | 1931 | 935 |

w/o LOM = with LOM removed     LOM = Large organic matter

Table 56.  Observed Concentrations of Dibenz(a,h)anthracene at Carroll’s Creek

| Size Range (µm) | Concentration (µg/kg), Sample Number |  |  |  |  | Mean Concentration (µg/kg) | Standard Deviation |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8/18 | 9/9 | 9/26 | 10/9 | 10/21 |  |  |
| < 45 | 204 | 266 | 463 | 655 | 321 | 382 | 180 |
| 45 – 90 | 130 | 160 | 93 | 142 | 204 | 146 | 41 |
| 90 – 180 | 690 | 54 | 1838 | 856 | 512 | 790 | 658 |
| 180 – 355 | 91 | 110 | 97 | 115 | 291 | 141 | 84 |
| 355 – 710 | 214 | 191 | 402 | 114 | 301 | 245 | 111 |
| 710 – 1400 | 96 | 63 | ND | 1248 | 682 | 522 | 561 |
| 1400 – 2800 | 165 | 132 | 44 | 623 | 321 | 257 | 228 |
| > 2800 (w/o LOM) | 158 | 146 | 128 | 146 | 289 | 173 | 65 |
| > 2800 (LOM) | 1700 | 982 | 1110 | 673 | 2692 | 1432 | 797 |

w/o LOM = with LOM removed     LOM = Large organic matter

Table 57.  Observed Concentrations of Benzo(ghi)perylene at Carroll’s Creek

| Size Range (µm) | Concentration (µg/kg), Sample Number |  |  |  |  | Mean Concentration (µg/kg) | Standard Deviation |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8/18 | 9/9 | 9/26 | 10/9 | 10/21 |  |  |
| < 45 | 56 | 172 | 522 | 371 | 261 | 277 | 180 |
| 45 – 90 | 132 | ND | 95 | 425 | 190 | 210 | 148 |
| 90 – 180 | 164 | 195 | 623 | 2578 | 528 | 818 | 1004 |
| 180 – 355 | 39 | 123 | 98 | 43 | 301 | 121 | 107 |
| 355 – 710 | 62 | ND | 365 | 50 | 112 | 147 | 148 |
| 710 – 1400 | 59 | ND | 263 | 1006 | 492 | 455 | 408 |
| 1400 – 2800 | 112 | 331 | 175 | 523 | 321 | 293 | 160 |
| > 2800 (w/o LOM) | 172 | 237 | 326 | 171 | 302 | 242 | 72 |
| > 2800 (LOM) | 1527 | 2386 | 3027 | 982 | 524 | 1689 | 1020 |

w/o LOM = with LOM removed     LOM = Large organic matter

# Analyses of Particulate Pollutant Strengths by Particle Size

The following tables and figures summarize the normalized particulate pollutant strengths from the prior listed data. These are normalized values that can be used to modify the standard pollutant concentration files in WinSLAMM to represent particulate strength concentrations by particle size. The WinSLAMM pollutant files are divided into particulate strength and filtered concentrations. The particulate strength values represent the bulk value occurring rain events, which are generally less than about 200 μm in size. Therefore, the particulate strength values by particle size shown above were normalized by comparing each particle size range by the averages of the <200 μm values (varied between 180 to 250 μm). In some cases (SERDP 2018 2022 metals and PAHs), data were only available between 0.45 and 63 μm, so the normalizing ratios were calculated using that range.

Table 58 is an example calculation showing the normalization for volatile solids. The average volatile solids values for <200 μm (closest here is 180 μm) is 24.1%. Therefore, each of the individual values in each size range were divided by this value to obtain the ratio values shown on the table.

Table 58. Example Normalized Volatile Solids Values by Particle Size Ranges (Cai 2013)

| Sieve size range (um) | Volatile Solids (%) | ratio with <200 μm |
| --- | --- | --- |
| <45 (Pan) | 26 | 1.08 |
| 45-75 | 25.7 | 1.07 |
| 75-180 | 20.6 | 0.85 |
| 180-355 | 19.4 | 0.80 |
| 355-710 | 26.1 | 1.08 |
| 710-1400 | 42.7 | 1.77 |
| 1400 - 2800 | 57.8 | 2.40 |
| >2800 | 70.9 | 2.94 |
| Sticks | 81.2 | 3.37 |
| Waterlogged decomposing leaves | 93.2 | 3.87 |

Figure 2 shows these ratios for particle size ranges used in WinSLAMM. The values from Table 58 are placed in the size ranges closest to the observed values. A plot of these ratios is also shown on Figure 2 showing how the volatile solids increase for the larger particle sizes, and Figure 3 shows the close relationship between specific gravity and volatile solids (organic content).

| WinSLAMM particle size range (um) | volatile solids (Cai 2013) |
| --- | --- |
| <1 |  |
| 1 to 2 |  |
| 2 to 5 |  |
| 5 to 10 |  |
| 10 to 20 |  |
| 20 to 40 |  |
| 40 to 50 | 1.08 |
| 50 to 60 | 1.07 |
| 60 to 80 | 1.07 |
| 80 to 100 | 0.85 |
| 100 to 150 | 0.85 |
| 150 to 200 | 0.85 |
| 200 to 300 | 0.80 |
| 300 to 500 | 1.08 |
| 500 to 800 | 1.08 |
| 800 to 1000 | 1.77 |
| 1000 to 2000 | 1.77 |
| >2000 | 2.67 |
| Sticks | 3.37 |
| Waterlogged decomposing leaves | 3.87 |

Figure 2. Volatile solids ratios for WinSLAMM particle size ranges.

Figure 3. Specific gravity and volatile solids.

These ratios are then used in WinSLAMM to distribute the calibrated gross particulate strength values for the range of particle sizes. In many cases, coefficient of variation values (the ratio of standard deviation to mean) are also shown that illustrate the variability to be expected with these data. Typical COV values range from about 0.7 to 1.3. The particulate strength ratios shown here should be used as a starting point when modeling an area. Locally obtained particulate strength ratio values should always be used when available. Summary tables and associated plots for many constituents are shown in the following subsections.

## Specific Gravity

The specific gravity values shown below are not ratios, but the actual values observed for the size ranges. Specific gravity is an important factor when calculating settling rates and therefore particulate removal performance of many stormwater controls.

| WinSLAMM particle size range (um) | specific gravity (g/cm3) (Cai 2013) |
| --- | --- |
| <1 |  |
| 1 to 2 |  |
| 2 to 5 |  |
| 5 to 10 |  |
| 10 to 20 |  |
| 20 to 40 |  |
| 40 to 50 | 3.46 |
| 50 to 60 | 3.30 |
| 60 to 80 | 3.30 |
| 80 to 100 | 2.97 |
| 100 to 150 | 2.97 |
| 150 to 200 | 2.76 |
| 200 to 300 | 2.76 |
| 300 to 500 | 2.56 |
| 500 to 800 | 1.43 |
| 800 to 1000 | 1.43 |
| 1000 to 2000 | 1.15 |
| >2000 | 0.66 |
| Sticks | 0.84 |
| Waterlogged decomposing leaves | 2.28 (much larger than for recently fallen leaves) |

Figure 4. Specific gravity vs. particle sizes.

## COD, TOC, and Nutrients

| WinSLAMM particle size range (um) | roof and outlet ratio with <250 (Morquecho 2005) | inlet ratio with <250 um (Pitt 1985) | sump ratio with <250 (Khambhammettu 2005) | sump ratio with <180 (Cai 2013) | average COD ratio |
| --- | --- | --- | --- | --- | --- |
| <1 | 1.30 |  |  |  | 1.30 |
| 1 to 2 | 1.20 |  |  |  | 1.20 |
| 2 to 5 | 0.66 |  |  |  | 0.66 |
| 5 to 10 | 0.66 |  |  |  | 0.66 |
| 10 to 20 | 0.67 |  |  |  | 0.67 |
| 20 to 40 | 0.67 |  |  |  | 0.67 |
| 40 to 50 | 0.98 |  |  | 1.32 | 1.15 |
| 50 to 60 | 0.98 | 1.26 |  | 1.05 | 1.09 |
| 60 to 80 | 0.98 | 1.02 | 1.76 | 1.05 | 1.20 |
| 80 to 100 | 0.98 | 1.02 | 0.97 | 0.64 | 0.90 |
| 100 to 150 | 1.19 | 1.02 | 0.97 | 0.64 | 0.96 |
| 150 to 200 |  | 0.72 | 0.27 | 0.64 | 0.54 |
| 200 to 300 |  | 0.72 | 0.45 | 0.80 | 0.66 |
| 300 to 500 |  | 0.79 | 0.45 | 0.80 | 0.68 |
| 500 to 800 |  | 1.10 | 0.34 | 0.66 | 0.70 |
| 800 to 1000 |  | 1.10 | 0.22 | 1.26 | 0.86 |
| 1000 to 2000 |  | 1.96 | 0.22 | 1.25 | 1.15 |
| >2000 | 1.19 | 2.00 | 1.49 | 1.04 | 1.43 |
|  |  |  |  | Sticks | 1.28 |
|  |  |  |  | Leaves | 3.35 |
|  |  |  |  | avg COV | 0.97 |

Figure 5. COD particulate strength ratios by size range.

| WinSLAMM particle size range (um) | resid and commer avg TOC ratio with <63 (SERDP 2018) |
| --- | --- |
| <1 | 2.20 |
| 1 to 2 | 2.20 |
| 2 to 5 | 2.20 |
| 5 to 10 | 0.29 |
| 10 to 20 | 0.29 |
| 20 to 40 | 0.52 |
| 40 to 50 | 0.52 |
| 50 to 60 | 0.52 |
| 60 to 80 |  |
| 80 to 100 |  |
| 100 to 150 |  |
| 150 to 200 |  |
| 200 to 300 |  |
| 300 to 500 |  |
| 500 to 800 |  |
| 800 to 1000 |  |
| 1000 to 2000 |  |
| >2000 | 0.81 |
| avg COV | 0.66 |

Figure 6. Residential and commercial area TOC particulate strength ratios by size range.

| WinSLAMM particle size range (um) | indus avg TOC ratio with <63 (SERDP 2018) |
| --- | --- |
| <1 | 0.95 |
| 1 to 2 | 0.95 |
| 2 to 5 | 0.95 |
| 5 to 10 | 0.63 |
| 10 to 20 | 0.63 |
| 20 to 40 | 1.42 |
| 40 to 50 | 1.42 |
| 50 to 60 | 1.42 |
| 60 to 80 |  |
| 80 to 100 |  |
| 100 to 150 |  |
| 150 to 200 |  |
| 200 to 300 |  |
| 300 to 500 |  |
| 500 to 800 |  |
| 800 to 1000 |  |
| 1000 to 2000 |  |
| >2000 | 2.75 |
| avg COV: | 0.63 |

Figure 7. Industrial area TOC particulate strength ratios by size range.

| WinSLAMM particle size range (um) | roof and inlet ratio with <250 (Morquecho 2005) | ratio with <250 um (Pitt 1985) | ratio with <250 (Khambhammettu 2005) | ratio with <180 (Cai 2013) | avg TP ratio with <250 um |
| --- | --- | --- | --- | --- | --- |
| <1 | 1.91 |  |  |  | 1.91 |
| 1 to 2 | 1.68 |  |  |  | 1.68 |
| 2 to 5 | 0.71 |  |  |  | 0.71 |
| 5 to 10 | 0.71 |  |  |  | 0.71 |
| 10 to 20 | 0.81 |  |  |  | 0.81 |
| 20 to 40 | 0.81 |  |  |  | 0.81 |
| 40 to 50 | 0.47 |  |  | 1.10 | 0.78 |
| 50 to 60 | 0.47 | 1.20 |  | 1.33 | 1.00 |
| 60 to 80 | 0.47 | 0.94 | 1.88 | 1.33 | 1.16 |
| 80 to 100 | 0.47 | 0.94 | 0.85 | 0.57 | 0.71 |
| 100 to 150 | 0.42 | 0.86 | 0.85 | 0.57 | 0.68 |
| 150 to 200 | 0.42 | 0.86 | 0.27 | 0.57 | 0.53 |
| 200 to 300 | 0.42 | 0.86 | 0.27 | 0.55 | 0.52 |
| 300 to 500 | 1.31 | 0.83 | 0.17 | 0.50 | 0.70 |
| 500 to 800 | 1.31 | 0.75 | 0.26 | 0.50 | 0.70 |
| 800 to 1000 | 1.31 | 0.75 | 0.45 | 0.26 | 0.69 |
| 1000 to 2000 | 1.31 | 1.27 | 0.45 | 0.51 | 0.88 |
| >2000 | 1.31 | 1.27 | 0.81 | 0.21 | 0.90 |
|  |  |  |  | Sticks | 0.19 |
|  |  |  |  | Leaves | 0.21 |
|  |  |  |  | avg COV | 0.97 |

Figure 8. Total phosphorus particulate strength ratios by size range.

| WinSLAMM particle size range (um) | ratio with <250 um (Pitt 1985) | ratio with <180 (Cai 2013) | TN ratio with <250 um |
| --- | --- | --- | --- |
| <1 |  |  |  |
| 1 to 2 |  |  |  |
| 2 to 5 |  |  |  |
| 5 to 10 |  |  |  |
| 10 to 20 |  |  |  |
| 20 to 40 |  |  |  |
| 40 to 50 |  | 1.29 | 1.29 |
| 50 to 60 | 1.34 | 0.81 | 1.07 |
| 60 to 80 | 0.97 | 0.81 | 0.89 |
| 80 to 100 | 0.97 | 0.90 | 0.93 |
| 100 to 150 | 0.97 | 0.90 | 0.93 |
| 150 to 200 | 0.69 | 0.90 | 0.79 |
| 200 to 300 | 0.69 | 0.28 | 0.49 |
| 300 to 500 | 0.74 | 0.38 | 0.56 |
| 500 to 800 | 0.74 | 0.38 | 0.56 |
| 800 to 1000 | 0.74 | 0.91 | 0.83 |
| 1000 to 2000 | 1.20 | 1.26 | 1.23 |
| >2000 | 1.06 | 1.29 | 1.18 |
|  |  | Sticks | 1.23 |
|  |  | Leaves | 0.85 |

Figure 9. Total nitrogen particulate strength ratios by size range.

## Heavy Metals

Cai (2013) also separated leaves and sticks and analyzed these separately from the >2800 um size fraction. These larger sizes are therefore shown separately in the summary tables but are not shown on the plots.

| WinSLAMM particle size range (um) | roof and outlet Cd ratio with <250 (Morquecho 2005) | sump Cd ratio with <180 (Cai 2013) | res/com Cd ratio with <63 (SERDP 2018) | avg Cd ratio with <180 |
| --- | --- | --- | --- | --- |
| <1 | 0.83 |  |  | 0.83 |
| 1 to 2 | 1.94 |  |  | 1.94 |
| 2 to 5 | 2.12 |  |  | 2.12 |
| 5 to 10 | 2.12 |  | 1.16 | 1.64 |
| 10 to 20 | 0.06 |  | 1.16 | 0.61 |
| 20 to 40 | 0.06 |  | 0.84 | 0.45 |
| 40 to 50 |  | 0.47 | 0.84 | 0.65 |
| 50 to 60 |  | 0.39 | 0.84 | 0.61 |
| 60 to 80 |  | 0.39 | 4.84 | 2.61 |
| 80 to 100 | 0.04 | 2.14 |  | 1.09 |
| 100 to 150 | 0.04 | 2.14 |  | 1.09 |
| 150 to 200 | 0.04 | 2.14 |  | 1.09 |
| 200 to 300 | 0.04 |  |  | 0.04 |
| 300 to 500 |  | 0.14 |  | 0.14 |
| 500 to 800 |  | 0.16 |  | 0.16 |
| 800 to 1000 |  | 0.16 |  | 0.16 |
| 1000 to 2000 |  | 0.16 |  | 0.16 |
| >2000 | 0.09 |  |  |  |
|  |  |  | Leaves | 0.41 |
|  |  |  | overall avg COV | 0.87 |

Figure 10. Cadmium particulate strength ratios by size range.

| WinSLAMM particle size range (um) | indus Cd ratio with <63 (SERDP 2018) |
| --- | --- |
| <1 |  |
| 1 to 2 |  |
| 2 to 5 | 0.44 |
| 5 to 10 | 0.44 |
| 10 to 20 | 0.44 |
| 20 to 40 | 1.56 |
| 40 to 50 | 1.56 |
| 50 to 60 | 1.56 |
| 60 to 80 |  |
| 80 to 100 |  |
| 100 to 150 |  |
| 150 to 200 |  |
| 200 to 300 |  |
| 300 to 500 |  |
| 500 to 800 |  |
| 800 to 1000 |  |
| 1000 to 2000 |  |
| >2000 | 40.52 |
| avg COV | 1.11 |

Figure 11. Industrial area cadmium particulate strength ratios by size range.

| WinSLAMM particle size range (um) | roof and outlet Cu ratio with <250 (Morquecho 2005) | Sed Cu ratio with <250 (Khambhammettu 2005) | sump Cu ratio with <180 (Cai 2013) | res/com Cu ratio with <63 (SERDP 2018) | avg Cu ratio with <180 |
| --- | --- | --- | --- | --- | --- |
| <1 | 3.70 |  |  | 2.08 | 2.89 |
| 1 to 2 | 0.44 |  |  | 2.08 | 1.26 |
| 2 to 5 | 0.21 |  |  | 2.08 | 1.15 |
| 5 to 10 | 0.21 |  |  | 0.61 | 0.41 |
| 10 to 20 | 0.31 |  |  | 0.61 | 0.46 |
| 20 to 40 | 0.31 |  |  | 0.31 | 0.31 |
| 40 to 50 | 0.31 |  | 1.22 | 0.31 | 0.61 |
| 50 to 60 | 1.33 |  | 1.04 | 0.31 | 0.90 |
| 60 to 80 | 1.33 | 1.69 | 1.04 |  | 1.35 |
| 80 to 100 | 1.33 | 0.89 | 0.74 |  | 0.99 |
| 100 to 150 | 0.02 | 0.89 | 0.74 |  | 0.55 |
| 150 to 200 | 0.02 | 0.43 | 0.74 |  | 0.39 |
| 200 to 300 | 0.02 | 0.43 | 0.64 |  | 0.36 |
| 300 to 500 |  | 0.30 | 0.38 |  | 0.34 |
| 500 to 800 |  | 0.20 | 0.38 |  | 0.29 |
| 800 to 1000 |  | 0.25 | 0.46 |  | 0.35 |
| 1000 to 2000 |  | 0.25 | 0.25 |  | 0.25 |
| >2000 | 0.19 | 0.46 | 0.08 | 1.36 | 0.52 |
|  |  |  |  | Sticks | 0.13 |
|  |  |  |  | Leaves | 0.23 |
|  |  |  |  | overall avg COV | 0.71 |

Figure 12. Copper particulate strength ratios by size range.

| WinSLAMM particle size range (um) | indus Cu ratio with <63 (SERDP 2018) |
| --- | --- |
| <1 | 1.64 |
| 1 to 2 | 1.64 |
| 2 to 5 | 1.64 |
| 5 to 10 | 1.64 |
| 10 to 20 | 1.64 |
| 20 to 40 | 1.21 |
| 40 to 50 | 1.21 |
| 50 to 60 | 1.21 |
| 60 to 80 |  |
| 80 to 100 |  |
| 100 to 150 |  |
| 150 to 200 |  |
| 200 to 300 |  |
| 300 to 500 |  |
| 500 to 800 |  |
| 800 to 1000 |  |
| 1000 to 2000 |  |
| >2000 | 5.91 |
| overall avg COV | 0.94 |

Figure 13. Industrial area copper particulate strength ratios by size range.

| WinSLAMM particle size range (um) | roof and outlet Pb ratio with <250 (Morquecho 2005) | inlet Pb ratio with <250 (Pitt 1985) | sed Pb ratio with <250 (Khambhammettu 2005) | sump Pb ratio with <180 (Cai 2013) | res/com Pb ratio with <63 (SERDP 2018) | overall avg Pb ratio with <250 |
| --- | --- | --- | --- | --- | --- | --- |
| <1 | 4.42 |  |  |  | 1.71 | 3.07 |
| 1 to 2 | 0.79 |  |  |  | 1.71 | 1.25 |
| 2 to 5 | 0.16 |  |  |  | 0.87 | 0.51 |
| 5 to 10 | 0.16 |  |  |  | 0.87 | 0.51 |
| 10 to 20 | 0.24 |  |  |  | 0.87 | 0.55 |
| 20 to 40 | 0.24 |  |  |  | 0.42 | 0.33 |
| 40 to 50 | 0.32 |  |  | 1.05 | 0.42 | 0.60 |
| 50 to 60 | 0.32 | 1.34 |  | 1.23 | 0.42 | 0.83 |
| 60 to 80 | 0.32 | 0.97 | 1.30 | 1.23 | 0.42 | 0.85 |
| 80 to 100 | 0.32 | 0.97 | 1.14 | 0.72 |  | 0.79 |
| 100 to 150 | 0.07 | 0.69 | 1.14 | 0.72 |  | 0.65 |
| 150 to 200 | 0.07 | 0.69 | 0.56 | 0.72 |  | 0.51 |
| 200 to 300 |  | 0.69 | 0.46 | 0.30 |  | 0.48 |
| 300 to 500 |  | 0.62 | 0.38 | 0.32 |  | 0.44 |
| 500 to 800 |  | 0.60 | 0.38 | 0.32 |  | 0.43 |
| 800 to 1000 |  | 0.60 | 0.41 | 0.31 |  | 0.44 |
| 1000 to 2000 |  | 0.60 | 0.41 | 0.12 |  | 0.38 |
| >2000 | 0.62 | 0.43 | 0.71 | 0.04 | 1.96 | 0.75 |
|  |  |  |  |  | Sticks | 0.04 |
|  |  |  |  |  | Leaves | 0.09 |
|  |  |  |  |  | avg COV | 0.93 |

Figure 14. Lead particulate strength ratios by size range.

| WinSLAMM particle size range (um) | indus Pb ratio with <63 (SERDP 2018) |
| --- | --- |
| <1 | 0.95 |
| 1 to 2 | 0.95 |
| 2 to 5 | 0.95 |
| 5 to 10 | 0.57 |
| 10 to 20 | 0.57 |
| 20 to 40 | 1.48 |
| 40 to 50 | 1.48 |
| 50 to 60 | 1.48 |
| 60 to 80 |  |
| 80 to 100 |  |
| 100 to 150 |  |
| 150 to 200 |  |
| 200 to 300 |  |
| 300 to 500 |  |
| 500 to 800 |  |
| 800 to 1000 |  |
| 1000 to 2000 |  |
| >2000 | 3.04 |
| avg COV | 1.00 |

Figure 15. Industrial area lead particulate strength ratios by size range.

| WinSLAMM particle size range (um) | roof and outlet Zn ratio with <250 (Morquecho 2005) | inlet Zn ratio with <250 (Pitt 1985) | sed Zn ratio with <250 (Khambhammettu 2005) | sump Zn ratio with <180 (Cai 2013) | res/com Zn ratio with <63 (SERDP 2018) | overall avg Zn ratio with <250 |
| --- | --- | --- | --- | --- | --- | --- |
| <1 | 3.98 |  |  |  | 1.79 | 2.89 |
| 1 to 2 | 0.09 |  |  |  | 1.79 | 0.94 |
| 2 to 5 | 0.32 |  |  |  | 1.79 | 1.05 |
| 5 to 10 | 0.32 |  |  |  | 0.65 | 0.48 |
| 10 to 20 | 0.56 |  |  |  | 0.65 | 0.61 |
| 20 to 40 | 0.56 |  |  |  | 0.55 | 0.56 |
| 40 to 50 | 0.56 | 1.30 |  | 1.42 | 0.55 | 0.96 |
| 50 to 60 |  | 1.04 |  | 1.14 | 0.55 | 0.91 |
| 60 to 80 |  | 1.04 | 1.44 | 1.14 |  | 1.21 |
| 80 to 100 |  | 1.04 | 1.03 | 0.44 |  | 0.84 |
| 100 to 150 | 0.05 | 1.04 | 1.03 | 0.44 |  | 0.64 |
| 150 to 200 | 0.05 | 0.65 | 0.54 | 0.44 |  | 0.42 |
| 200 to 300 |  | 0.65 | 0.54 | 0.41 |  | 0.53 |
| 300 to 500 |  | 0.65 | 0.58 | 0.41 |  | 0.55 |
| 500 to 800 |  | 0.65 | 0.29 | 0.41 |  | 0.45 |
| 800 to 1000 |  | 0.65 | 0.44 | 0.32 |  | 0.47 |
| 1000 to 2000 |  | 0.75 | 0.44 | 0.18 |  | 0.46 |
| >2000 | 0.31 | 0.55 | 0.54 |  | 3.43 | 1.21 |
|  |  |  |  |  | Sticks | 0.27 |
|  |  |  |  |  | Leaves | 0.16 |
|  |  |  |  |  | avg COV | 0.77 |

Figure 16. Zinc particulate strength ratios by size range.

| WinSLAMM particle size range (um) | indus Zn ratio with <63 (SERDP 2018) |
| --- | --- |
| <1 | 1.59 |
| 1 to 2 | 1.59 |
| 2 to 5 | 1.59 |
| 5 to 10 | 1.23 |
| 10 to 20 | 1.23 |
| 20 to 40 | 1.23 |
| 40 to 50 | 1.23 |
| 50 to 60 | 1.23 |
| 60 to 80 |  |
| 80 to 100 |  |
| 100 to 150 |  |
| 150 to 200 |  |
| 200 to 300 |  |
| 300 to 500 |  |
| 500 to 800 |  |
| 800 to 1000 |  |
| 1000 to 2000 |  |
| >2000 | 7.97 |
| avg COV | 0.63 |

Figure 17. Industrial area zinc particulate strength ratios by size range.

| WinSLAMM particle range (um) | Sed Cr ratio with <250 (Khambhammettu 2005) | sump Cr ratio with <180 (Cai 2013) | overall avg Cr ratio with <250 |
| --- | --- | --- | --- |
| <1 |  |  |  |
| 1 to 2 |  |  |  |
| 2 to 5 |  |  |  |
| 5 to 10 |  |  |  |
| 10 to 20 |  |  |  |
| 20 to 40 |  |  |  |
| 40 to 50 |  | 1.26 | 1.26 |
| 50 to 60 |  | 1.09 | 1.09 |
| 60 to 80 | 1.36 | 1.09 | 1.23 |
| 80 to 100 | 1.12 | 0.65 | 0.89 |
| 100 to 150 | 1.12 | 0.65 | 0.89 |
| 150 to 200 | 0.52 | 0.65 | 0.58 |
| 200 to 300 | 0.43 | 0.68 | 0.56 |
| 300 to 500 | 0.24 | 0.78 | 0.51 |
| 500 to 800 | 0.24 | 0.78 | 0.51 |
| 800 to 1000 | 0.44 | 1.29 | 0.87 |
| 1000 to 2000 | 0.44 | 0.48 | 0.46 |
| >2000 | 0.64 | 0.48 | 0.56 |
|  |  | Sticks | 0.11 |
|  |  | Leaves | 0.19 |

Figure 18. Chromium particulate strength ratios by size range.

| WinSLAMM particle range (um) | Sed Fe ratio with <250 (Khambhammettu 2005) |
| --- | --- |
| <1 |  |
| 1 to 2 |  |
| 2 to 5 |  |
| 5 to 10 |  |
| 10 to 20 |  |
| 20 to 40 |  |
| 40 to 50 |  |
| 50 to 60 |  |
| 60 to 80 | 1.29 |
| 80 to 100 | 1.06 |
| 100 to 150 | 1.06 |
| 150 to 200 | 0.64 |
| 200 to 300 | 0.60 |
| 300 to 500 | 0.60 |
| 500 to 800 | 0.49 |
| 800 to 1000 | 0.87 |
| 1000 to 2000 | 0.87 |
| >2000 | 1.22 |

Figure 19. Iron particulate strength ratios by size range.

| WinSLAMM particle range (um) | res/com Hg ratio with <63 (SERDP 2018) |
| --- | --- |
| <1 | 0.49 |
| 1 to 2 | 0.49 |
| 2 to 5 | 0.49 |
| 5 to 10 | 1.58 |
| 10 to 20 | 1.58 |
| 20 to 40 | 0.93 |
| 40 to 50 | 0.93 |
| 50 to 60 | 0.93 |
| 60 to 80 |  |
| 80 to 100 |  |
| 100 to 150 |  |
| 150 to 200 |  |
| 200 to 300 |  |
| 300 to 500 |  |
| 500 to 800 |  |
| 800 to 1000 |  |
| 1000 to 2000 |  |
| >2000 | 3.94 |
| avg COV | 0.63 |

Figure 20. Mercury particulate strength ratios by size range.

| WinSLAMM particle range (um) | indus Hg ratio with <63 (SERDP 2018) |
| --- | --- |
| <1 | 0.69 |
| 1 to 2 | 0.69 |
| 2 to 5 | 0.69 |
| 5 to 10 | 0.38 |
| 10 to 20 | 0.38 |
| 20 to 40 | 1.93 |
| 40 to 50 | 1.93 |
| 50 to 60 | 1.93 |
| 60 to 80 |  |
| 80 to 100 |  |
| 100 to 150 |  |
| 150 to 200 |  |
| 200 to 300 |  |
| 300 to 500 |  |
| 500 to 800 |  |
| 800 to 1000 |  |
| 1000 to 2000 |  |
| >2000 | 6.46 |
| avg COV | 0.98 |

Figure 21. Industrial area mercury particulate strength ratios by size range.

| WinSLAMM particle range (um) | res/com Ni ratio with <63 (SERDP 2018) |
| --- | --- |
| <1 | 1.82 |
| 1 to 2 | 1.82 |
| 2 to 5 | 1.82 |
| 5 to 10 | 0.75 |
| 10 to 20 | 0.75 |
| 20 to 40 | 0.43 |
| 40 to 50 | 0.43 |
| 50 to 60 | 0.43 |
| 60 to 80 |  |
| 80 to 100 |  |
| 100 to 150 |  |
| 150 to 200 |  |
| 200 to 300 |  |
| 300 to 500 |  |
| 500 to 800 |  |
| 800 to 1000 |  |
| 1000 to 2000 |  |
| >2000 | 1.95 |
| avg COV | 0.89 |

Figure 22. Nickel particulate strength ratios by size range.

| WinSLAMM particle range (um) | indus Ni ratio with <63 (SERDP 2018) |
| --- | --- |
| <1 | 0.43 |
| 1 to 2 | 0.43 |
| 2 to 5 | 0.43 |
| 5 to 10 | 0.17 |
| 10 to 20 | 0.17 |
| 20 to 40 | 2.40 |
| 40 to 50 | 2.40 |
| 50 to 60 | 2.40 |
| 60 to 80 |  |
| 80 to 100 |  |
| 100 to 150 |  |
| 150 to 200 |  |
| 200 to 300 |  |
| 300 to 500 |  |
| 500 to 800 |  |
| 800 to 1000 |  |
| 1000 to 2000 |  |
| >2000 | 6.77 |
| avg COV | 0.51 |

Figure 23. Industrial area nickel particulate strength ratios by size range.

| WinSLAMM particle range (um) | res/com As ratio with <63 (SERDP 2018) |
| --- | --- |
| <1 | 1.73 |
| 1 to 2 | 1.73 |
| 2 to 5 | 1.73 |
| 5 to 10 | 0.77 |
| 10 to 20 | 0.77 |
| 20 to 40 | 0.50 |
| 40 to 50 | 0.50 |
| 50 to 60 | 0.50 |
| 60 to 80 |  |
| 80 to 100 |  |
| 100 to 150 |  |
| 150 to 200 |  |
| 200 to 300 |  |
| 300 to 500 |  |
| 500 to 800 |  |
| 800 to 1000 |  |
| 1000 to 2000 |  |
| >2000 | 12.82 |
| avg COV | 0.73 |

Figure 24. Arsenic particulate strength ratios by size range.

| WinSLAMM particle range (um) | indus As ratio with <63 (SERDP 2018) |
| --- | --- |
| <1 |  |
| 1 to 2 |  |
| 2 to 5 | 0.03 |
| 5 to 10 | 0.03 |
| 10 to 20 | 0.03 |
| 20 to 40 | 1.97 |
| 40 to 50 | 1.97 |
| 50 to 60 | 1.97 |
| 60 to 80 |  |
| 80 to 100 |  |
| 100 to 150 |  |
| 150 to 200 |  |
| 200 to 300 |  |
| 300 to 500 |  |
| 500 to 800 |  |
| 800 to 1000 |  |
| 1000 to 2000 |  |
| >2000 | 4.41 |
| avg COV | 0.69 |

Figure 24. Industrial area nickel particulate strength ratios by size range.

# PAHs

Bathi (2008) separated the large organic material (LOM) from the >2800 μm size fraction. These are shown separately on the tables but are not plotted on the graphs. Also, during the SERDP (2018 and 2022) analyses, the largest particle size analyzed separately was 63 μm, with all larger material grouped as >63 μm. The >63 μm are expected to represent the largest particles collected and not a broad distribution of sizes, based on the high particulate strength values and ratios. It is expected that these large particles are large organic material (LOM), such as leaf material or oily debris. The organic leaf material (expected in most urban areas) has a great attraction to PAHs, and the oily debris (expected in industrial areas) have high concentrations of the PAHs. These are therefore shown separately on the tables but are not included on the graphs. Limited PAH detections for the SERDP (2018) samples did not allow those samples to be separated by particle size, as was possible for the previously described heavy metal results.

| WinSLAMM particle range (um) | Naphthalene ratio with <63 (SERDP 2018) | Naphthalene ratio with <200 (Bathi 2008) | avg Naphthalene ratio with <200 |
| --- | --- | --- | --- |
| <1 | 1.45 |  | 1.45 |
| 1 to 2 | 1.45 |  | 1.45 |
| 2 to 5 | 0.23 |  | 0.23 |
| 5 to 10 | 0.23 |  | 0.23 |
| 10 to 20 | 0.23 |  | 0.23 |
| 20 to 40 | 1.32 |  | 1.32 |
| 40 to 50 | 1.32 | 1.28 | 1.30 |
| 50 to 60 | 1.32 | 0.89 | 1.11 |
| 60 to 80 |  | 0.89 | 0.89 |
| 80 to 100 |  | 0.89 | 0.89 |
| 100 to 150 |  | 0.82 | 0.82 |
| 150 to 200 |  | 0.82 | 0.82 |
| 200 to 300 |  | 0.47 | 0.47 |
| 300 to 500 |  | 0.63 | 0.63 |
| 500 to 800 |  | 0.63 | 0.63 |
| 800 to 1000 |  | 3.98 | 3.98 |
| 1000 to 2000 |  | 3.98 | 3.98 |
| >2000 |  | 4.49 | 4.49 |
| LOM and oily debris | 22.68 | 13.27 | 17.98 |
|  |  | avg COV | 1.28 |

Figure 26. Naphthalene particulate strength ratios by size range.

| WinSLAMM particle range (um) | Fluorene ratio with <63 (SERDP 2018) | Fluorene ratio with <200 (Bathi 2008) | avg Fluorene ratio with <200 |
| --- | --- | --- | --- |
| <1 | 0.88 |  | 0.88 |
| 1 to 2 | 0.88 |  | 0.88 |
| 2 to 5 | 0.88 |  | 0.88 |
| 5 to 10 | 0.50 |  | 0.50 |
| 10 to 20 | 0.50 |  | 0.50 |
| 20 to 40 | 1.62 |  | 1.62 |
| 40 to 50 | 1.62 | 1.16 | 1.39 |
| 50 to 60 | 1.62 | 0.85 | 1.24 |
| 60 to 80 |  | 0.85 | 0.85 |
| 80 to 100 |  | 0.85 | 0.85 |
| 100 to 150 |  | 0.99 | 0.99 |
| 150 to 200 |  | 0.99 | 0.99 |
| 200 to 300 |  | 0.56 | 0.56 |
| 300 to 500 |  | 0.62 | 0.62 |
| 500 to 800 |  | 0.62 | 0.62 |
| 800 to 1000 |  | 0.90 | 0.90 |
| 1000 to 2000 |  | 0.90 | 0.90 |
| >2000 |  | 1.15 | 1.15 |
| LOM and oily debris | 1.04 | 7.97 | 4.51 |
|  |  | avg COV | 1.47 |

Figure 27. Fluorene particulate strength ratios by size range.

| WinSLAMM particle range (um) | Acenaphthylene ratio with <63 (SERDP 2018) |
| --- | --- |
| <1 | 2.42 |
| 1 to 2 | 2.42 |
| 2 to 5 | 2.42 |
| 5 to 10 | 0.06 |
| 10 to 20 | 0.06 |
| 20 to 40 | 0.52 |
| 40 to 50 | 0.52 |
| 50 to 60 | 0.52 |
| 60 to 80 |  |
| 80 to 100 |  |
| 100 to 150 |  |
| 150 to 200 |  |
| 200 to 300 |  |
| 300 to 500 |  |
| 500 to 800 |  |
| 800 to 1000 |  |
| 1000 to 2000 |  |
| >2000 |  |
| LOM and oily debris | 4.30 |

Figure 28. Acenaphthylene particulate strength ratios by size range.

| WinSLAMM particle range (um) | Phenanthrene ratio with <63 (SERDP 2018) | Phenanthrene ratio with <200 (Bathi 2008) | avg Phenanthrene ratio with <200 |
| --- | --- | --- | --- |
| <1 | 0.22 |  | 0.22 |
| 1 to 2 | 0.22 |  | 0.22 |
| 2 to 5 | 0.22 |  | 0.22 |
| 5 to 10 | 0.96 |  | 0.96 |
| 10 to 20 | 0.96 |  | 0.96 |
| 20 to 40 | 1.83 |  | 1.83 |
| 40 to 50 | 1.83 | 1.31 | 1.57 |
| 50 to 60 | 1.83 | 1.02 | 1.42 |
| 60 to 80 |  | 1.02 | 1.02 |
| 80 to 100 |  | 1.02 | 1.02 |
| 100 to 150 |  | 0.66 | 0.66 |
| 150 to 200 |  | 0.66 | 0.66 |
| 200 to 300 |  | 0.47 | 0.47 |
| 300 to 500 |  | 0.53 | 0.53 |
| 500 to 800 |  | 0.53 | 0.53 |
| 800 to 1000 |  | 0.65 | 0.65 |
| 1000 to 2000 |  | 0.65 | 0.65 |
| >2000 |  | 0.96 | 0.96 |
| LOM and oily debris | 32.64 | 9.98 | 21.31 |
|  |  | avg COV | 1.23 |

Figure 29. Phenanthrene particulate strength ratios by size range.

| WinSLAMM particle range (um) | Anthracene ratio with <63 (SERDP 2018) | Anthracene ratio with <200 (Bathi 2008) | avg Anthracene ratio with <200 |
| --- | --- | --- | --- |
| <1 | 0.31 |  | 0.31 |
| 1 to 2 | 0.31 |  | 0.31 |
| 2 to 5 | 0.31 |  | 0.31 |
| 5 to 10 | 0.25 |  | 0.25 |
| 10 to 20 | 0.25 |  | 0.25 |
| 20 to 40 | 2.44 |  | 2.44 |
| 40 to 50 | 2.44 | 1.21 | 1.82 |
| 50 to 60 | 2.44 | 0.99 | 1.71 |
| 60 to 80 |  | 0.99 | 0.99 |
| 80 to 100 |  | 0.99 | 0.99 |
| 100 to 150 |  | 0.80 | 0.80 |
| 150 to 200 |  | 0.80 | 0.80 |
| 200 to 300 |  | 0.53 | 0.53 |
| 300 to 500 |  | 0.62 | 0.62 |
| 500 to 800 |  | 0.62 | 0.62 |
| 800 to 1000 |  | 1.21 | 1.21 |
| 1000 to 2000 |  | 1.21 | 1.21 |
| >2000 |  | 1.21 | 1.21 |
| LOM and oily debris | 43.76 | 7.71 | 7.71 |
|  |  | avg COV | 1.28 |

Figure 30. Anthracene particulate strength ratios by size range.

| WinSLAMM particle range (um) | Fluoranthene ratio with <63 (SERDP 2018) | Fluoranthene ratio with <200 (Bathi 2008) | avg Fluoranthene ratio with <200 |
| --- | --- | --- | --- |
| <1 | 1.60 |  | 1.60 |
| 1 to 2 | 1.60 |  | 1.60 |
| 2 to 5 | 1.60 |  | 1.60 |
| 5 to 10 | 0.25 |  | 0.25 |
| 10 to 20 | 0.25 |  | 0.25 |
| 20 to 40 | 1.15 |  | 1.15 |
| 40 to 50 | 1.15 | 1.21 | 1.18 |
| 50 to 60 | 1.15 | 1.15 | 1.15 |
| 60 to 80 |  | 1.15 | 1.15 |
| 80 to 100 |  | 1.15 | 1.15 |
| 100 to 150 |  | 0.64 | 0.64 |
| 150 to 200 |  | 0.64 | 0.64 |
| 200 to 300 |  | 0.37 | 0.37 |
| 300 to 500 |  | 0.46 | 0.46 |
| 500 to 800 |  | 0.46 | 0.46 |
| 800 to 1000 |  | 0.48 | 0.48 |
| 1000 to 2000 |  | 0.48 | 0.48 |
| >2000 |  | 0.39 | 0.39 |
| LOM and oily debris | 24.52 | 2.80 | 13.66 |
|  |  | avg COV | 1.78 |

Figure 31. Fluoranthene particulate strength ratios by size range.

| WinSLAMM particle range (um) | Pyrene ratio with <63 (SERDP 2018) | Pyrene ratio with <200 (Bathi 2008) | avg Pyrene ratio with <200 |
| --- | --- | --- | --- |
| <1 | 1.63 |  | 1.63 |
| 1 to 2 | 1.63 |  | 1.63 |
| 2 to 5 | 1.63 |  | 1.63 |
| 5 to 10 | 0.52 |  | 0.52 |
| 10 to 20 | 0.52 |  | 0.52 |
| 20 to 40 | 0.86 |  | 0.86 |
| 40 to 50 | 0.86 | 1.24 | 1.05 |
| 50 to 60 | 0.86 | 0.98 | 0.92 |
| 60 to 80 |  | 0.98 | 0.98 |
| 80 to 100 |  | 0.78 | 0.78 |
| 100 to 150 |  | 0.78 | 0.78 |
| 150 to 200 |  | 0.78 | 0.78 |
| 200 to 300 |  | 0.33 | 0.33 |
| 300 to 500 |  | 0.45 | 0.45 |
| 500 to 800 |  | 0.45 | 0.45 |
| 800 to 1000 |  | 0.39 | 0.39 |
| 1000 to 2000 |  | 0.39 | 0.39 |
| >2000 |  | 0.35 | 0.35 |
| LOM and oily debris | 26.06 | 3.89 | 14.97 |
|  |  | avg COV | 1.18 |

Figure 32. Pyrene particulate strength ratios by size range.

| WinSLAMM particle range (um) | Chrysene ratio with <63 (SERDP 2018) | Chrysene ratio with <200 (Bathi 2008) | avg Chrysene ratio with <200 |
| --- | --- | --- | --- |
| <1 | 1.37 |  | 1.37 |
| 1 to 2 | 1.37 |  | 1.37 |
| 2 to 5 | 1.37 |  | 1.37 |
| 5 to 10 | 0.53 |  | 0.53 |
| 10 to 20 | 0.53 |  | 0.53 |
| 20 to 40 | 1.10 |  | 1.10 |
| 40 to 50 | 1.10 | 1.14 | 1.12 |
| 50 to 60 | 1.10 | 1.16 | 1.13 |
| 60 to 80 |  | 1.16 | 1.16 |
| 80 to 100 |  | 0.70 | 0.70 |
| 100 to 150 |  | 0.70 | 0.70 |
| 150 to 200 |  | 0.70 | 0.70 |
| 200 to 300 |  | 0.39 | 0.39 |
| 300 to 500 |  | 0.53 | 0.53 |
| 500 to 800 |  | 0.53 | 0.53 |
| 800 to 1000 |  | 0.36 | 0.36 |
| 1000 to 2000 |  | 0.36 | 0.36 |
| >2000 |  | 0.45 | 0.45 |
| LOM and oily debris | 18.96 | 3.49 | 11.23 |
|  |  | avg COV | 1.12 |

Figure 33. Chrysene particulate strength ratios by size range.

| WinSLAMM particle range (um) | Benzo(a)anthracene ratio with <63 (SERDP 2018) | Benzo(a)anthracene ratio with <200 (Bathi 2008) | avg Benzo(a)anthracene ratio with <200 |
| --- | --- | --- | --- |
| <1 | 1.40 |  | 1.40 |
| 1 to 2 | 1.40 |  | 1.40 |
| 2 to 5 | 1.40 |  | 1.40 |
| 5 to 10 | 0.57 |  | 0.57 |
| 10 to 20 | 0.57 |  | 0.57 |
| 20 to 40 | 1.03 |  | 1.03 |
| 40 to 50 | 1.03 | 1.28 | 1.15 |
| 50 to 60 | 1.03 | 1.04 | 1.03 |
| 60 to 80 |  | 1.04 | 1.04 |
| 80 to 100 |  | 1.04 | 1.04 |
| 100 to 150 |  | 0.69 | 0.69 |
| 150 to 200 |  | 0.69 | 0.69 |
| 200 to 300 |  | 0.43 | 0.43 |
| 300 to 500 |  | 0.57 | 0.57 |
| 500 to 800 |  | 0.57 | 0.57 |
| 800 to 1000 |  | 0.42 | 0.42 |
| 1000 to 2000 |  | 0.42 | 0.42 |
| >2000 |  | 0.70 | 0.70 |
| LOM and oily debris | 17.95 | 5.51 | 11.73 |
|  |  | avg COV | 1.42 |

Figure 34. Benzo(a)anthracene particulate strength ratios by size range.

| WinSLAMM particle range (um) | Benzo(b)fluoranthene ratio with <63 (SERDP 2018) | Benzo(b)fluoranthene ratio with <200 (Bathi 2008) | avg Benzo(b)fluoranthene ratio with <200 |
| --- | --- | --- | --- |
| <1 | 1.40 |  | 1.40 |
| 1 to 2 | 1.40 |  | 1.40 |
| 2 to 5 | 1.40 |  | 1.40 |
| 5 to 10 | 0.48 |  | 0.48 |
| 10 to 20 | 0.48 |  | 0.48 |
| 20 to 40 | 1.13 |  | 1.13 |
| 40 to 50 | 1.13 | 1.22 | 1.17 |
| 50 to 60 | 1.13 | 1.05 | 1.09 |
| 60 to 80 |  | 1.05 | 1.05 |
| 80 to 100 |  | 1.05 | 1.05 |
| 100 to 150 |  | 0.73 | 0.73 |
| 150 to 200 |  | 0.73 | 0.73 |
| 200 to 300 |  | 0.82 | 0.82 |
| 300 to 500 |  | 0.46 | 0.46 |
| 500 to 800 |  | 0.46 | 0.46 |
| 800 to 1000 |  | 0.64 | 0.64 |
| 1000 to 2000 |  | 0.64 | 0.64 |
| >2000 |  | 0.72 | 0.72 |
| LOM and oily debris | 14.39 | 4.43 | 9.41 |
|  |  | avg COV | 1.45 |

Figure 35. Benzo(b)fluoranthene particulate strength ratios by size range.

| WinSLAMM particle range (um) | Benzo(k)fluoranthene ratio with <63 (SERDP 2018) |
| --- | --- |
| <1 | 1.37 |
| 1 to 2 | 1.37 |
| 2 to 5 | 1.37 |
| 5 to 10 | 0.49 |
| 10 to 20 | 0.49 |
| 20 to 40 | 1.14 |
| 40 to 50 | 1.14 |
| 50 to 60 | 1.14 |
| 60 to 80 |  |
| 80 to 100 |  |
| 100 to 150 |  |
| 150 to 200 |  |
| 200 to 300 |  |
| 300 to 500 |  |
| 500 to 800 |  |
| 800 to 1000 |  |
| 1000 to 2000 |  |
| >2000 |  |
| LOM and oily debris | 14.26 |
| avg COV | 1.57 |

Figure 36. Benzo(k)fluoranthene particulate strength ratios by size range.

| WinSLAMM particle range (um) | Benzo(a)pyrene ratio with <63 (SERDP 2018) | Benzo(a)pyrene ratio with <200 (Bathi 2008) | avg Benzo(a)pyrene ratio with <200 |
| --- | --- | --- | --- |
| <1 | 1.33 |  | 1.33 |
| 1 to 2 | 1.33 |  | 1.33 |
| 2 to 5 | 1.33 |  | 1.33 |
| 5 to 10 | 0.48 |  | 0.48 |
| 10 to 20 | 0.48 |  | 0.48 |
| 20 to 40 | 1.19 |  | 1.19 |
| 40 to 50 | 1.19 | 1.21 | 1.20 |
| 50 to 60 | 1.19 | 1.25 | 1.22 |
| 60 to 80 |  | 1.25 | 1.25 |
| 80 to 100 |  | 1.25 | 1.25 |
| 100 to 150 |  | 0.54 | 0.54 |
| 150 to 200 |  | 0.54 | 0.54 |
| 200 to 300 |  | 0.36 | 0.36 |
| 300 to 500 |  | 0.28 | 0.28 |
| 500 to 800 |  | 0.28 | 0.28 |
| 800 to 1000 |  | 0.39 | 0.39 |
| 1000 to 2000 |  | 0.39 | 0.39 |
| >2000 |  | 0.62 | 0.62 |
| LOM and oily debris | 11.94 | 1.91 | 6.93 |
|  |  | avg COV | 1.52 |

Figure 37. Benzo(a)pyrene particulate strength ratios by size range.

| WinSLAMM particle range (um) | Dibenzo(a,h)anthracene ratio with <63 (SERDP 2018) | Dibenzo(a,h)anthracene ratio with <200 (Bathi 2008) | avg Dibenzo(a,h)anthracene ratio with <200 |
| --- | --- | --- | --- |
| <1 | 0.94 |  | 0.94 |
| 1 to 2 | 0.94 |  | 0.94 |
| 2 to 5 | 0.94 |  | 0.94 |
| 5 to 10 | 0.55 |  | 0.55 |
| 10 to 20 | 0.55 |  | 0.55 |
| 20 to 40 | 1.51 |  | 1.51 |
| 40 to 50 | 1.51 | 1.39 | 1.45 |
| 50 to 60 | 1.51 | 0.87 | 1.19 |
| 60 to 80 |  | 0.87 | 0.87 |
| 80 to 100 |  | 0.87 | 0.87 |
| 100 to 150 |  | 0.74 | 0.74 |
| 150 to 200 |  | 0.74 | 0.74 |
| 200 to 300 |  | 0.30 | 0.30 |
| 300 to 500 |  | 0.39 | 0.39 |
| 500 to 800 |  | 0.39 | 0.39 |
| 800 to 1000 |  | 0.77 | 0.77 |
| 1000 to 2000 |  | 0.77 | 0.77 |
| >2000 |  | 0.62 | 0.62 |
| LOM and oily debris | 27.99 | 1.64 | 14.82 |
|  |  | avg COV | 1.48 |

Figure 38. Dibenzo(a,h)anthracene particulate strength ratios by size range.

| WinSLAMM particle range (um) | Benzo(ghi)perylene+Indeno(1,2,3-ed)pyrene ratio with <63 (SERDP 2018) | Benzo(ghi)perylene ratio with <200 (Bathi 2008) | avg Benzo(ghi)perylene ratio with <200 |
| --- | --- | --- | --- |
| <1 | 1.24 |  | 1.24 |
| 1 to 2 | 1.24 |  | 1.24 |
| 2 to 5 | 1.24 |  | 1.24 |
| 5 to 10 | 0.48 |  | 0.48 |
| 10 to 20 | 0.48 |  | 0.48 |
| 20 to 40 | 1.28 |  | 1.28 |
| 40 to 50 | 1.28 | 1.22 | 1.25 |
| 50 to 60 | 1.28 | 0.77 | 1.03 |
| 60 to 80 |  | 0.77 | 0.77 |
| 80 to 100 |  | 0.77 | 0.77 |
| 100 to 150 |  | 1.01 | 1.01 |
| 150 to 200 |  | 1.01 | 1.01 |
| 200 to 300 |  | 0.36 | 0.36 |
| 300 to 500 |  | 0.29 | 0.29 |
| 500 to 800 |  | 0.29 | 0.29 |
| 800 to 1000 |  | 0.94 | 0.94 |
| 1000 to 2000 |  | 0.94 | 0.94 |
| >2000 |  | 0.64 | 0.64 |
| LOM and oily debris | 5.26 | 3.86 | 4.56 |
|  |  | avg COV | 1.38 |

Figure 39. Benzo(ghi)perylene particulate strength ratios by size range.

## Chlordane and PCBs

Only the SERDP (2018) study included data for these two constituents. Limited detection did not allow these to be separated by land use.

| WinSLAMM particle range (um) | Chlordane ratio with <63 um (SERDP 2018) |
| --- | --- |
| <1 | 0.96 |
| 1 to 2 | 0.96 |
| 2 to 5 | 0.96 |
| 5 to 10 | 0.25 |
| 10 to 20 | 0.25 |
| 20 to 40 | 1.88 |
| 40 to 50 | 1.88 |
| 50 to 60 | 1.88 |
| 60 to 80 |  |
| 80 to 100 |  |
| 100 to 150 |  |
| 150 to 200 |  |
| 200 to 300 |  |
| 300 to 500 |  |
| 500 to 800 |  |
| 800 to 1000 |  |
| 1000 to 2000 |  |
| >2000 | 0.24 |
| avg COV | 1.72 |

Figure 40. Chlordane particulate strength ratios by size range.

| WinSLAMM particle range (um) | Total PCBs ratio with <63 um (SERDP 2018) |
| --- | --- |
| <1 | 1.79 |
| 1 to 2 | 1.79 |
| 2 to 5 | 1.79 |
| 5 to 10 | 0.39 |
| 10 to 20 | 0.39 |
| 20 to 40 | 0.82 |
| 40 to 50 | 0.82 |
| 50 to 60 | 0.82 |
| 60 to 80 |  |
| 80 to 100 |  |
| 100 to 150 |  |
| 150 to 200 |  |
| 200 to 300 |  |
| 300 to 500 |  |
| 500 to 800 |  |
| 800 to 1000 |  |
| 1000 to 2000 |  |
| >2000 | 0.15 |
| avg COV | 1.55 |

Figure 41. Total PCBs particulate strength ratios by size range.

# Recent SERDP Monitoring of Particulate Strengths of Metals, PAHs, and PFAS Compounds

The recent SERDP (2022) project monitored stormwater and treatment systems at several locations in the US, at Lubbock, TX, San Diego, CA, and Bremerton, WA. These data were obtained during monitoring by the Texas Tech research group and local cooperators during the SERDP Project: Development of Tools to Inform the Selection of Stormwater Controls at DoD Bases to Limit Potential Sediment Recontamination. Prior reports presented the data with descriptions of the sites and monitoring programs and data evaluations for each location, and a compilation of the performance data in a separate report. This white paper includes of particulate strengths of heavy metals, PAHs and PFAS compounds and outlines how these data can be used to modify the WinSLAMM modeling outcomes to consider the varying particulate strengths by particle size. The sites monitored during this project were:

- The drainage area for the monitored Picnic Lake at the Reese Technology Center in Lubbock, TX, was about 255 acres. This location is a repurposed Air Force base, and the current land use is comprised of mixed industrial and commercial uses. Two events were monitored at inlets to the lake and within the lake.

- Two locations were monitored at Naval Base San Diego, the Commissary Bioinfiltration System and the Federal Credit Union Bioswale. These both had drainage areas of about 0.38 acres of paved parking areas for the adjacent commercial areas. Three events were monitored at each of these two locations.

- The Naval Base Point Loma (adjacent to the Naval Base San Diego) site included monitoring of a media bed stormwater control. The drainage area was about 0.8 acres of paved parking and small roofs surrounding a light industrial area. Three rains were monitored at this location.

- Three locations were monitored at the Puget Sound Naval Base, two events were sampled at the Pier B (3.1 acres of naval pier with storage areas and small sheds) media filter, three events were sampled at the Recycled Metal Transfer Station (RMTS) (2.6 acres mostly paved storage and staging areas, with some roofs), and four events at the Metals Yard (2.5 acres paved storage yard with sheds and containers).

Particulate strengths were calculated for each sample and particle size using the previously described equation, based on the difference of the total and filtered concentrations (the particulate bound concentrations) and the total particulate solids (TSS in this case). Due to the few samples available, data from all the sites were combined to calculate the overall average particulate strengths by particle size. The treatment systems would not affect the particulate strengths by particle size, only the particle size distribution. Therefore, influent and effluent samples were also both combined for these calculations. The samples were collected at outfall locations of the drainage areas and do not represent separate source areas (except for the two NBSD small paved areas). Therefore, the factors describing the particulate strength by size can only be applied to the final calculated outfall conditions. Also, as reference, total particulate strength (mg/kg) for the influent total samples along with the filtered influent concentrations (µg/L for the metals and PAHs, and ng/L for the PFAS) were also calculated. The following sections summarize these data for heavy metals, PFAS congeners, and PAH compounds.

## Heavy Metals

The following table shows the overall summary of the filtered water concentration and particulate strength for the total sample portions, as used in WinSLAMM.

| Filtered inlet water concentrations (< 0.45 µm) µg/L |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Cr | Mn | Ni | Cu | Zn | As | Cd | Pb | Hg |
| site count | 7 | 7 | 7 | 7 | 7 | 7 | 6 | 7 | 4 |
| minimum | 0.29 | 1.05 | 1.17 | 3.5 | 1.15 | 0.44 | 0.06 | 0.26 | 0.001 |
| maximum | 2.88 | 138 | 9.76 | 265 | 313 | 3.58 | 1.17 | 2.37 | 0.004 |
| average | 1.44 | 32.7 | 5.20 | 70.5 | 161 | 1.96 | 0.47 | 1.12 | 0.003 |
| median | 1.23 | 12. 6 | 4.41 | 37.8 | 201 | 1.87 | 0.27 | 0.91 | 0.003 |
| standard deviation | 0.85 | 47.5 | 3.09 | 92.7 | 109 | 1.28 | 0.48 | 0.82 | 0.001 |
| COV | 0.59 | 1.45 | 0.59 | 1.31 | 0.68 | 0.66 | 1.02 | 0.74 | 0.50 |
| Total Particulates (> 0.45 µm) mg/kg |  |  |  |  |  |  |  |  |  |
| site count | 7 | 7 | 7 | 7 | 7 | 7 | 7 | 7 | 4 |
| minimum | 29.3 | 182.3 | 18.0 | 29.9 | 149 | 3.0 | 0.6 | 28.6 | 0.049 |
| maximum | 308 | 1,587 | 442 | 4,310 | 15,500 | 86.1 | 80.6 | 115 | 0.542 |
| average | 129 | 785 | 145 | 1,640 | 5,40 | 34.6 | 17.6 | 65.0 | 0.236 |
| median | 63.7 | 845 | 61.4 | 1,130 | 1,260 | 14.1 | 2.7 | 64.3 | 0.176 |
| standard deviation | 110 | 496 | 159 | 1,530 | 6,080 | 34.0 | 29.6 | 27.8 | 0.225 |
| COV | 0.85 | 0.63 | 1.09 | 0.93 | 1.13 | 0.98 | 1.68 | 0.43 | 0.95 |

The following figure illustrates how the particulate strengths for the four particle sizes vary for the monitored heavy metals. All of the metals had higher relative particulate strengths for the largest sample fractions (>63 µm).

The following table lists the relative particulate strengths by the particle sizes used in WinSLAMM. The data for the four size ranges shown in the figure above were used to fill in this longer table.

| WinSLAMM particle size range (µm) | Cr | Mn | Ni | Cu | Zn | As | Cd | Pb | Hg |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 to 1.9 | 0.68 | 0.73 | 1.11 | 0.61 | 1.56 | 1.05 | 1.23 | 0.69 | 0.66 |
| 2 to 2.9 | 0.68 | 0.73 | 1.11 | 0.61 | 1.56 | 1.05 | 1.23 | 0.69 | 0.66 |
| 3 to 3.9 | 0.68 | 0.73 | 1.11 | 0.61 | 1.56 | 1.05 | 1.23 | 0.69 | 0.66 |
| 4 to 4.9 | 0.68 | 0.73 | 1.11 | 0.61 | 1.56 | 1.05 | 1.23 | 0.69 | 0.66 |
| 5 to 5.9 | 0.66 | 0.69 | 1.20 | 1.16 | 0.47 | 0.71 | 0.75 | 1.09 | 1.16 |
| 6 to 6.9 | 0.66 | 0.69 | 1.20 | 1.16 | 0.47 | 0.71 | 0.75 | 1.09 | 1.16 |
| 7 to 7.9 | 0.66 | 0.69 | 1.20 | 1.16 | 0.47 | 0.71 | 0.75 | 1.09 | 1.16 |
| 8 to 8.9 | 0.66 | 0.69 | 1.20 | 1.16 | 0.47 | 0.71 | 0.75 | 1.09 | 1.16 |
| 9 to 9.9 | 0.66 | 0.69 | 1.20 | 1.16 | 0.47 | 0.71 | 0.75 | 1.09 | 1.16 |
| 10 to 10.9 | 0.66 | 0.69 | 1.20 | 1.16 | 0.47 | 0.71 | 0.75 | 1.09 | 1.16 |
| 11 to 11.9 | 0.66 | 0.69 | 1.20 | 1.16 | 0.47 | 0.71 | 0.75 | 1.09 | 1.16 |
| 12 to 12.9 | 0.66 | 0.69 | 1.20 | 1.16 | 0.47 | 0.71 | 0.75 | 1.09 | 1.16 |
| 13 to 13.9 | 0.66 | 0.69 | 1.20 | 1.16 | 0.47 | 0.71 | 0.75 | 1.09 | 1.16 |
| 14 to 14.9 | 0.66 | 0.69 | 1.20 | 1.16 | 0.47 | 0.71 | 0.75 | 1.09 | 1.16 |
| 15 to 19.9 | 0.66 | 0.69 | 1.20 | 1.16 | 0.47 | 0.71 | 0.75 | 1.09 | 1.16 |
| 20 to 24.9 | 0.73 | 0.75 | 0.52 | 0.70 | 0.56 | 1.00 | 0.79 | 0.75 | 0.61 |
| 25 to 29.9 | 0.73 | 0.75 | 0.52 | 0.70 | 0.56 | 1.00 | 0.79 | 0.75 | 0.61 |
| 30 to 34.9 | 0.73 | 0.75 | 0.52 | 0.70 | 0.56 | 1.00 | 0.79 | 0.75 | 0.61 |
| 35 to 39.9 | 0.73 | 0.75 | 0.52 | 0.70 | 0.56 | 1.00 | 0.79 | 0.75 | 0.61 |
| 40 to 49.9 | 0.73 | 0.75 | 0.52 | 0.70 | 0.56 | 1.00 | 0.79 | 0.75 | 0.61 |
| 50 to 59.9 | 0.73 | 0.75 | 0.52 | 0.70 | 0.56 | 1.00 | 0.79 | 0.75 | 0.61 |
| 60 to 79.9 | 1.92 | 1.79 | 1.18 | 1.47 | 1.57 | 1.28 | 1.26 | 1.43 | 1.57 |
| 80 to 99.9 | 1.92 | 1.79 | 1.18 | 1.47 | 1.57 | 1.28 | 1.26 | 1.43 | 1.57 |
| 100 to 149.9 | 1.92 | 1.79 | 1.18 | 1.47 | 1.57 | 1.28 | 1.26 | 1.43 | 1.57 |
| 150 to 199.9 | 1.92 | 1.79 | 1.18 | 1.47 | 1.57 | 1.28 | 1.26 | 1.43 | 1.57 |
| 200 to 299.9 | 1.92 | 1.79 | 1.18 | 1.47 | 1.57 | 1.28 | 1.26 | 1.43 | 1.57 |
| 300 to 499.9 | 1.92 | 1.79 | 1.18 | 1.47 | 1.57 | 1.28 | 1.26 | 1.43 | 1.57 |
| 500 to 799.9 | 1.92 | 1.79 | 1.18 | 1.47 | 1.57 | 1.28 | 1.26 | 1.43 | 1.57 |
| 800 to 999.9 | 1.92 | 1.79 | 1.18 | 1.47 | 1.57 | 1.28 | 1.26 | 1.43 | 1.57 |
| 1000 to 1999.9 | 1.92 | 1.79 | 1.18 | 1.47 | 1.57 | 1.28 | 1.26 | 1.43 | 1.57 |
| >1999.9 | 1.92 | 1.79 | 1.18 | 1.47 | 1.57 | 1.28 | 1.26 | 1.43 | 1.57 |

## PAHs

The tables and figures for PAHs are organized in a similar manner as for the heavy metals, except that the larger number of constituents required the tables and figures to be separated to fit the page width.

|  | Naphthalene | 2-methylnaphthalene | 1-methylnaphthalene | 2-ethylnaphthalene | 1-ethylnaphthalene | 2.6-dimethylnaphthalene | 1.3-dimethylnaphthalene | 2-isopropylnaphthalene | acenaphthylene | 1.2-dimethylnaphthalene |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| inlet filtered conc (µg/L) |  |  |  |  |  |  |  |  |  |  |
| site count | 6 | 6 | 5 | 4 | 3 | 5 | 5 | 2 | 5 | 3 |
| minimum | 1.86 | 1.30 | 0.555 | 0.340 | 0.087 | 0.423 | 0.456 | 0.036 | 0.065 | 0.127 |
| maximum | 8.98 | 6.43 | 3.26 | 0.881 | 0.147 | 2.07 | 1.65 | 0.055 | 1.48 | 0.456 |
| average | 5.55 | 3.92 | 2.02 | 0.664 | 0.126 | 1.22 | 1.09 | 0.046 | 0.644 | 0.289 |
| median | 5.62 | 4.12 | 2.25 | 0.717 | 0.142 | 1.30 | 1.13 | 0.046 | 0.161 | 0.283 |
| standard deviation | 2.59 | 2.25 | 1.29 | 0.259 | 0.033 | 0.679 | 0.486 | 0.013 | 0.713 | 0.165 |
| COV | 0.47 | 0.57 | 0.64 | 0.39 | 0.26 | 0.55 | 0.45 | 0.29 | 1.11 | 0.57 |
| total part strength (mg/kg) |  |  |  |  |  |  |  |  |  |  |
| site count | 7 | 7 | 5 | 5 | 4 | 7 | 6 | 2 | 6 | 5 |
| minimum | 0.014 | 0.006 | 0.003 | 0.002 | 0.001 | 0.004 | 0.003 | 0.000 | 0.001 | 0.000 |
| maximum | 1.100 | 0.482 | 0.144 | 0.080 | 0.002 | 0.146 | 0.088 | 0.001 | 0.091 | 0.020 |
| average | 0.311 | 0.170 | 0.067 | 0.022 | 0.002 | 0.057 | 0.041 | 0.001 | 0.021 | 0.009 |
| median | 0.084 | 0.110 | 0.042 | 0.009 | 0.002 | 0.050 | 0.036 | 0.001 | 0.008 | 0.009 |
| standard deviation | 0.400 | 0.180 | 0.066 | 0.033 | 0.001 | 0.050 | 0.035 | 0.001 | 0.035 | 0.008 |
| COV | 1.29 | 1.06 | 0.99 | 1.54 | 0.32 | 0.87 | 0.85 | 1.41 | 1.69 | 0.92 |

|  | 1.8-dimethylnaphthalene | acenaphthene | 2.3.5-trimethylnaphthalene | fluorene | 1-methylfluorene | phenanthrene | anthracene | 2-methylphenanthrene | 2-methylanthracene | 1-methylphenanthrene |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| inlet filtered conc (µg/L) |  |  |  |  |  |  |  |  |  |  |
| site count | 2 | 3 | 3 | 6 | 4 | 4 | 5 | 6 | 5 | 4 |
| minimum | 0.041 | 0.239 | 0.648 | 1.01 | 0.439 | 8.76 | 0.243 | 0.592 | 0.045 | 0.324 |
| maximum | 0.390 | 1.263 | 1.367 | 5.19 | 2.26 | 25.2 | 14.0 | 5.48 | 1.71 | 2.35 |
| average | 0.216 | 0.722 | 0.970 | 2.96 | 1.22 | 13.7 | 3.21 | 2.09 | 0.557 | 1.19 |
| median | 0.216 | 0.666 | 0.895 | 2.50 | 1.09 | 10.4 | 0.503 | 1.71 | 0.150 | 1.05 |
| standard deviation | 0.246 | 0.514 | 0.365 | 1.81 | 0.816 | 7.70 | 6.05 | 1.75 | 0.702 | 0.853 |
| COV | 1.14 | 0.71 | 0.38 | 0.61 | 0.67 | 0.56 | 1.89 | 0.84 | 1.26 | 0.71 |
| total part strength (mg/kg) |  |  |  |  |  |  |  |  |  |  |
| site count | 3 | 5 | 4 | 7 | 5 | 5 | 6 | 7 | 6 | 5 |
| minimum | 0.000 | 0.000 | 0.001 | 0.013 | 0.004 | 0.336 | 0.015 | 0.041 | 0.005 | 0.027 |
| maximum | 0.006 | 0.070 | 0.026 | 0.221 | 0.031 | 0.817 | 0.910 | 0.154 | 0.131 | 0.080 |
| average | 0.003 | 0.022 | 0.014 | 0.081 | 0.014 | 0.485 | 0.176 | 0.102 | 0.043 | 0.049 |
| median | 0.002 | 0.015 | 0.014 | 0.066 | 0.013 | 0.449 | 0.031 | 0.085 | 0.008 | 0.037 |
| standard deviation | 0.003 | 0.027 | 0.013 | 0.070 | 0.011 | 0.192 | 0.360 | 0.042 | 0.056 | 0.024 |
| COV | 1.27 | 1.23 | 0.92 | 0.86 | 0.77 | 0.40 | 2.05 | 0.41 | 1.32 | 0.50 |

|  | 9-methylanthracene | 2-ethylanthracene | fluoranthene | pyrene | 9.10-dimethylanthracene | 2-tertbutylanthracene | 1-methylpyrene | benzo(a)anthracene | chrysene | benzo(b)fluoranthene |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| inlet filtered conc (µg/L) |  |  |  |  |  |  |  |  |  |  |
| site count | 3 | 4 | 6 | 6 | 3 | 3 | 4 | 6 | 6 | 6 |
| minimum | 0.026 | 0.085 | 1.563 | 1.18 | 0.117 | 0.040 | 0.006 | 0.046 | 0.382 | 0.058 |
| maximum | 0.064 | 1.14 | 12.9 | 26.9 | 2.74 | 0.189 | 0.214 | 0.419 | 2.81 | 1.07 |
| average | 0.044 | 0.595 | 5.25 | 9.10 | 0.995 | 0.103 | 0.118 | 0.226 | 1.36 | 0.427 |
| median | 0.042 | 0.573 | 3.58 | 3.41 | 0.128 | 0.079 | 0.125 | 0.207 | 1.40 | 0.271 |
| standard deviation | 0.019 | 0.438 | 4.39 | 10.8 | 1.51 | 0.077 | 0.109 | 0.155 | 0.870 | 0.382 |
| COV | 0.43 | 0.74 | 0.84 | 1.20 | 1.52 | 0.75 | 0.93 | 0.69 | 0.64 | 0.89 |
| total part strength (mg/kg) |  |  |  |  |  |  |  |  |  |  |
| site count | 5 | 5 | 7 | 7 | 2 | 3 | 5 | 7 | 7 | 7 |
| minimum | 0.001 | 0.013 | 0.282 | 0.150 | 0.000 | 0.001 | 0.028 | 0.049 | 0.241 | 0.160 |
| maximum | 0.002 | 0.060 | 1.553 | 1.413 | 0.028 | 0.002 | 0.061 | 0.343 | 1.079 | 1.072 |
| average | 0.001 | 0.028 | 0.789 | 0.720 | 0.014 | 0.001 | 0.035 | 0.135 | 0.565 | 0.349 |
| median | 0.002 | 0.020 | 0.733 | 0.636 | 0.014 | 0.001 | 0.028 | 0.096 | 0.502 | 0.224 |
| standard deviation | 0.001 | 0.018 | 0.413 | 0.418 | 0.019 | 0.001 | 0.014 | 0.097 | 0.302 | 0.327 |
| COV | 0.46 | 0.66 | 0.52 | 0.58 | 1.37 | 0.49 | 0.41 | 0.72 | 0.53 | 0.94 |

|  | 7.12-methylbenz(a)anthracene | benzo(k)fluoranthene | benzo(e)pyrene | benzo(a)pyrene | perylene | Indeno(123-cd)pyrene | Dibenzo(ah)anthracene | benzo(ghi)perylene | total PAHs |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| inlet filtered conc (µg/L) |  |  |  |  |  |  |  |  |  |
| site count | 4 | 5 | 5 | 3 | 4 | 5 | 4 | 5 | 6 |
| minimum | 0.000 | 0.034 | 0.008 | 0.103 | 0.085 | 0.119 | 0.043 | 0.053 | 12.6 |
| maximum | 0.063 | 0.664 | 0.207 | 0.633 | 0.148 | 0.540 | 0.388 | 0.408 | 93.3 |
| average | 0.042 | 0.312 | 0.131 | 0.412 | 0.110 | 0.256 | 0.200 | 0.182 | 52.8 |
| median | 0.053 | 0.192 | 0.162 | 0.499 | 0.102 | 0.205 | 0.184 | 0.147 | 54.9 |
| standard deviation | 0.030 | 0.262 | 0.079 | 0.276 | 0.027 | 0.164 | 0.145 | 0.147 | 28.0 |
| COV | 0.70 | 0.84 | 0.60 | 0.67 | 0.25 | 0.64 | 0.73 | 0.81 | 0.53 |
| total part strength (mg/kg) |  |  |  |  |  |  |  |  |  |
| site count | 5 | 6 | 6 | 4 | 5 | 6 | 5 | 6 | 7 |
| minimum | 0.003 | 0.075 | 0.005 | 0.106 | 0.005 | 0.032 | 0.014 | 0.057 | 2.52 |
| maximum | 0.031 | 0.861 | 0.496 | 0.478 | 0.151 | 5.630 | 0.342 | 0.584 | 21.5 |
| average | 0.011 | 0.288 | 0.212 | 0.265 | 0.041 | 1.025 | 0.095 | 0.272 | 6.62 |
| median | 0.006 | 0.139 | 0.206 | 0.238 | 0.014 | 0.116 | 0.034 | 0.251 | 4.00 |
| standard deviation | 0.011 | 0.301 | 0.161 | 0.167 | 0.062 | 2.256 | 0.139 | 0.178 | 6.70 |
| COV | 1.04 | 1.04 | 0.76 | 0.63 | 1.52 | 2.20 | 1.47 | 0.66 | 1.01 |

| WinSLAMM particle size range (um) | Naphthalene | 2-methylnaphthalene | 1-methylnaphthalene | 2-ethylnaphthalene | 2.6-dimethylnaphthalene | 1.3-dimethylnaphthalene | acenaphthylene | 1.2-dimethylnaphthalene | acenaphthene | fluorene |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 to 1.9 | 1.58 | 1.92 | 2.04 | 3.93 | 1.09 | 1.19 | 2.49 | 2.68 | 1.89 | 0.68 |
| 2 to 2.9 | 1.58 | 1.92 | 2.04 | 3.93 | 1.09 | 1.19 | 2.49 | 2.68 | 1.89 | 0.68 |
| 3 to 3.9 | 1.58 | 1.92 | 2.04 | 3.93 | 1.09 | 1.19 | 2.49 | 2.68 | 1.89 | 0.68 |
| 4 to 4.9 | 1.58 | 1.92 | 2.04 | 3.93 | 1.09 | 1.19 | 2.49 | 2.68 | 1.89 | 0.68 |
| 5 to 5.9 | 0.43 | 0.40 | 0.47 | 0.35 | 0.33 | 0.59 | 0.62 | 0.12 | 0.15 | 0.94 |
| 6 to 6.9 | 0.43 | 0.40 | 0.47 | 0.35 | 0.33 | 0.59 | 0.62 | 0.12 | 0.15 | 0.94 |
| 7 to 7.9 | 0.43 | 0.40 | 0.47 | 0.35 | 0.33 | 0.59 | 0.62 | 0.12 | 0.15 | 0.94 |
| 8 to 8.9 | 0.43 | 0.40 | 0.47 | 0.35 | 0.33 | 0.59 | 0.62 | 0.12 | 0.15 | 0.94 |
| 9 to 9.9 | 0.43 | 0.40 | 0.47 | 0.35 | 0.33 | 0.59 | 0.62 | 0.12 | 0.15 | 0.94 |
| 10 to 10.9 | 0.43 | 0.40 | 0.47 | 0.35 | 0.33 | 0.59 | 0.62 | 0.12 | 0.15 | 0.94 |
| 11 to 11.9 | 0.43 | 0.40 | 0.47 | 0.35 | 0.33 | 0.59 | 0.62 | 0.12 | 0.15 | 0.94 |
| 12 to 12.9 | 0.43 | 0.40 | 0.47 | 0.35 | 0.33 | 0.59 | 0.62 | 0.12 | 0.15 | 0.94 |
| 13 to 13.9 | 0.43 | 0.40 | 0.47 | 0.35 | 0.33 | 0.59 | 0.62 | 0.12 | 0.15 | 0.94 |
| 14 to 14.9 | 0.43 | 0.40 | 0.47 | 0.35 | 0.33 | 0.59 | 0.62 | 0.12 | 0.15 | 0.94 |
| 15 to 19.9 | 0.43 | 0.40 | 0.47 | 0.35 | 0.33 | 0.59 | 0.62 | 0.12 | 0.15 | 0.94 |
| 20 to 24.9 | 1.14 | 0.41 | 0.50 | 0.46 | 0.96 | 0.82 | 0.62 | 0.38 | 0.43 | 0.69 |
| 25 to 29.9 | 1.14 | 0.41 | 0.50 | 0.46 | 0.96 | 0.82 | 0.62 | 0.38 | 0.43 | 0.69 |
| 30 to 34.9 | 1.14 | 0.41 | 0.50 | 0.46 | 0.96 | 0.82 | 0.62 | 0.38 | 0.43 | 0.69 |
| 35 to 39.9 | 1.14 | 0.41 | 0.50 | 0.46 | 0.96 | 0.82 | 0.62 | 0.38 | 0.43 | 0.69 |
| 40 to 49.9 | 1.14 | 0.41 | 0.50 | 0.46 | 0.96 | 0.82 | 0.62 | 0.38 | 0.43 | 0.69 |
| 50 to 59.9 | 1.14 | 0.41 | 0.50 | 0.46 | 0.96 | 0.82 | 0.62 | 0.38 | 0.43 | 0.69 |
| 60 to 79.9 | 0.95 | 1.37 | 1.30 | 1.80 | 1.63 | 1.56 | 1.14 | 2.44 | 1.57 | 1.97 |
| 80 to 99.9 | 0.95 | 1.37 | 1.30 | 1.80 | 1.63 | 1.56 | 1.14 | 2.44 | 1.57 | 1.97 |
| 100 to 149.9 | 0.95 | 1.37 | 1.30 | 1.80 | 1.63 | 1.56 | 1.14 | 2.44 | 1.57 | 1.97 |
| 150 to 199.9 | 0.95 | 1.37 | 1.30 | 1.80 | 1.63 | 1.56 | 1.14 | 2.44 | 1.57 | 1.97 |
| 200 to 299.9 | 0.95 | 1.37 | 1.30 | 1.80 | 1.63 | 1.56 | 1.14 | 2.44 | 1.57 | 1.97 |
| 300 to 499.9 | 0.95 | 1.37 | 1.30 | 1.80 | 1.63 | 1.56 | 1.14 | 2.44 | 1.57 | 1.97 |
| 500 to 799.9 | 0.95 | 1.37 | 1.30 | 1.80 | 1.63 | 1.56 | 1.14 | 2.44 | 1.57 | 1.97 |
| 800 to 999.9 | 0.95 | 1.37 | 1.30 | 1.80 | 1.63 | 1.56 | 1.14 | 2.44 | 1.57 | 1.97 |
| 1000 to 1999.9 | 0.95 | 1.37 | 1.30 | 1.80 | 1.63 | 1.56 | 1.14 | 2.44 | 1.57 | 1.97 |
| >1999.9 | 0.95 | 1.37 | 1.30 | 1.80 | 1.63 | 1.56 | 1.14 | 2.44 | 1.57 | 1.97 |

| WinSLAMM particle size range (um) | 1-methylfluorene | phenanthrene | anthracene | 2-methylphenanthrene | 2-methylanthracene | 1-methylphenanthrene | 9-methylanthracene | 2-ethylanthracene | fluoranthene | pyrene |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 to 1.9 | 1.58 | 0.17 | 1.60 | 0.59 | 2.27 | 1.70 | 3.22 | 2.19 | 1.77 | 1.64 |
| 2 to 2.9 | 1.58 | 0.17 | 1.60 | 0.59 | 2.27 | 1.70 | 3.22 | 2.19 | 1.77 | 1.64 |
| 3 to 3.9 | 1.58 | 0.17 | 1.60 | 0.59 | 2.27 | 1.70 | 3.22 | 2.19 | 1.77 | 1.64 |
| 4 to 4.9 | 1.58 | 0.17 | 1.60 | 0.59 | 2.27 | 1.70 | 3.22 | 2.19 | 1.77 | 1.64 |
| 5 to 5.9 | 0.66 | 0.64 | 0.76 | 1.21 | 0.92 | 0.99 | 0.70 | 0.82 | 0.39 | 0.48 |
| 6 to 6.9 | 0.66 | 0.64 | 0.76 | 1.21 | 0.92 | 0.99 | 0.70 | 0.82 | 0.39 | 0.48 |
| 7 to 7.9 | 0.66 | 0.64 | 0.76 | 1.21 | 0.92 | 0.99 | 0.70 | 0.82 | 0.39 | 0.48 |
| 8 to 8.9 | 0.66 | 0.64 | 0.76 | 1.21 | 0.92 | 0.99 | 0.70 | 0.82 | 0.39 | 0.48 |
| 9 to 9.9 | 0.66 | 0.64 | 0.76 | 1.21 | 0.92 | 0.99 | 0.70 | 0.82 | 0.39 | 0.48 |
| 10 to 10.9 | 0.66 | 0.64 | 0.76 | 1.21 | 0.92 | 0.99 | 0.70 | 0.82 | 0.39 | 0.48 |
| 11 to 11.9 | 0.66 | 0.64 | 0.76 | 1.21 | 0.92 | 0.99 | 0.70 | 0.82 | 0.39 | 0.48 |
| 12 to 12.9 | 0.66 | 0.64 | 0.76 | 1.21 | 0.92 | 0.99 | 0.70 | 0.82 | 0.39 | 0.48 |
| 13 to 13.9 | 0.66 | 0.64 | 0.76 | 1.21 | 0.92 | 0.99 | 0.70 | 0.82 | 0.39 | 0.48 |
| 14 to 14.9 | 0.66 | 0.64 | 0.76 | 1.21 | 0.92 | 0.99 | 0.70 | 0.82 | 0.39 | 0.48 |
| 15 to 19.9 | 0.66 | 0.64 | 0.76 | 1.21 | 0.92 | 0.99 | 0.70 | 0.82 | 0.39 | 0.48 |
| 20 to 24.9 | 0.05 | 0.87 | 0.64 | 0.71 | 0.66 | 0.40 | 0.61 | 0.50 | 0.80 | 0.43 |
| 25 to 29.9 | 0.05 | 0.87 | 0.64 | 0.71 | 0.66 | 0.40 | 0.61 | 0.50 | 0.80 | 0.43 |
| 30 to 34.9 | 0.05 | 0.87 | 0.64 | 0.71 | 0.66 | 0.40 | 0.61 | 0.50 | 0.80 | 0.43 |
| 35 to 39.9 | 0.05 | 0.87 | 0.64 | 0.71 | 0.66 | 0.40 | 0.61 | 0.50 | 0.80 | 0.43 |
| 40 to 49.9 | 0.05 | 0.87 | 0.64 | 0.71 | 0.66 | 0.40 | 0.61 | 0.50 | 0.80 | 0.43 |
| 50 to 59.9 | 0.05 | 0.87 | 0.64 | 0.71 | 0.66 | 0.40 | 0.61 | 0.50 | 0.80 | 0.43 |
| 60 to 79.9 | 2.05 | 1.94 | 1.32 | 1.67 | 0.50 | 1.76 | 0.58 | 1.54 | 1.06 | 1.44 |
| 80 to 99.9 | 2.05 | 1.94 | 1.32 | 1.67 | 0.50 | 1.76 | 0.58 | 1.54 | 1.06 | 1.44 |
| 100 to 149.9 | 2.05 | 1.94 | 1.32 | 1.67 | 0.50 | 1.76 | 0.58 | 1.54 | 1.06 | 1.44 |
| 150 to 199.9 | 2.05 | 1.94 | 1.32 | 1.67 | 0.50 | 1.76 | 0.58 | 1.54 | 1.06 | 1.44 |
| 200 to 299.9 | 2.05 | 1.94 | 1.32 | 1.67 | 0.50 | 1.76 | 0.58 | 1.54 | 1.06 | 1.44 |
| 300 to 499.9 | 2.05 | 1.94 | 1.32 | 1.67 | 0.50 | 1.76 | 0.58 | 1.54 | 1.06 | 1.44 |
| 500 to 799.9 | 2.05 | 1.94 | 1.32 | 1.67 | 0.50 | 1.76 | 0.58 | 1.54 | 1.06 | 1.44 |
| 800 to 999.9 | 2.05 | 1.94 | 1.32 | 1.67 | 0.50 | 1.76 | 0.58 | 1.54 | 1.06 | 1.44 |
| 1000 to 1999.9 | 2.05 | 1.94 | 1.32 | 1.67 | 0.50 | 1.76 | 0.58 | 1.54 | 1.06 | 1.44 |
| >1999.9 | 2.05 | 1.94 | 1.32 | 1.67 | 0.50 | 1.76 | 0.58 | 1.54 | 1.06 | 1.44 |

| WinSLAMM particle size range (um) | 1-methylpyrene | benz(a)anthracene | chrysene | benzo(b)fluoranthene | 7.12-methylbenz(a)anthracene | benzo(k)fluoranthene | benzo(e)pyrene | perylene | Indeno(123-cd)pyrene | Dibenzo(ah)anthracene | benzo(ghi)perylene | total PAHs |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 to 1.9 | 1.04 | 1.64 | 1.30 | 1.46 | 1.33 | 1.70 | 1.38 | 1.80 | 1.69 | 1.65 | 0.99 | 1.66 |
| 2 to 2.9 | 1.04 | 1.64 | 1.30 | 1.46 | 1.33 | 1.70 | 1.38 | 1.80 | 1.69 | 1.65 | 0.99 | 1.66 |
| 3 to 3.9 | 1.04 | 1.64 | 1.30 | 1.46 | 1.33 | 1.70 | 1.38 | 1.80 | 1.69 | 1.65 | 0.99 | 1.66 |
| 4 to 4.9 | 1.04 | 1.64 | 1.30 | 1.46 | 1.33 | 1.70 | 1.38 | 1.80 | 1.69 | 1.65 | 0.99 | 1.66 |
| 5 to 5.9 | 0.93 | 1.00 | 0.56 | 0.57 | 0.53 | 0.53 | 0.95 | 0.18 | 0.48 | 0.42 | 0.82 | 0.34 |
| 6 to 6.9 | 0.93 | 1.00 | 0.56 | 0.57 | 0.53 | 0.53 | 0.95 | 0.18 | 0.48 | 0.42 | 0.82 | 0.34 |
| 7 to 7.9 | 0.93 | 1.00 | 0.56 | 0.57 | 0.53 | 0.53 | 0.95 | 0.18 | 0.48 | 0.42 | 0.82 | 0.34 |
| 8 to 8.9 | 0.93 | 1.00 | 0.56 | 0.57 | 0.53 | 0.53 | 0.95 | 0.18 | 0.48 | 0.42 | 0.82 | 0.34 |
| 9 to 9.9 | 0.93 | 1.00 | 0.56 | 0.57 | 0.53 | 0.53 | 0.95 | 0.18 | 0.48 | 0.42 | 0.82 | 0.34 |
| 10 to 10.9 | 0.93 | 1.00 | 0.56 | 0.57 | 0.53 | 0.53 | 0.95 | 0.18 | 0.48 | 0.42 | 0.82 | 0.34 |
| 11 to 11.9 | 0.93 | 1.00 | 0.56 | 0.57 | 0.53 | 0.53 | 0.95 | 0.18 | 0.48 | 0.42 | 0.82 | 0.34 |
| 12 to 12.9 | 0.93 | 1.00 | 0.56 | 0.57 | 0.53 | 0.53 | 0.95 | 0.18 | 0.48 | 0.42 | 0.82 | 0.34 |
| 13 to 13.9 | 0.93 | 1.00 | 0.56 | 0.57 | 0.53 | 0.53 | 0.95 | 0.18 | 0.48 | 0.42 | 0.82 | 0.34 |
| 14 to 14.9 | 0.93 | 1.00 | 0.56 | 0.57 | 0.53 | 0.53 | 0.95 | 0.18 | 0.48 | 0.42 | 0.82 | 0.34 |
| 15 to 19.9 | 0.93 | 1.00 | 0.56 | 0.57 | 0.53 | 0.53 | 0.95 | 0.18 | 0.48 | 0.42 | 0.82 | 0.34 |
| 20 to 24.9 | 1.03 | 0.58 | 1.03 | 0.96 | 1.57 | 0.99 | 1.16 | 0.68 | 1.38 | 1.16 | 1.72 | 0.59 |
| 25 to 29.9 | 1.03 | 0.58 | 1.03 | 0.96 | 1.57 | 0.99 | 1.16 | 0.68 | 1.38 | 1.16 | 1.72 | 0.59 |
| 30 to 34.9 | 1.03 | 0.58 | 1.03 | 0.96 | 1.57 | 0.99 | 1.16 | 0.68 | 1.38 | 1.16 | 1.72 | 0.59 |
| 35 to 39.9 | 1.03 | 0.58 | 1.03 | 0.96 | 1.57 | 0.99 | 1.16 | 0.68 | 1.38 | 1.16 | 1.72 | 0.59 |
| 40 to 49.9 | 1.03 | 0.58 | 1.03 | 0.96 | 1.57 | 0.99 | 1.16 | 0.68 | 1.38 | 1.16 | 1.72 | 0.59 |
| 50 to 59.9 | 1.03 | 0.58 | 1.03 | 0.96 | 1.57 | 0.99 | 1.16 | 0.68 | 1.38 | 1.16 | 1.72 | 0.59 |
| 60 to 79.9 | 1.00 | 0.82 | 1.05 | 0.93 | 0.91 | 0.92 | 0.70 | 1.38 | 0.58 | 0.87 | 0.63 | 1.41 |
| 80 to 99.9 | 1.00 | 0.82 | 1.05 | 0.93 | 0.91 | 0.92 | 0.70 | 1.38 | 0.58 | 0.87 | 0.63 | 1.41 |
| 100 to 149.9 | 1.00 | 0.82 | 1.05 | 0.93 | 0.91 | 0.92 | 0.70 | 1.38 | 0.58 | 0.87 | 0.63 | 1.41 |
| 150 to 199.9 | 1.00 | 0.82 | 1.05 | 0.93 | 0.91 | 0.92 | 0.70 | 1.38 | 0.58 | 0.87 | 0.63 | 1.41 |
| 200 to 299.9 | 1.00 | 0.82 | 1.05 | 0.93 | 0.91 | 0.92 | 0.70 | 1.38 | 0.58 | 0.87 | 0.63 | 1.41 |
| 300 to 499.9 | 1.00 | 0.82 | 1.05 | 0.93 | 0.91 | 0.92 | 0.70 | 1.38 | 0.58 | 0.87 | 0.63 | 1.41 |
| 500 to 799.9 | 1.00 | 0.82 | 1.05 | 0.93 | 0.91 | 0.92 | 0.70 | 1.38 | 0.58 | 0.87 | 0.63 | 1.41 |
| 800 to 999.9 | 1.00 | 0.82 | 1.05 | 0.93 | 0.91 | 0.92 | 0.70 | 1.38 | 0.58 | 0.87 | 0.63 | 1.41 |
| 1000 to 1999.9 | 1.00 | 0.82 | 1.05 | 0.93 | 0.91 | 0.92 | 0.70 | 1.38 | 0.58 | 0.87 | 0.63 | 1.41 |
| >1999.9 | 1.00 | 0.82 | 1.05 | 0.93 | 0.91 | 0.92 | 0.70 | 1.38 | 0.58 | 0.87 | 0.63 | 1.41 |

## PFAS

No data for PFAS congeners were available for different particle sizes due to their very low concentrations. The following tables therefore only show the filtered concentrations and the total sample particulate strengths.

| average inlet <0.7µm (filtered) ng/L |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PFPeA | PFHxA | PFHpA | PFOA | PFDA | PFUdA | PFNA | PFOS | 6:2 FTS |
| site count | 3 | 5 | 3 | 5 | 3 | 2 | 4 | 5 | 3 |
| minimum | 0.7 | 2.2 | 0.2 | 2.0 | 0.8 | 0.1 | 0.2 | 3.9 | 1.7 |
| maximum | 11.8 | 53.6 | 4.8 | 29.0 | 2.5 | 0.9 | 2.5 | 85.8 | 28.8 |
| average | 5.3 | 15.3 | 2.1 | 13.4 | 1.3 | 0.5 | 1.0 | 29.4 | 18.2 |
| median | 3.3 | 7.1 | 1.2 | 16.5 | 0.8 | 0.5 | 0.6 | 24.1 | 24.1 |
| standard deviation | 5.8 | 21.7 | 2.4 | 11.4 | 1.0 | 0.6 | 1.1 | 33.0 | 14.5 |
| COV | 1.10 | 1.42 | 1.16 | 0.85 | 0.72 | 1.09 | 1.09 | 1.12 | 0.80 |
| average PFAS particulate strengths (mg/kg) |  |  |  |  |  |  |  |  |  |
|  | PFPeA | PFHxA | PFHpA | PFOA | PFDA | PFUdA | PFNA | PFOS | 6:2 FTS |
| site count | 3 | 4 | 3 | 4 | 4 | 3 | 4 | 4 | 2 |
| minimum | 0.037 | 0.011 | 0.002 | 0.021 | 0.011 | 0.001 | 0.004 | 0.076 | 0.031 |
| maximum | 0.109 | 0.148 | 0.081 | 0.536 | 0.087 | 0.051 | 0.081 | 1.729 | 0.094 |
| average | 0.065 | 0.086 | 0.031 | 0.189 | 0.046 | 0.021 | 0.028 | 0.702 | 0.063 |
| median | 0.050 | 0.092 | 0.010 | 0.100 | 0.044 | 0.012 | 0.012 | 0.502 | 0.063 |
| standard deviation | 0.038 | 0.056 | 0.043 | 0.236 | 0.038 | 0.026 | 0.036 | 0.758 | 0.044 |
| COV | 0.59 | 0.66 | 1.39 | 1.24 | 0.81 | 1.23 | 1.32 | 1.08 | 0.71 |

## Example WinSLAMM Calculations

The purpose of this white paper is to summarize data that can be used to calculate stormwater characterization and stormwater control performance as a function of particle size. WinSLAMM tracks particle sizes from source areas and through the drainage system and stormwater controls. However, it currently uses a single value for the particulate strengths corresponding to the whole sample fraction. The particle size particulate strength data can be used to adjust the effluent concentrations based on the particle size distributions, as illustrated in this section of the white paper.

The following table summarizes the particulate strength factors for the different particle sizes modeled in WinSLAMM for the “standard” pollutants listed in the model, along with examples for “other” pollutants that can be described and modeled.

The last two tables are examples of how the calculation adjustments can be accomplished in the model. Currently, a data output option can be selected that shows the fraction of particulates associated with the different particle sizes for each modeled rain and for different locations in the modeled system. The particulate strengths by particle sizes are mostly associated with outfall samples at mixed land use areas. Therefore, the correction factors should be applied to the flow-weighted particle size distribution information associated with the outfall location. These two examples show how the calculations can be used for an example for a location with stormwater treatment having very few larger particles. The second example is for an untreated stormwater having a wide range of stormwater particulates. In these examples, the correction factors and the resulting concentrations vary by about 5 to 20 percent compared to the original concentrations that are not corrected by these factors.

Particulate Strength Factors for Particle Sizes for Standard WinSLAMM Particulate Pollutants (and examples for “others’)

|  |  |  |  |  |  |  |  |  |  | example "others" |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| bin# | % > size | range | Phosphorus | TKN | COD | Cr | Cu | Pb | Zn | Cd (1) | pyrene (2) | naphthalene (3) | fluorene (4) | phenanthrene (5) | anthracene (6) |
| 1 | 0.45 | 0.45 to 1.9 | 1.68 | 1.00* | 1.20 | 0.68 | 0.61 | 0.69 | 1.56 | 1.23 | 1.64 | 1.58 | 0.68 | 0.17 | 1.60 |
| 2 | 2 | 2 to 2.9 | 0.71 | 1.00 | 0.66 | 0.68 | 0.61 | 0.69 | 1.56 | 1.23 | 1.64 | 1.58 | 0.68 | 0.17 | 1.60 |
| 3 | 3 | 3 to 3.9 | 0.71 | 1.00 | 0.66 | 0.68 | 0.61 | 0.69 | 1.56 | 1.23 | 1.64 | 1.58 | 0.68 | 0.17 | 1.60 |
| 4 | 4 | 4 to 4.9 | 0.71 | 1.00 | 0.66 | 0.68 | 0.61 | 0.69 | 1.56 | 1.23 | 1.64 | 1.58 | 0.68 | 0.17 | 1.60 |
| 5 | 5 | 5 to 5.9 | 0.71 | 1.00 | 0.66 | 0.66 | 1.16 | 1.09 | 0.47 | 0.75 | 0.48 | 0.43 | 0.94 | 0.64 | 0.76 |
| 6 | 6 | 6 to 6.9 | 0.71 | 1.00 | 0.66 | 0.66 | 1.16 | 1.09 | 0.47 | 0.75 | 0.48 | 0.43 | 0.94 | 0.64 | 0.76 |
| 7 | 7 | 7 to 7.9 | 0.71 | 1.00 | 0.66 | 0.66 | 1.16 | 1.09 | 0.47 | 0.75 | 0.48 | 0.43 | 0.94 | 0.64 | 0.76 |
| 8 | 8 | 8 to 8.9 | 0.71 | 1.00 | 0.66 | 0.66 | 1.16 | 1.09 | 0.47 | 0.75 | 0.48 | 0.43 | 0.94 | 0.64 | 0.76 |
| 9 | 9 | 9 to 9.9 | 0.71 | 1.00 | 0.66 | 0.66 | 1.16 | 1.09 | 0.47 | 0.75 | 0.48 | 0.43 | 0.94 | 0.64 | 0.76 |
| 10 | 10 | 10 to 10.9 | 0.81 | 1.00 | 0.67 | 0.66 | 1.16 | 1.09 | 0.47 | 0.75 | 0.48 | 0.43 | 0.94 | 0.64 | 0.76 |
| 11 | 11 | 11 to 11.9 | 0.81 | 1.00 | 0.67 | 0.66 | 1.16 | 1.09 | 0.47 | 0.75 | 0.48 | 0.43 | 0.94 | 0.64 | 0.76 |
| 12 | 12 | 12 to 12.9 | 0.81 | 1.00 | 0.67 | 0.66 | 1.16 | 1.09 | 0.47 | 0.75 | 0.48 | 0.43 | 0.94 | 0.64 | 0.76 |
| 13 | 13 | 13 to 13.9 | 0.81 | 1.00 | 0.67 | 0.66 | 1.16 | 1.09 | 0.47 | 0.75 | 0.48 | 0.43 | 0.94 | 0.64 | 0.76 |
| 14 | 14 | 14 to 14.9 | 0.81 | 1.00 | 0.67 | 0.66 | 1.16 | 1.09 | 0.47 | 0.75 | 0.48 | 0.43 | 0.94 | 0.64 | 0.76 |
| 15 | 15 | 15 to 19.9 | 0.81 | 1.00 | 0.67 | 0.66 | 1.16 | 1.09 | 0.47 | 0.75 | 0.48 | 0.43 | 0.94 | 0.64 | 0.76 |
| 16 | 20 | 20 to 24.9 | 0.81 | 1.00 | 0.67 | 0.73 | 0.70 | 0.75 | 0.56 | 0.79 | 0.43 | 1.14 | 0.69 | 0.87 | 0.64 |
| 17 | 25 | 25 to 29.9 | 0.81 | 1.00 | 0.67 | 0.73 | 0.70 | 0.75 | 0.56 | 0.79 | 0.43 | 1.14 | 0.69 | 0.87 | 0.64 |
| 18 | 30 | 30 to 34.9 | 0.81 | 1.00 | 0.67 | 0.73 | 0.70 | 0.75 | 0.56 | 0.79 | 0.43 | 1.14 | 0.69 | 0.87 | 0.64 |
| 19 | 35 | 35 to 39.9 | 0.81 | 1.00 | 0.67 | 0.73 | 0.70 | 0.75 | 0.56 | 0.79 | 0.43 | 1.14 | 0.69 | 0.87 | 0.64 |
| 20 | 40 | 40 to 49.9 | 0.78 | 1.29 | 1.15 | 0.73 | 0.70 | 0.75 | 0.56 | 0.79 | 0.43 | 1.14 | 0.69 | 0.87 | 0.64 |
| 21 | 50 | 50 to 59.9 | 1.00 | 1.07 | 1.09 | 0.73 | 0.70 | 0.75 | 0.56 | 0.79 | 0.43 | 1.14 | 0.69 | 0.87 | 0.64 |
| 22 | 60 | 60 to 79.9 | 1.16 | 0.89 | 1.20 | 1.92 | 1.47 | 1.43 | 1.57 | 1.26 | 1.44 | 0.95 | 1.97 | 1.94 | 1.32 |
| 23 | 80 | 80 to 99.9 | 0.71 | 0.93 | 0.90 | 1.92 | 1.47 | 1.43 | 1.57 | 1.26 | 1.44 | 0.95 | 1.97 | 1.94 | 1.32 |
| 24 | 100 | 100 to 149.9 | 0.68 | 0.93 | 0.96 | 1.92 | 1.47 | 1.43 | 1.57 | 1.26 | 1.44 | 0.95 | 1.97 | 1.94 | 1.32 |
| 25 | 150 | 150 to 199.9 | 0.53 | 0.79 | 0.54 | 1.92 | 1.47 | 1.43 | 1.57 | 1.26 | 1.44 | 0.95 | 1.97 | 1.94 | 1.32 |
| 26 | 200 | 200 to 299.9 | 0.52 | 0.49 | 0.66 | 1.92 | 1.47 | 1.43 | 1.57 | 1.26 | 1.44 | 0.95 | 1.97 | 1.94 | 1.32 |
| 27 | 300 | 300 to 499.9 | 0.70 | 0.56 | 0.68 | 1.92 | 1.47 | 1.43 | 1.57 | 1.26 | 1.44 | 0.95 | 1.97 | 1.94 | 1.32 |
| 28 | 500 | 500 to 799.9 | 0.70 | 0.56 | 0.70 | 1.92 | 1.47 | 1.43 | 1.57 | 1.26 | 1.44 | 0.95 | 1.97 | 1.94 | 1.32 |
| 29 | 800 | 800 to 999.9 | 0.69 | 0.83 | 0.86 | 1.92 | 1.47 | 1.43 | 1.57 | 1.26 | 1.44 | 0.95 | 1.97 | 1.94 | 1.32 |
| 30 | 1000 | 1000 to 1999.9 | 0.88 | 1.23 | 1.15 | 1.92 | 1.47 | 1.43 | 1.57 | 1.26 | 1.44 | 0.95 | 1.97 | 1.94 | 1.32 |
| 31 | 2000 | >1999.9 | 0.90 | 1.18 | 1.43 | 1.92 | 1.47 | 1.43 | 1.57 | 1.26 | 1.44 | 0.95 | 1.97 | 1.94 | 1.32 |
|  |  | source: | avg | avg | avg | SERDP 2022 | SERDP 2022 | SERDP 2022 | SERDP 2022 | SERDP 2022 | SERDP 2022 | SERDP 2022 | SERDP 2022 | SERDP 2022 | SERDP 2022 |

*note: use 1.00 if unknown or missing

Calculation Example 1 (treated effluent with few large particles)

| Part. Size No. | Particle Size (microns) | Inf. % > Particle Size | Eff. % > Particle Size (4) | size range | Outfall particulates % in size range (6) | P PSD fraction, from above Table (7) | weighted P conc fraction (8) = (6)x(7)/100 | Cu PSD fraction | weighted Cu conc fraction | Zn PSD fraction | weighted Zn conc fraction | Pb PSD fraction | weighted Pb conc fraction | pyrene (2) PSD fraction | weighted pyrene conc fraction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 100.000 | 100.000 | 0 to 1 | 14.075 | 1.68 | 0.236 | 0.61 | 0.086 | 1.56 | 0.220 | 0.686 | 0.097 | 1.64 | 0.230 |
| 1 | 1 | 85.925 | 85.925 | 1 to 1.9 | 25.335 | 1.68 | 0.426 | 0.61 | 0.155 | 1.56 | 0.395 | 0.686 | 0.174 | 1.64 | 0.414 |
| 2 | 2 | 60.590 | 60.590 | 2 to 2.9 | 11.260 | 0.71 | 0.080 | 0.61 | 0.069 | 1.56 | 0.175 | 0.686 | 0.077 | 1.64 | 0.184 |
| 3 | 3 | 49.330 | 49.330 | 3 to 3.9 | 5.630 | 0.71 | 0.040 | 0.61 | 0.035 | 1.56 | 0.088 | 0.686 | 0.039 | 1.64 | 0.092 |
| 4 | 4 | 43.700 | 43.700 | 4 to 4.9 | 5.630 | 0.71 | 0.040 | 0.61 | 0.035 | 1.56 | 0.088 | 0.686 | 0.039 | 1.64 | 0.092 |
| 5 | 5 | 38.070 | 38.070 | 5 to 5.9 | 5.630 | 0.71 | 0.040 | 1.16 | 0.066 | 0.47 | 0.026 | 1.092 | 0.061 | 0.48 | 0.027 |
| 6 | 6 | 32.440 | 32.440 | 6 to 6.9 | 5.630 | 0.71 | 0.040 | 1.16 | 0.066 | 0.47 | 0.026 | 1.092 | 0.061 | 0.48 | 0.027 |
| 7 | 7 | 26.810 | 26.810 | 7 to 7.9 | 2.815 | 0.71 | 0.020 | 1.16 | 0.033 | 0.47 | 0.013 | 1.092 | 0.031 | 0.48 | 0.013 |
| 8 | 8 | 23.995 | 23.995 | 8 to 8.9 | 5.630 | 0.71 | 0.040 | 1.16 | 0.066 | 0.47 | 0.026 | 1.092 | 0.061 | 0.48 | 0.027 |
| 9 | 9 | 18.366 | 18.366 | 9 to 9.9 | 2.815 | 0.71 | 0.020 | 1.16 | 0.033 | 0.47 | 0.013 | 1.092 | 0.031 | 0.48 | 0.013 |
| 10 | 10 | 15.551 | 15.551 | 10 to 10.9 | 2.815 | 0.81 | 0.023 | 1.16 | 0.033 | 0.47 | 0.013 | 1.092 | 0.031 | 0.48 | 0.013 |
| 11 | 11 | 12.736 | 12.736 | 11 to 11.9 | 2.815 | 0.81 | 0.023 | 1.16 | 0.033 | 0.47 | 0.013 | 1.092 | 0.031 | 0.48 | 0.013 |
| 12 | 12 | 9.921 | 9.921 | 12 to 12.9 | 2.815 | 0.81 | 0.023 | 1.16 | 0.033 | 0.47 | 0.013 | 1.092 | 0.031 | 0.48 | 0.013 |
| 13 | 13 | 7.106 | 7.106 | 13 to 13.9 | 2.815 | 0.81 | 0.023 | 1.16 | 0.033 | 0.47 | 0.013 | 1.092 | 0.031 | 0.48 | 0.013 |
| 14 | 14 | 4.291 | 4.291 | 14 to 14.9 | 2.815 | 0.81 | 0.023 | 1.16 | 0.033 | 0.47 | 0.013 | 1.092 | 0.031 | 0.48 | 0.013 |
| 15 | 15 | 1.476 | 1.476 | 15 to 19.9 | 1.476 | 0.81 | 0.012 | 1.16 | 0.017 | 0.47 | 0.007 | 1.092 | 0.016 | 0.48 | 0.007 |
| 16 | 20 | 0.000 | 0.000 | 20 to 24.9 | 0.000 | 0.81 | 0.000 | 0.70 | 0.000 | 0.56 | 0.000 | 0.747 | 0.000 | 0.43 | 0.000 |
| 17 | 25 | 0.000 | 0.000 | 25 to 29.9 | 0.000 | 0.81 | 0.000 | 0.70 | 0.000 | 0.56 | 0.000 | 0.747 | 0.000 | 0.43 | 0.000 |
| 18 | 30 | 0.000 | 0.000 | 30 to 34.9 | 0.000 | 0.81 | 0.000 | 0.70 | 0.000 | 0.56 | 0.000 | 0.747 | 0.000 | 0.43 | 0.000 |
| 19 | 35 | 0.000 | 0.000 | 35 to 39.9 | 0.000 | 0.81 | 0.000 | 0.70 | 0.000 | 0.56 | 0.000 | 0.747 | 0.000 | 0.43 | 0.000 |
| 20 | 40 | 0.000 | 0.000 | 40 to 49.9 | 0.000 | 0.78 | 0.000 | 0.70 | 0.000 | 0.56 | 0.000 | 0.747 | 0.000 | 0.43 | 0.000 |
| 21 | 50 | 0.000 | 0.000 | 50 to 59.9 | 0.000 | 1 | 0.000 | 0.70 | 0.000 | 0.56 | 0.000 | 0.747 | 0.000 | 0.43 | 0.000 |
| 22 | 60 | 0.000 | 0.000 | 60 to 79.9 | 0.000 | 1.16 | 0.000 | 1.47 | 0.000 | 1.57 | 0.000 | 1.430 | 0.000 | 1.44 | 0.000 |
| 23 | 80 | 0.000 | 0.000 | 80 to 99.9 | 0.000 | 0.71 | 0.000 | 1.47 | 0.000 | 1.57 | 0.000 | 1.430 | 0.000 | 1.44 | 0.000 |
| 24 | 100 | 0.000 | 0.000 | 100 to 149.9 | 0.000 | 0.68 | 0.000 | 1.47 | 0.000 | 1.57 | 0.000 | 1.430 | 0.000 | 1.44 | 0.000 |
| 25 | 150 | 0.000 | 0.000 | 150 to 199.9 | 0.000 | 0.53 | 0.000 | 1.47 | 0.000 | 1.57 | 0.000 | 1.430 | 0.000 | 1.44 | 0.000 |
| 26 | 200 | 0.000 | 0.000 | 200 to 299.9 | 0.000 | 0.52 | 0.000 | 1.47 | 0.000 | 1.57 | 0.000 | 1.430 | 0.000 | 1.44 | 0.000 |
| 27 | 300 | 0.000 | 0.000 | 300 to 499.9 | 0.000 | 0.7 | 0.000 | 1.47 | 0.000 | 1.57 | 0.000 | 1.430 | 0.000 | 1.44 | 0.000 |
| 28 | 500 | 0.000 | 0.000 | 500 to 799.9 | 0.000 | 0.7 | 0.000 | 1.47 | 0.000 | 1.57 | 0.000 | 1.430 | 0.000 | 1.44 | 0.000 |
| 29 | 800 | 0.000 | 0.000 | 800 to 999.9 | 0.000 | 0.69 | 0.000 | 1.47 | 0.000 | 1.57 | 0.000 | 1.430 | 0.000 | 1.44 | 0.000 |
| 30 | 1000 | 0.000 | 0.000 | 1000 to 1999.9 | 0.000 | 0.88 | 0.000 | 1.47 | 0.000 | 1.57 | 0.000 | 1.430 | 0.000 | 1.44 | 0.000 |
| 31 | 2000 | 0.000 | 0.000 | >1999.9 | 0.000 | 0.9 | 0.000 | 1.47 | 0.000 | 1.57 | 0.000 | 1.430 | 0.000 | 1.44 | 0.000 |
| (a) correction factor (sum of weighted concentration fractions) |  |  |  |  |  |  | 1.108 |  | 0.823 |  | 1.144 |  | 0.840 |  | 1.194 |
| (b) basic outfall particulate bound concentration, from model output |  |  |  |  |  |  | 0.087 |  | 3.7 |  | 93.8 |  | 23.8 |  | 3.1 |
| (c) corrected particulate bound concentration with factor (mg/L for P; ug/L for others) = (a)x(b) |  |  |  |  |  |  | 0.096 |  | 3.0 |  | 107.3 |  | 20.0 |  | 3.7 |
| (d) outfall filtered concentration (mg/L for P; ug/L for others), from model output |  |  |  |  |  |  | 0.263 |  | 48.8 |  | 113.4 |  | 2.1 |  | 0.06 |
| (e) Total corrected outfall concentration (mg/L for P; ug/L for others) = (c)+(d) |  |  |  |  |  |  | 0.359 |  | 51.8 |  | 220.7 |  | 22.1 |  | 3.8 |
|  | original total concentration (mg/L for P; ug/L for others), for reference only from model ouput (b)+(d) |  |  |  |  |  | 0.350 |  | 52.5 |  | 207.2 |  | 25.9 |  | 3.2 |

Calculation Example 2 (wide range of particles in untreated outfall)

| Part. Size No. | Particle Size (microns) | Inf. % > Particle Size | Eff. % > Particle Size | size range | Outfall particulates % in size range | P PSD fraction | weighted P conc fraction | Cu PSD fraction | weighted Cu conc fraction | Zn PSD fraction | weighted Zn conc fraction | Pb PSD fraction | weighted Pb conc fraction | pyrene (2) PSD fraction | weighted pyrene conc fraction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 100.000 | 100.000 | 0 to 1 | 11 | 1.68 | 0.185 | 0.61 | 0.067 | 1.56 | 0.172 | 0.686 | 0.075 | 1.64 | 0.180 |
| 1 | 1 | 85.925 | 85.925 | 1 to 1.9 | 2 | 1.68 | 0.034 | 0.61 | 0.012 | 1.56 | 0.031 | 0.686 | 0.014 | 1.64 | 0.033 |
| 2 | 2 | 60.590 | 60.590 | 2 to 2.9 | 4 | 0.71 | 0.028 | 0.61 | 0.025 | 1.56 | 0.062 | 0.686 | 0.027 | 1.64 | 0.065 |
| 3 | 3 | 49.330 | 49.330 | 3 to 3.9 | 4 | 0.71 | 0.028 | 0.61 | 0.025 | 1.56 | 0.062 | 0.686 | 0.027 | 1.64 | 0.065 |
| 4 | 4 | 43.700 | 43.700 | 4 to 4.9 | 7 | 0.71 | 0.050 | 0.61 | 0.043 | 1.56 | 0.109 | 0.686 | 0.048 | 1.64 | 0.115 |
| 5 | 5 | 38.070 | 38.070 | 5 to 5.9 | 3 | 0.71 | 0.021 | 1.16 | 0.035 | 0.47 | 0.014 | 1.092 | 0.033 | 0.48 | 0.014 |
| 6 | 6 | 32.440 | 32.440 | 6 to 6.9 | 4 | 0.71 | 0.028 | 1.16 | 0.047 | 0.47 | 0.019 | 1.092 | 0.044 | 0.48 | 0.019 |
| 7 | 7 | 26.810 | 26.810 | 7 to 7.9 | 4 | 0.71 | 0.028 | 1.16 | 0.047 | 0.47 | 0.019 | 1.092 | 0.044 | 0.48 | 0.019 |
| 8 | 8 | 23.995 | 23.995 | 8 to 8.9 | 3 | 0.71 | 0.021 | 1.16 | 0.035 | 0.47 | 0.014 | 1.092 | 0.033 | 0.48 | 0.014 |
| 9 | 9 | 18.366 | 18.366 | 9 to 9.9 | 1 | 0.71 | 0.007 | 1.16 | 0.012 | 0.47 | 0.005 | 1.092 | 0.011 | 0.48 | 0.005 |
| 10 | 10 | 15.551 | 15.551 | 10 to 10.9 | 1 | 0.81 | 0.008 | 1.16 | 0.012 | 0.47 | 0.005 | 1.092 | 0.011 | 0.48 | 0.005 |
| 11 | 11 | 12.736 | 12.736 | 11 to 11.9 | 1 | 0.81 | 0.008 | 1.16 | 0.012 | 0.47 | 0.005 | 1.092 | 0.011 | 0.48 | 0.005 |
| 12 | 12 | 9.921 | 9.921 | 12 to 12.9 | 1 | 0.81 | 0.008 | 1.16 | 0.012 | 0.47 | 0.005 | 1.092 | 0.011 | 0.48 | 0.005 |
| 13 | 13 | 7.106 | 7.106 | 13 to 13.9 | 2 | 0.81 | 0.016 | 1.16 | 0.023 | 0.47 | 0.009 | 1.092 | 0.022 | 0.48 | 0.010 |
| 14 | 14 | 4.291 | 4.291 | 14 to 14.9 | 2 | 0.81 | 0.016 | 1.16 | 0.023 | 0.47 | 0.009 | 1.092 | 0.022 | 0.48 | 0.010 |
| 15 | 15 | 1.476 | 1.476 | 15 to 19.9 | 13 | 0.81 | 0.105 | 1.16 | 0.151 | 0.47 | 0.061 | 1.092 | 0.142 | 0.48 | 0.062 |
| 16 | 20 | 0.000 | 0.000 | 20 to 24.9 | 2 | 0.81 | 0.016 | 0.70 | 0.014 | 0.56 | 0.011 | 0.747 | 0.015 | 0.43 | 0.009 |
| 17 | 25 | 0.000 | 0.000 | 25 to 29.9 | 5 | 0.81 | 0.041 | 0.70 | 0.035 | 0.56 | 0.028 | 0.747 | 0.037 | 0.43 | 0.022 |
| 18 | 30 | 0.000 | 0.000 | 30 to 34.9 | 2 | 0.81 | 0.016 | 0.70 | 0.014 | 0.56 | 0.011 | 0.747 | 0.015 | 0.43 | 0.009 |
| 19 | 35 | 0.000 | 0.000 | 35 to 39.9 | 3 | 0.81 | 0.024 | 0.70 | 0.021 | 0.56 | 0.017 | 0.747 | 0.022 | 0.43 | 0.013 |
| 20 | 40 | 0.000 | 0.000 | 40 to 49.9 | 3 | 0.78 | 0.023 | 0.70 | 0.021 | 0.56 | 0.017 | 0.747 | 0.022 | 0.43 | 0.013 |
| 21 | 50 | 0.000 | 0.000 | 50 to 59.9 | 2 | 1 | 0.020 | 0.70 | 0.014 | 0.56 | 0.011 | 0.747 | 0.015 | 0.43 | 0.009 |
| 22 | 60 | 0.000 | 0.000 | 60 to 79.9 | 2 | 1.16 | 0.023 | 1.47 | 0.029 | 1.57 | 0.031 | 1.430 | 0.029 | 1.44 | 0.029 |
| 23 | 80 | 0.000 | 0.000 | 80 to 99.9 | 3 | 0.71 | 0.021 | 1.47 | 0.044 | 1.57 | 0.047 | 1.430 | 0.043 | 1.44 | 0.043 |
| 24 | 100 | 0.000 | 0.000 | 100 to 149.9 | 3 | 0.68 | 0.020 | 1.47 | 0.044 | 1.57 | 0.047 | 1.430 | 0.043 | 1.44 | 0.043 |
| 25 | 150 | 0.000 | 0.000 | 150 to 199.9 | 2 | 0.53 | 0.011 | 1.47 | 0.029 | 1.57 | 0.031 | 1.430 | 0.029 | 1.44 | 0.029 |
| 26 | 200 | 0.000 | 0.000 | 200 to 299.9 | 2 | 0.52 | 0.010 | 1.47 | 0.029 | 1.57 | 0.031 | 1.430 | 0.029 | 1.44 | 0.029 |
| 27 | 300 | 0.000 | 0.000 | 300 to 499.9 | 2 | 0.7 | 0.014 | 1.47 | 0.029 | 1.57 | 0.031 | 1.430 | 0.029 | 1.44 | 0.029 |
| 28 | 500 | 0.000 | 0.000 | 500 to 799.9 | 1 | 0.7 | 0.007 | 1.47 | 0.015 | 1.57 | 0.016 | 1.430 | 0.014 | 1.44 | 0.014 |
| 29 | 800 | 0.000 | 0.000 | 800 to 999.9 | 3 | 0.69 | 0.021 | 1.47 | 0.044 | 1.57 | 0.047 | 1.430 | 0.043 | 1.44 | 0.043 |
| 30 | 1000 | 0.000 | 0.000 | 1000 to 1999.9 | 2 | 0.88 | 0.018 | 1.47 | 0.029 | 1.57 | 0.031 | 1.430 | 0.029 | 1.44 | 0.029 |
| 31 | 2000 | 0.000 | 0.000 | >1999.9 | 0 | 0.9 | 0.000 | 1.47 | 0.000 | 1.57 | 0.000 | 1.430 | 0.000 | 1.44 | 0.000 |
| correction factor (sum of weighted concentration fractions) |  |  |  |  |  |  | 0.879 |  | 0.991 |  | 1.010 |  | 0.987 |  | 0.986 |
| basic outfall particulate bound concentration |  |  |  |  |  |  | 0.087 |  | 3.7 |  | 93.8 |  | 23.8 |  | 3.1 |
| corrected particulate bound concentration with factor (mg/L for P; ug/L for others) |  |  |  |  |  |  | 0.076 |  | 3.7 |  | 94.8 |  | 23.5 |  | 3.1 |
| outfall filtered concentration (mg/L for P; ug/L for others) |  |  |  |  |  |  | 0.263 |  | 48.8 |  | 113.4 |  | 2.1 |  | 0.06 |
| Total corrected outfall concentration (mg/L for P; ug/L for others) |  |  |  |  |  |  | 0.339 |  | 52.5 |  | 208.2 |  | 25.6 |  | 3.12 |
|  | original total concentration (mg/L for P; ug/L for others) |  |  |  |  |  | 0.350 |  | 52.5 |  | 207.2 |  | 25.9 |  | 3.16 |

# Conclusions

Knowing the distribution of pollutants associated with different sized stormwater particles allows more accurate determinations of their sources, transport, and control. This white paper data is being used to expand the WinSLAMM model by expanding the particulate strength characteristics by particle size. WinSLAMM currently tracks particulates based on particle size distributions but uses a single particulate strength value for all sizes. Stormwater controls usually preferentially remove the largest particles, resulting in increased bulk particle strength values after treatment. Therefore, using particulate strength values for different particle sizes will result in more accurate calculations assisting stormwater management decisions.

Pollutants in stormwater runoff can be separated into particulate-bound or filtered (filterable or “dissolved”) forms, as reflected in the pollutant concentration parameter file. Pollutant strengths are the contaminant concentrations associated with the particulate matter in the stormwater and vary by particle size. The following are brief comments concerning these characteristics of stormwater particulates from the previously summarized data.

## Specific Gravity and Volatile Solids

In most cases, stormwater particulates have specific gravities in the range of 1.5 to 2.5, depending on the mixture of organic and inert material in the particle. This corresponds to a relatively narrow range of settling rates for a specific particle size. Newton (laminar) and Stoke (turbulent) settling equations are used to calculate settling rates for these particulates. Specific gravity decreases as the organic content increases; larger particle sizes generally have lower specific gravity and greater organic material. With sedimentation treatment, preferential removal of higher specific gravity materials results in a shift to lower overall specific gravity of particulates in the effluent water for similar size classes. Scour of deposited sediment is dependent on the particle characteristics, including compaction of the material, compared to the shear stress (tractive force) of the flowing water.

## COD and Nutrients

In residential and commercial areas, the smaller stormwater particles have the greatest concentrations of COD and nutrients (due to large surface areas per mass with pollutant sorption onto surfaces). Large particles have somewhat greater pollutant strengths for nutrients as they contain more organic material than other particle sizes. At industrial sites, larger particles may be of metallic (and/or oily) material instead of soil and organic material, with less of an increase in nutrients for the larger particles, but greater COD and TOC content if oily material.

At residential and commercial areas, removal of all particulates down to about 10 µm in the stormwater would remove almost all of the particulate associated phosphorus. However, the filterable phosphorus concentrations can still be high, requiring additional treatment targeting these filterable forms to reduce the phosphorus levels to typically acceptable levels. Similar results were found for industrial stormwater: removal of all particulates down to about 10 or 20 µm would remove almost all of the particulate bound COD. The filterable COD may still be substantial, indicating large amounts of dissolved organic material.

## Heavy Metals

In almost all cases, the heavy metals are mostly associated with particulates in stormwater, except for Zn which can have large portions associated with the filterable fraction. Residential and commercial areas may have greater filterable fractions of metals in the stormwater than industrial sites that may have greater fractions associated with particulates. The normal trend is increases in pollutant strengths of metals with decreases in particle size. However, at industrial sites, large metallic portions may be associated with larger particles sizes due to the nature of the source material. At a monitored industrial area, the majority of the pollutant concentrations (and mass) were associated with the 10 to 100 µm particle size range.

Residential and commercial stormwater filterable heavy metals (zinc, cadmium, and lead) are mostly bound to organic complexes or as colloids that are difficult to remove by ion exchange but may be removed by sorption processes. Copper was mostly in ionic forms that can be readily removed by ion exchange (depending on the ionic strength).

Pre-treatment sedimentation controls removing only the largest particles (>100 µm for example) would only result in small reductions of the resulting effluent water concentrations. More effective treatment controls that can remove smaller particles would result in much better effluent quality (down to about 5 µm, beyond which little additional benefit is likely by sedimentation processes).

## PAHs

Because of their low volatility (low Henry’s Law constant), high octonal-water partition coefficients (KOW) and high soil organic coefficients (KOC), many of the stormwater PAHs are preferentially adsorbed to particulate matter, especially organics. The smaller and larger particles can have relatively higher PAH particulate strength values compared to the intermediate sized particles, depending on the organic content of the material. PAHs can be effectively controlled concurrent with high levels of stormwater particulate control.

## Overall Particulate Strength Summary Tables

Table 59 are summary tables showing the particulate strength ratios for the constituents examined. There are some missing values. The closest values should be selected on the WinSLAMM factor tables for the particle size ranges.

Table 59. Overall Table of Particulate Strength Ratios

| WinSLAMM particle range (um) | volatile solids (Cai 2013) | specific gravity (g/cm3) (Cai 2013) | average COD ratio | avg TP ratio with <250 um | TN ratio with <250 um | resid and commer avg TOC ratio with <63 (SERDP 2018) | indus avg TOC ratio with <63 (SERDP 2018) | Chlordane ratio with <63 um (SERDP 2018) | Total PCBs ratio with <63 um (SERDP 2018) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| <1 |  |  | 1.30 | 1.91 |  | 2.20 | 0.95 | 0.96 | 1.79 |
| 1 to 2 |  |  | 1.20 | 1.68 |  | 2.20 | 0.95 | 0.96 | 1.79 |
| 2 to 5 |  |  | 0.66 | 0.71 |  | 2.20 | 0.95 | 0.96 | 1.79 |
| 5 to 10 |  |  | 0.66 | 0.71 |  | 0.29 | 0.63 | 0.25 | 0.39 |
| 10 to 20 |  |  | 0.67 | 0.81 |  | 0.29 | 0.63 | 0.25 | 0.39 |
| 20 to 40 |  |  | 0.67 | 0.81 |  | 0.52 | 1.42 | 1.88 | 0.82 |
| 40 to 50 | 1.08 | 3.46 | 1.15 | 0.78 | 1.29 | 0.52 | 1.42 | 1.88 | 0.82 |
| 50 to 60 | 1.07 | 3.30 | 1.09 | 1.00 | 1.07 | 0.52 | 1.42 | 1.88 | 0.82 |
| 60 to 80 | 1.07 | 3.30 | 1.20 | 1.16 | 0.89 |  |  |  |  |
| 80 to 100 | 0.85 | 2.97 | 0.90 | 0.71 | 0.93 |  |  |  |  |
| 100 to 150 | 0.85 | 2.97 | 0.96 | 0.68 | 0.93 |  |  |  |  |
| 150 to 200 | 0.85 | 2.76 | 0.54 | 0.53 | 0.79 |  |  |  |  |
| 200 to 300 | 0.80 | 2.76 | 0.66 | 0.52 | 0.49 |  |  |  |  |
| 300 to 500 | 1.08 | 2.56 | 0.68 | 0.70 | 0.56 |  |  |  |  |
| 500 to 800 | 1.08 | 1.43 | 0.70 | 0.70 | 0.56 |  |  |  |  |
| 800 to 1000 | 1.77 | 1.43 | 0.86 | 0.69 | 0.83 |  |  |  |  |
| 1000 to 2000 | 1.77 | 1.15 | 1.15 | 0.88 | 1.23 |  |  |  |  |
| >2000 | 2.67 | 0.66 | 1.43 | 0.90 | 1.18 | 0.81 | 2.75 | 0.24 | 0.15 |
| Sticks | 3.37 | 0.84 | 1.28 | 0.19 | 1.23 |  |  |  |  |
| Decomposing leaves | 3.87 | 2.28 | 3.35 | 0.21 | 0.85 |  |  |  |  |
| overall avg COV |  |  | 0.97 | 0.97 |  | 0.66 | 0.63 | 1.72 | 1.55 |

Table 59. Overall Table of Particulate Strength Ratios (continued)

| WinSLAMM particle range (um) | avg Cd ratio with <180 | avg Cu ratio with <180 | overall avg Pb ratio with <250 | overall avg Zn ratio with <250 | overall avg Cr ratio with <250 | Sed Fe ratio with <250 (Khambhammettu 2005) | res/com Hg ratio with <63 (SERDP 2018) | res/com Ni ratio with <63 (SERDP 2018) | res/com As ratio with <63 (SERDP 2018) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| <1 | 0.83 | 2.89 | 3.07 | 2.89 |  |  | 0.49 | 1.82 | 1.73 |
| 1 to 2 | 1.94 | 1.26 | 1.25 | 0.94 |  |  | 0.49 | 1.82 | 1.73 |
| 2 to 5 | 2.12 | 1.15 | 0.51 | 1.05 |  |  | 0.49 | 1.82 | 1.73 |
| 5 to 10 | 1.64 | 0.41 | 0.51 | 0.48 |  |  | 1.58 | 0.75 | 0.77 |
| 10 to 20 | 0.61 | 0.46 | 0.55 | 0.61 |  |  | 1.58 | 0.75 | 0.77 |
| 20 to 40 | 0.45 | 0.31 | 0.33 | 0.56 |  |  | 0.93 | 0.43 | 0.50 |
| 40 to 50 | 0.65 | 0.61 | 0.60 | 0.96 | 1.26 |  | 0.93 | 0.43 | 0.50 |
| 50 to 60 | 0.61 | 0.90 | 0.83 | 0.91 | 1.09 |  | 0.93 | 0.43 | 0.50 |
| 60 to 80 | 2.61 | 1.35 | 0.85 | 1.21 | 1.23 | 1.29 |  |  |  |
| 80 to 100 | 1.09 | 0.99 | 0.79 | 0.84 | 0.89 | 1.06 |  |  |  |
| 100 to 150 | 1.09 | 0.55 | 0.65 | 0.64 | 0.89 | 1.06 |  |  |  |
| 150 to 200 | 1.09 | 0.39 | 0.51 | 0.42 | 0.58 | 0.64 |  |  |  |
| 200 to 300 | 0.04 | 0.36 | 0.48 | 0.53 | 0.56 | 0.60 |  |  |  |
| 300 to 500 | 0.14 | 0.34 | 0.44 | 0.55 | 0.51 | 0.60 |  |  |  |
| 500 to 800 | 0.16 | 0.29 | 0.43 | 0.45 | 0.51 | 0.49 |  |  |  |
| 800 to 1000 | 0.16 | 0.35 | 0.44 | 0.47 | 0.87 | 0.87 |  |  |  |
| 1000 to 2000 | 0.16 | 0.25 | 0.38 | 0.46 | 0.46 | 0.87 |  |  |  |
| >2000 |  | 0.52 | 0.75 | 1.21 | 0.56 | 1.22 | 3.94 | 1.95 | 12.82 |
| Sticks |  | 0.13 | 0.04 | 0.27 | 0.11 |  |  |  |  |
| Decomposing leaves | 0.41 | 0.23 | 0.09 | 0.16 | 0.19 |  |  |  |  |
| overall avg COV | 0.87 | 0.71 | 0.93 | 0.77 |  |  | 0.63 | 0.89 | 0.73 |

Table 59. Overall Table of Particulate Strength Ratios (continued)

| WinSLAMM particle range (um) | indus Cd ratio with <63 (SERDP 20180 | indus Cu ratio with <63 (SERDP 2018) | indus Pb ratio with <63 (SERDP 2018) | indus Zn ratio with <63 (SERDP 2018) | indus Hg ratio with <63 (SERDP 2018) | indus Ni ratio with <63 (SERDP 2018) | indus As ratio with <63 (SERDP 2018) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| <1 |  | 1.64 | 0.95 | 1.59 | 0.69 | 0.43 |  |
| 1 to 2 |  | 1.64 | 0.95 | 1.59 | 0.69 | 0.43 |  |
| 2 to 5 | 0.44 | 1.64 | 0.95 | 1.59 | 0.69 | 0.43 | 0.03 |
| 5 to 10 | 0.44 | 1.64 | 0.57 | 1.23 | 0.38 | 0.17 | 0.03 |
| 10 to 20 | 0.44 | 1.64 | 0.57 | 1.23 | 0.38 | 0.17 | 0.03 |
| 20 to 40 | 1.56 | 1.21 | 1.48 | 1.23 | 1.93 | 2.40 | 1.97 |
| 40 to 50 | 1.56 | 1.21 | 1.48 | 1.23 | 1.93 | 2.40 | 1.97 |
| 50 to 60 | 1.56 | 1.21 | 1.48 | 1.23 | 1.93 | 2.40 | 1.97 |
| 60 to 80 |  |  |  |  |  |  |  |
| 80 to 100 |  |  |  |  |  |  |  |
| 100 to 150 |  |  |  |  |  |  |  |
| 150 to 200 |  |  |  |  |  |  |  |
| 200 to 300 |  |  |  |  |  |  |  |
| 300 to 500 |  |  |  |  |  |  |  |
| 500 to 800 |  |  |  |  |  |  |  |
| 800 to 1000 |  |  |  |  |  |  |  |
| 1000 to 2000 |  |  |  |  |  |  |  |
| >2000 | 40.52 | 5.91 | 3.04 | 7.97 | 6.46 | 6.77 | 4.41 |
| Sticks |  |  |  |  |  |  |  |
| Decomposing leaves |  |  |  |  |  |  |  |
| overall avg COV | 1.11 | 0.94 | 1.00 | 0.63 | 0.98 | 0.51 | 0.69 |

Table 59. Overall Table of Particulate Strength Ratios (continued)

| WinSLAMM particle range (um) | avg Naphthalene ratio with <200 | avg Fluorene ratio with <200 | Acenaphthylene ratio with <63 (SERDP 2018) | avg Phenanthrene ratio with <200 | avg Anthracene ratio with <200 | avg Fluoranthene ratio with <200 | avg Pyrene ratio with <200 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| <1 | 1.45 | 0.88 | 2.42 | 0.22 | 0.31 | 1.60 | 1.63 |
| 1 to 2 | 1.45 | 0.88 | 2.42 | 0.22 | 0.31 | 1.60 | 1.63 |
| 2 to 5 | 0.23 | 0.88 | 2.42 | 0.22 | 0.31 | 1.60 | 1.63 |
| 5 to 10 | 0.23 | 0.50 | 0.06 | 0.96 | 0.25 | 0.25 | 0.52 |
| 10 to 20 | 0.23 | 0.50 | 0.06 | 0.96 | 0.25 | 0.25 | 0.52 |
| 20 to 40 | 1.32 | 1.62 | 0.52 | 1.83 | 2.44 | 1.15 | 0.86 |
| 40 to 50 | 1.30 | 1.39 | 0.52 | 1.57 | 1.82 | 1.18 | 1.05 |
| 50 to 60 | 1.11 | 1.24 | 0.52 | 1.42 | 1.71 | 1.15 | 0.92 |
| 60 to 80 | 0.89 | 0.85 |  | 1.02 | 0.99 | 1.15 | 0.98 |
| 80 to 100 | 0.89 | 0.85 |  | 1.02 | 0.99 | 1.15 | 0.78 |
| 100 to 150 | 0.82 | 0.99 |  | 0.66 | 0.80 | 0.64 | 0.78 |
| 150 to 200 | 0.82 | 0.99 |  | 0.66 | 0.80 | 0.64 | 0.78 |
| 200 to 300 | 0.47 | 0.56 |  | 0.47 | 0.53 | 0.37 | 0.33 |
| 300 to 500 | 0.63 | 0.62 |  | 0.53 | 0.62 | 0.46 | 0.45 |
| 500 to 800 | 0.63 | 0.62 |  | 0.53 | 0.62 | 0.46 | 0.45 |
| 800 to 1000 | 3.98 | 0.90 |  | 0.65 | 1.21 | 0.48 | 0.39 |
| 1000 to 2000 | 3.98 | 0.90 |  | 0.65 | 1.21 | 0.48 | 0.39 |
| >2000 | 4.49 | 1.15 |  | 0.96 | 1.21 | 0.39 | 0.35 |
| LOM and or oily debris | 17.98 | 4.51 | 4.30 | 21.31 | 7.71 | 13.66 | 14.97 |
| avg COV | 1.28 | 1.47 | 1.36 | 1.23 | 1.28 | 1.78 | 1.18 |

Table 59. Overall Table of Particulate Strength Ratios (continued)

| WinSLAMM particle range (um) | avg Chrysene ratio with <200 | avg Benzo(a)anthracene ratio with <200 | avg Benzo(b)flourantene ratio with <200 | Benzo(k)fluoranthene ratio with <63 (SERDP 2018) | avg Benzo(a)pyrene ratio with <200 | avg Dibenzo(a,h)anthracene ratio with <200 | avg Benzo(ghi)perylene ratio with <200 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| <1 | 1.37 | 1.40 | 1.40 | 1.37 | 1.33 | 0.94 | 1.24 |
| 1 to 2 | 1.37 | 1.40 | 1.40 | 1.37 | 1.33 | 0.94 | 1.24 |
| 2 to 5 | 1.37 | 1.40 | 1.40 | 1.37 | 1.33 | 0.94 | 1.24 |
| 5 to 10 | 0.53 | 0.57 | 0.48 | 0.49 | 0.48 | 0.55 | 0.48 |
| 10 to 20 | 0.53 | 0.57 | 0.48 | 0.49 | 0.48 | 0.55 | 0.48 |
| 20 to 40 | 1.10 | 1.03 | 1.13 | 1.14 | 1.19 | 1.51 | 1.28 |
| 40 to 50 | 1.12 | 1.15 | 1.17 | 1.14 | 1.20 | 1.45 | 1.25 |
| 50 to 60 | 1.13 | 1.03 | 1.09 | 1.14 | 1.22 | 1.19 | 1.03 |
| 60 to 80 | 1.16 | 1.04 | 1.05 |  | 1.25 | 0.87 | 0.77 |
| 80 to 100 | 0.70 | 1.04 | 1.05 |  | 1.25 | 0.87 | 0.77 |
| 100 to 150 | 0.70 | 0.69 | 0.73 |  | 0.54 | 0.74 | 1.01 |
| 150 to 200 | 0.70 | 0.69 | 0.73 |  | 0.54 | 0.74 | 1.01 |
| 200 to 300 | 0.39 | 0.43 | 0.82 |  | 0.36 | 0.30 | 0.36 |
| 300 to 500 | 0.53 | 0.57 | 0.46 |  | 0.28 | 0.39 | 0.29 |
| 500 to 800 | 0.53 | 0.57 | 0.46 |  | 0.28 | 0.39 | 0.29 |
| 800 to 1000 | 0.36 | 0.42 | 0.64 |  | 0.39 | 0.77 | 0.94 |
| 1000 to 2000 | 0.36 | 0.42 | 0.64 |  | 0.39 | 0.77 | 0.94 |
| >2000 | 0.45 | 0.70 | 0.72 |  | 0.62 | 0.62 | 0.64 |
| LOM and or oily debris | 11.23 | 11.73 | 9.41 | 14.26 | 6.93 | 14.82 | 4.56 |
| avg COV | 1.12 | 1.42 | 1.45 | 1.57 | 1.52 | 1.48 | 1.38 |

# References Cited and Bibliography

Aryal, R. K., H. Furumai, F. Nakajima, and M. Boller.  2005.  Dynamic behavior of fractional suspended solids and particle bond polycyclic hydrocarbons in highway runoff.  Water Research 39: 5126-5134.

Barry, S. C., S. E. Taylor, and G. F. Birch. 1999. Heavy metals in urban stormwater canals entering Port Jackson, Australia and their impact on the estuarine environment. In Proceedings of the eighth international conference on urban storm drainage. Held in Sydney, Australia August 30 – September 3, 1999, edited by I. B. Joliffe and J. E. Ball. The Institution of Engineers Australia, the International Association for Hydraulic Research, and the International Association on Water Quality.

Bathi, J.R. Associations of Polycyclic Aromatic Hydrocarbons (PAHS) with Urban Creek Sediments. Ph.D. Dissertation submitted to the Department of Civil, Construction, and Environmental Engineering, the University of Alabama, Tuscaloosa, AL. 2008. 296 pgs. http://unix.eng.ua.edu/~rpitt/Publications/11_Theses_and_Dissertations/Dissertation_Jejal_bathi.pdf

Boethling, R.S., D.Mackay. Handbook of Property Estimation Methods for Chemicals. Boca Raton Washington DC: Lewis Publishers, 2000.

Buffleben, M. S., K. Zayeed, D. Kimbrough, M. K. Stenstrom, and I. H. Suffet. 2001. Evaluation of urban non-point source runoff of hazardous metals that enters Santa Monica Bay, California. In Proceedings of the 5th International Conference on diffuse/nonpoint pollution and watershed management. CD-ROM.

Cai, Y. Full-scale UpFlo® Stormwater Filter Field Performance Verification Tests. MSCE Thesis submitted to the Department of Civil, Construction, and Environmental Engineering, the University of Alabama, Tuscaloosa, AL. 2013. 686 pgs. http://unix.eng.ua.edu/~rpitt/Publications/11_Theses_and_Dissertations/Cai_thesis.pdf

Chakrabarti, C. L.,  Y. Lu, and J. Cheng. 1993. Studies on metal speciation in the natural environment. Anal. Chim. Acta. 276: 47-64.

Characklis, G. W., and M. R. Wiesner. 1997. Particles, metals, and water quality in runoff from large urban watershed. J. Environ. Eng. 123, no. 8:753.

Cheng, J., C. L. Chakrabarti, M. H. Back, and W. H. Schroeder. 1994. Chemical speciation of Cu, Zn, Pb and Cd in rain water. Anal. Chim. Acta. 288: 141-156.

Chiou, C.T., L.J. Peters, and  D.W.Schmedding. "Partition Equilibria of Nonionic Organic Compounds between Soil Organic Matter and Water." Environmental Science and Technology 17(4) (1983), 227-231.

Colich, F. 2004. Trace metals concentrations in stormwater runoff from the Evergreen Point Floating Bridge in the Seattle, Washington area. In proceedings of 2003 Georgia Basin/Puget Sound Research Conference.

Dean, C.M.; Sansalone, J.J.; Cartledge, F.K.; Pardue, J.H. (2005). Influence of hydrology on rainfall-runoff metal element speciation. J. Environ. Engineering. 131(4):632-642.

DeCarlo, E. H., V. L. Beltran and M. S. Tomlinson. 2004. Composition of water and suspended sediment in streams of urbanized subtropical watersheds in Hawaii. Applied Geochemistry. 19, no. 7: 1001-1037.

Deletic, A. and D. W. Orr. 2005. Pollution on road surfaces. J. Environ. Eng. 131, no. 1.

Driver, N.E., Mustard, M.H., Rhinesmith, R.B., and Middleburg, R.F. (1985). U.S. Geological Survey Urban Stormwater Database for 22 Metropolitan Areas Throughout the United States. U.S. Geological Survey Open File Report 85-337. Denver, CO: USGS

EPA (USA Environmental Protection Agency) December 1983. Final Report for the Nationwide Urban Runoff Program. Water Planning Division, PB 84-185552, Washington, D.C.

Eppakayala, V.K. Performance Evaluation of Stormwater Treatment Controls at an Industrial Site. Ph.D. Dissertation submitted to the Department of Civil, Construction, and Environmental Engineering, the University of Alabama, Tuscaloosa, AL. 2015. 728 pgs.

Fan, C. Y., T. Field, D. Sullivan, and F. H. Lai. 2001. Toxic pollutants in urban wet-weather flows: An overview of multi-media transport, impacts, and control measures. In proceedings of the ASCE EWRI Confernce – Bridging the gap: Meeting the world’s water and environmental resources challenges. CD-ROM.

Figura, P. and B. McDuffle. 1980. Determination of labilities of soluble trace metal species in aqueous environmental samples by anodic stripping voltammetry and chelex column and batch methods. Anal. Chem. 52:1433-1439.

Florence, T. M. 1977. Trace metal species in fresh water. Water Research 11:681-687.

Florence, T. M., and G. E. Bately. 1980. Chemical speciation in natural waters. CRC Crit. Rev. Anal. Chem. 9:219-296.

Garnaud, S.; Mouchel, J.M.; Chebbo, G.; and Thevenot 1999, D.R. Heavy Metal Concentrations In Dry And Wet Atmospheric Deposits In  Paris District: Comparison With Urban Runoff. Sco. Total Environ. 235, 1-3, 235.

Goodson, K.L. Treatability of Emerging Contaminants in Wastewater Treatment Plants during Wet Weather Flows. Ph.D. Dissertation submitted to the Department of Civil, Construction, and Environmental Engineering, the University of Alabama, Tuscaloosa, AL. 2013. 267 pgs. http://unix.eng.ua.edu/~rpitt/Publications/11_Theses_and_Dissertations/Kenya_dissertation.pdf

Gromaire-Mertz, M. C., Garnaud, S., Gonzalez, A., & Chebbo, G. (1999). Characterisation of urban runoff pollution in Paris. Water Science and Technology, 39(2), 1-8

Grout, H., M. R. Wiesner, and J. Y. Bottero. Analysis of colloidal phases in urban stormwater runoff. Environ. Sci. Technol.33, no. 6: 831. 1999.

Harrison, R.M. and S. J. Wilson. 1985. The chemical composition of highway drainage waters III; runoff water metal speciation characteristics. Sci. Total Environ 43: 89-102.

Hatje, V., Apte, S. C., Hales, L. T., & Birch, G. F. (2003). Dissolved trace metal distributions in Port Jackson estuary (Sydney Harbour), Australia. Marine Pollution Bulletin, 46(6), 719-730

House, L. B., R. J. Waschbusch, and P. E. Hughes. 1993. Water quality of an urban wet detention pond in Madison, Wisconsin, 1987-1988. U. S. Geological Survey, in cooperation with the Wisconsin Department of Natural Resources. USGS Open File Report 93-172.

Hurley, S.E. Urban Watershed Redevelopment: Design Scenarios for Reducing Phosphorus Pollution from Stormwater in Boston’s Charles River Basin, USA. Ph.D. Dissertation, Harvard Design School, Harvard University. Cambridge, MA. May 2009. 453 pages.

Hwang, H.M. and G.D. Foster. "Characterization of Polycyclic Aromatic Hydrocarbons in Urban Stormwater Runoff Flowing into the Tidal Anacostia River."  Environmental Pollution. 140 (2006), 416-426.

Johnson, P. D., R. Pitt, S. R. Durrans, M. Urrutia, and S. Clark. 2003. Metals removal technologies for urban stormwater. Water Environment Research Foundation. WERF 97-IRM-2. ISBN: 1-94339-682-3. Alexandria, VA. 701 pgs. Oct. 2003.

Karichoff, S.W. 1979. "Semi-empirical Estimation of Sorption of Hydrophobic Pollutants on Natural Sediments and Soils." Chemosphere. 10(8) (1981), 833-846.

Karlsson, K., & Viklander, M. (2008). Trace metal composition in water and sediment from catch basins. Journal of Environmental Engineering, 134(10), 870-878

Kearney Foundation. Background Concentrations of Trace and Major Elements in California Soils. Kearney Foundation of Soil Science, Division of Agriculture and Natural Resources, University of California. March 1996.

Khambhammettu, U. Evaluation of Upflow Filtration for the Treatment of Stormwater. MSCE Thesis submitted to the Department of Civil, Construction, and Environmental Engineering, the University of Alabama, Tuscaloosa, AL. 2005. 303 pgs. http://unix.eng.ua.edu/~rpitt/Publications/11_Theses_and_Dissertations/Uday_thesis.pdf

Krein, A., and M. Schorer. 2000. Road runoff pollution by polycyclic aromatic hydrocarbons and its contribution to river sediments. Water Res. (Great Britain) 34:4110.

Mackay, D., W.Y. Shiu, and K.C. Ma. Illustrated Handbook of Physical-Chemical Properties and   Environmental Fate for Organic Chemicals. Boca Raton, FL: Lewis Publishers, 1992.

Maestre, A. (2005). Stormwater Characteristics as Described in the National Stormwater Quality Database (Doctoral dissertation, Ph. D. Dissertation. Department of Civil and Environmental Engineering, University of Alabama. Tuscaloosa, Alabama)

Maestre, A. and R. Pitt. “Identification of significant factors affecting stormwater quality using the National Stormwater Quality Database.” In: Stormwater and Urban Water Systems Modeling, Monograph 14. (edited by W. James, K.N. Irvine, E.A. McBean, and R.E. Pitt). CHI. Guelph, Ontario, pp. 287 – 326. 2006.

Magnuson, M.L.; Kelty, C.A.; and Kelty 2001, K.C. Trace Metal Loading on Water-Borne Soil and Dust Particles Characterized Through the Use of Split-Flow Thin Cell Fractionation. Analytical Chemistry. 73:3492.

Mahler, B.J., P.C. Van Metre, and T.W. Wilson. Parking Lot Sealcoat: An Unrecognized Source of Urban Polycyclic Aromatic Hydrocarbons. Environmental Science & Technology 39 (2005), 5560-5566.

Maniquiz-Redillas, M., & Kim, L. H. (2014). Fractionation of heavy metals in runoff and discharge of a stormwater management system and its implications for treatment. Journal of Environmental Sciences, 26(6), 1214-1222

Morquecho, R.E. Pollutant Associations with Particulates in Stormwater. Ph.D. Dissertation submitted to the Department of Civil, Construction, and Environmental Engineering, the University of Alabama, Tuscaloosa, AL. 2005. 229 pgs. http://unix.eng.ua.edu/~rpitt/Publications/11_Theses_and_Dissertations/Renee_dissertation.pdf

Morrison, G. M. P. and G. Diaz-Diaz. 1988. Size distribution and copper association of dissolved organic material in urban runoff. Environmental Technology Letters 9:109-116.

Mosley, L. M., and B. M. Peake. (2001). Partitioning of metals (Fe, Pb, Cu, Zn) in urban runoff from the Kaikorai Valley, Dunedin, New Zealand. New Zealand J. Marine Freshwater Res 35:615.

Mota, A. M. and M. M. Correia dos Santos. 1995. Trace metal speciation of labile chemical species in natural waters: electrochemical methods. In Metal speciation and bioavailability in aquatic systems. eds. A. Tessier and D. R. Turner, n.p. John Wiley and Sons, Ltd.

Pitt, R. 1979. Demonstration of nonpoint pollution abatement through improved street cleaning practices. EPA-600/2-79-161, U.S. Environmental Protection Agency, Cincinnati, Ohio, August 1979.

Pitt, R. and R. Sutherland August 1982. Washoe County Urban Stormwater Management Program; Volume 2, Street Particulate Data Collection and Analyses. Washoe Council of Governments, Reno, Nevada.

Pitt, R. and J. McLean. 1986. Toronto area watershed management strategy study:  Humber River pilot watershed project. Ontario Ministry of the Environment, Toronto, Ontario, 1986.

Pitt R. (1987). Small Storm Urban Flow and Particulate Washoff Contributions to Outfall Discharges. Ph.D. Dissertation. Department of Civil and Environmental Engineering, University of Wisconsin. Madison, Wisconsin.

Pitt, R. E., R. Field, M. Lalor, and M. Brown. 1995. Urban stormwater toxic pollutants: Assessment, sources, and treatability. Water Environment Research 67 (May/June), no. 3: 260-275.

Pitt, R., B. Robertson, P. Barron, A. Ayyoubi and S. Clark. 1999. Stormwater treatment at critical areas: The multi-chambered treatment train (MCTT). U.S. Environmental Protection Agency, Water Supply and Water Resources Division. National Risk Management Research Laboratory. EPA 600/R-99/017. Cincinnati, Ohio. 169 pgs. March 1999.

Pitt, R., R. Bannerman, S. Clark, and D. Williamson. “Sources of pollutants in urban areas (Part 1) – Older monitoring projects.” In: Effective Modeling of Urban Water Systems, Monograph 13. (edited by W. James, K.N. Irvine, E.A. McBean, and R.E. Pitt). CHI. Guelph, Ontario, pp. 465 – 484 and 507 – 530. 2005a. http://rpitt.eng.ua.edu/Publications/4_Stormwater_Characteristics_Pollutant_Sources_and_Land_Development_Characteristics/Stormwater_pollutant_sources/Mono13Chap23a_part1.pdf

Pitt, R., R. Bannerman, S. Clark, and D. Williamson. “Sources of pollutants in urban areas (Part 2) – Recent sheetflow monitoring results.” In: Effective Modeling of Urban Water Systems, Monograph 13. (edited by W. James, K.N. Irvine, E.A. McBean, and R.E. Pitt). CHI. Guelph, Ontario, pp. 485 – 530. 2005b. http://rpitt.eng.ua.edu/Publications/4_Stormwater_Characteristics_Pollutant_Sources_and_Land_Development_Characteristics/Stormwater_pollutant_sources/Mono13Chap24a_part2.pdf

Ray, H. "Street Dirt as a Phosphorus Source in Urban Stormwater." M.S. thesis, University of Alabama at Birmingham. 1997.

Reible, D., B. Rao, M. Rakowska, D. Athanasiou, I. Drygannaki, B. Chadwick, M. Colvin, G. Rosen. A. Burton, E. Strecker, B. Steets, M. Otto, and R. Pitt. Final Report, Assessment and Management of Stormwater Impacts on Sediment Recontamination. Strategic Environmental Research and Development Program (SERDP) Project ER-2428. Alexandria, VA. December 2017. 72 pgs.

Rubin, A.J., (editor) 1976. Aqueous-Environmental Chemistry of Metals. Ann Arbor Science Publishers, Ann Arbor, Michigan.

Rushton, B.  2006.  Broadway outfall stormwater retrofit project, monitoring CDS unit and constructed pond.  South Florida Water Management District and City of Temple Terrace, W241.  Brooksville, FL.

Sansalone, J. J. 1996. Immobilization of metals and solids transported in urban pavement runoff. In proceedings of the North American water and environmental congress, held in Anaheim, Calif. New York: American Society of Civil Engineers.

Sansalone, J. J. and S.G. Buchberger. 1997a. Characterization of solid and metal element distributions in urban highway stormwater. Wat. Sci. Tech. 36, no. 8-9:155-160.

Sansalone, J. J., and S. G. Buchberger. 1997b. Partitioning and first flush of metals in urban roadway storm water. J. Environ. Eng., 123, no. 2:134.

Sansalone, J. J. and D. W. Glenn III. 2000. Temporal variations in heavy metal partitioning and loading in urban highway pavement sheet flow. Transportation Research Record #1720.

Santos, A., E. Alonso, M. Callejón, and J. C. Jiménez. 2002. Heavy metal content and speciation in groundwater of the Guadiamar River Basin. Chemosphere 48: 279-285.

Smullen, J.T. and Cave K.A. (2002). “National stormwater runoff pollution database.” In: Wet-Weather Flow in the Urban Watershed (Field, R., and D. Sullivan. Ed.). Boca Raton, Florida: Lewis Publishers.

Spokes, J. J., M. Lucia, A. M. Campos, and T. D. Jickells. 1996. The role of organic matter in controlling copper speciation in precipitation. Atmospheric Environment. 30, no. 23:3959-3966.

Spring, R. J., R. B. Howell, and E. Shirley. April 1978. Dustfall Analysis for the Pavement Storm Runoff Study (I-405 Los Angeles). Office of Transportation Laboratory, California Dept. of Transportation, Sacramento, California.

Vignoles M., and L. Herremans. 1995. Metal pollution of sediments contained in runoff water in the Toulouse city. (in French). In Novatech 95, 2nd International Conference on Innovative Technologies in Urban Storm Drainage. May 30 – June 1, 1995. Lyon, France. Pp. 611-614. Organized by Eurydice 92 and Graie.

Zgheib, S., Moilleron, R., Saad, M., & Chebbo, G. (2011). Partition of pollution between dissolved and particulate phases: what about emerging substances in urban stormwater catchments? Water Research, 45(2), 913-925

Zhou, J.L., T.W. Fileman, S. Evans, P. Donkin, C. Llewllyn, J.W. Readman, R.F. Mantoura, and S.J. Rowland. "Fluoranthene and Pyrene in the Suspended Particulate Matter and Surface Sediments of the Humber Estuary, UK." Marine Pollution Bulletin, 36(8) (1998), 587-597.

# Appendix A: Characteristics of Stormwater PAHs and PPCPs Affecting their Portioning to Particulates

Bathi (2008) examined polycyclic aromatic hydrocarbons (PAHs) in urban runoff in both insoluble and particulate-associated forms. Because of their low volatility (low Henry’s Law constant), high octonal-water partition coefficients (KOW) and high soil organic coefficients (KOC), many PAHs are preferentially adsorbed to particulate matter.

In contrast to what one would expect, high molecular weight (HMW) PAHs, which are assumed to be pyrogenic in origin, were found to be from original petroleum sources in these sediments. Of course, some of these primary petroleum materials have undergone combustion in transportation and industrial operations. Tracking the sources of PAHs based on the presence of low molecular weight (LWM) or HMW PAHs also becomes questionable as the PAHs are released into the environment and undergo chemical, physical and biological changes (Countway, et al. 2003). Physical changes (such as evaporation, or physical transport of by air or water from one location to other), chemical changes (such as photo transformation of PAHs to daughter products), and biological changes (such as biotransformation of the PAHs), changes their profile in the environment. Differentiating the sources of PAHs based on observed PAH molecular weights may be a useful tool if the samples analyzed for PAHs are assumed not to be affected by any of these modifications and are recently discharged. Aged sediment has a greater likelihood of undergoing these modifications compared to recently discharged materials, although many of these modifications can occur in the watershed (such as street dirt or other contaminated particulates) before the material is discharged.

The solid-water sorption coefficient (Kd) of a contaminant describes the distribution between the aqueous and solid phases of the system at equilibrium. According to Boethling, et al. (2000), the organic carbon normalized sorption coefficient (Koc) approach is the most appropriate procedure for estimating the sorption coefficients, where:

The Kd is the solid-water sorption coefficient and OC is the organic fraction of the solid. There are many regression models available to estimate the Log Koc of PAHs from Log Kow, where Kow is the octonal water partition coefficient, for example:

Log Koc = 0.904 log Kow – 0.006 (Chiou, et al. 1983)

Log Koc = 1.000 log Kow – 0.210 (Karichhoff, et al. 1979)

Regression equations relating the Log Koc and Log S are also available in the literature, where S is the solubility of PAH in water, for example:

Log Koc = log S + 0.44 (Karichhoff, et al. 1979)

In general, the relationship between the dissolved and sorbed chemical concentrations of PAHs is non-linear in nature which can be represented by the Freundlich isotherm:

Csorb = Kf . (Cw)n ;

The Csorb is the concentration of the sorbed chemical, Kf is the Freundlich constant, Cw is the concentration of the dissolved chemical, and n reflects the nonlinearity, with n equal to one representing a linear partition relationship.

Under equilibrium conditions, the partition coefficients discussed above may be effective in predicting the PAH partition concentrations in the liquid and solid phases, but these predictions may not be accurate for real time systems which are not usually at equilibrium. Differences between predicted sorption coefficients and actual measured observations were seen by Hwang, et al. (2006) in their study of PAHs in stormwater samples along the lower Anacostia River in Washington, D.C. Though the report did not provide the details about how different the predicted and observed values were, they reported that the concentrations of particulate-bond PAHs were higher than the predicted concentrations, as one could expect based on analyses of the solid-water partition coefficient (Kd).

Factors that affect the PAH associations with the particulate matter in the aquatic environment include the physical and chemical properties of the specific PAH contaminant, the physical and chemical properties of the aquatic medium, and the physical and chemical properties of the particulate matter. For the purpose of understanding such effects, Zhou, et al. (1998) studied the relationships between the concentrations of fluoranthene and pyrene on suspended solids with salinity, suspended solids concentration and particulate organic carbon, in the Humber estuary, UK. The concentrations of selected PAHs on suspended solids showed no correlation with the salinity of the samples, while concentrations of suspended solids and particulate organic carbon showed a clear relationship with concentrations of PAHs on the suspended solids. Concentrations of suspended solids in the samples showed negative correlations with the concentrations of selected PAHs on suspended solids, whereas particulate organic content showed positive correlations with the concentrations of particulate-associated PAHs. Zhou, et al. (1998) also showed that higher concentrations of PAHs are likely associated with the finer particles (generally classified as clay material which have large surface areas per unit weight), compared to the coarser particles (generally classified as sand particles which have comparatively less organic matter which are needed for greater sorption of PAHs).

A similar pattern was observed by Aryal, et al. (2005) who monitored suspended solids and PAHs associated with fractionated suspended solids in highway runoff for four rain events (samples were only collected during the initial 3 mm of runoff) at an inlet point of treatment facilities for a highway drainage system in Winterthur, Switzerland. The measured concentrations of PAHs in fine fractions (<45μm) were higher than their concentrations in coarse fractions (>45μm).

Mahler, et al. (2005), of the U.S. Geological Survey, examined PAHs in washoff water runoff and particulates collected from four parking lot test plots. Results indicated that the coal-tar-sealed parking lots had higher concentrations of PAHs than those from any other examined type of surface. The reported average total PAH concentrations in particulates in the runoff from the parking lots were 3,500,000 µg/kg from coal-tar-sealed, 620,000 µg/kg from asphalt-sealed, and 54,000 µg/kg from unsealed parking lots.

Rushton (2006) studied the association of selected PAHs on gross solids while analyzing the performance of a hydrodynamic separator retrofit unit installed to control stormwater discharging to the Hillsborough River, south Florida. The gross solids, consisting of litter, leaves, trash and sediment, collected by the CDS unit was found to have a wide range of concentrations for the selected PAHs. They found high concentrations of PAHs on the gross solids that had high organic content.

Fugacity level 1 (Mackay, et al. 1992) calculations were used by Bathi (2008) to predict the partitioning of PAHs among the environmental phases (only applicable for equilibrium conditions). Prediction fate model calculations for selected PAHs were performed based on typical environmental conditions and with the assumption of system equilibrium. Based on this model, the partition percentages of selected PAHs into different phases were calculated. The equations involved in the model calculations are:

(or)

Where, C = Concentration of contaminant, mol/m3; Z = fugacity capacity constant, mol/m3; f = fugacity, Pa; Vi = Volume of the corresponding phases; and Zi = fugacity capacities of phases for air, water, sediment, suspended sediment, and fish for i =1, 2,3, 4, 5 respectively and are defined as follows.

Where R = gas constant (8.314 J/mol K), T = absolute temperature (K), H= Henry’s law constant (Pa.m3/mol), KOC = Organic-water partition coefficient, KOW  = Octonal-water partition coefficient, P3 = density of sediment (kg/m3), P4 = density of suspended sediment (kg/m3), Ø3= organic fraction of sediment, Ø4= organic fraction of suspended sediment, P5 = density of fish in the aquatic system (kg/m3), L= Lipid content of fish.

During her UA dissertation research, Goodson (2013) investigated the sources and fates of PAHs and pharmaceutical and personal care products (PPCPs) at a Tuscaloosa, AL, wastewater treatment plant that also received substantial stormwater during wet weather.

PAHs are divided into two categories: low molecular weight (LMW) PAHs and high molecular weight (HMW) PAHs.

Table A-1. PAH Categories

| Low Molecular Weight PAHs | High Molecular Weight PAHs |
| --- | --- |
| Naphthalene | Benzo(a)anthracene |
| Acenaphthene | Pyrene |
| Acenaphthylene | Benzo(a)pyrene |
| Fluorene | Chrysene |
| Phenanthrene | Benzo(b)fluoranthene |
| Anthracene | Fluoranthene |

Table A-2. Summary of Characteristics and Treatability of Targeted Pollutants

| Constituent | Log Kow | Solubility (mg/L) | pka | Biodegradation half-life * **rate | Toxicity |
| --- | --- | --- | --- | --- | --- |
| Pharmaceuticals |  |  |  |  |  |
| Gemfibrozil | 4.78 | 5.0 | 4.7 | 1.5 hours | EC 50 D. Magna 22.85 mg/L |
| Ibuprofen | 3.5-4.0 | 41.5 | 4.9 | 2 hours | EC 50 Daphnia. 108 mg/L |
| Triclosan | 4.8-5.4 | 2-4.6 | 7.8 | 125 hours | IC 50 P. subcapitata. 1.4 μg/L |
| Carbamazepine | 2.25 | 17.7 | 13.9 | 10-20 hours | LC 50 D. magna >100 mg/L |
| Fluoxetine | 4.05 | 38.4 | 9.5 | 24-72 hours | LC 50 P. subcapitata 24 μg/L |
| Sulfamethoxazole | 0.9 | 600 | 5.7 | 10 hours | IC 50 P. subcapitata. 1.5 mg/L |
| Trimethoprim | 0.79 | 400 | 6.8 | 8-10 hours | IC 50 P. subcapitata. 80.3 to 130 mg/L |
| Polycyclic Aromatic Hydrocarbons | Log kow | Solubility | Volatility | Biodegradation rate | Toxicity |
| Napthalene | 3.37 | 31.7 | 4.6 x 10-4 | 0.8-43 days | LC 50 Pimephales promelas 7.76 mg/L |
| Acenaphthene | 4.02 | 1.93 | 7.91 x 10-5 | 1-25 days | LC 50 Salmo gairdneri 1570 μg/L |
| Fluorene | 4.12 | 1.68-1.98 | 1.0 x 10-4 | 2-64 days | EC 50 V. fischeri 4.10 μg/mL |
| Fluoranthene | 5.14 | 0.20-0.26 | 6.5 x 10-6 | 880 days | EC 50 S. capricornutum 54,400 μg/L |
| Acenaphthylene | 3.89 | 3.93 | 1.5 x 10-3 | 21-121 days | Did not find |
| Phenanthrene | 4.48 | 1.20 | 2.56 x 10-5 | 19 days ; 35-37 days; | Did not find |
| Anthracene | 4.53 | 0.0076 | 1.77 x10-5 | 108-139 days | EC 50 D.magna 211 μg/L; |
| Pyrene | 5.12 | 0.0.077 (Dabestani and Ivanov 1999, 10-34) | 4.3 x 10-4 | 34 to 90 weeks | EC 50 D.magna 67000 μg/L |
| Benzo(a) anthracene and chrysene | 5.61-5.71 | 0.0016-0.011 | n/a | n/a | n/a |
| Benzo(b) fluoranthene, Benzo(k) fluoranthene, Benzo(a) pyrene, and indeno(1,2,3,cd) pryene |  |  | n/a | n/a | n/a |
| Benzo(a,h) anthracene and Benzo(g,h,i) perlene |  |  | n/a | n/a | n/a |
| Pesticides | Log kow | solubility | Reported most important treatment method | Biodegradation rate | Toxicity |
| Methoxychlor | 4.68-5.08 | 0.1 | Adsorption/biodegradation | 7 to 29 days | D. magna EC 50=1800 μg/L |
| Aldrin | 6.5 | 0.027 | Adsorption/biodegradation | 20-100 days | Salmo gairdneri LC 50 2.6 μg/L |
| Dieldrin | 6.2 | 0.1 | Adsorption/biodegradation | None found | Salmo gairdneri LC 50 1.2 μg/L |
| Chlordane | ~5.54 | insoluble* | Adsorption/biodegradation | 60 days | Chironomus plummosus LC 50 10 μg/L |
| Arochlor Σ | 5.6-6.8 | insoluble* | Adsorption/biodegradation | Variable. Depends on chlorination of compound | P. subcapitata 182nmol/L |
| Lindane | 3.8 | 17 | Adsorption/biodegradation | 69.41 hours | D. magna EC 50=1.64 mg/L |
| Heptachlor | 6.10 | 0.056 | Adsorption/biodegradation | 6 months-3.5 years | S. capricornutum LC 50 26.7 μg/L |
| Heptachlor-epoxide | 5.40 | not found | Adsorption/biodegradation | None found; metabolite | None found |
|  | 4.68-5.08 | 0.1 | Adsorption/biodegradation |  |  |

Naphthalene has a Henry’s Law constant of 0.019 atm-m3/mol, making it more volatile than the other PAHs and more likely to volatize during wastewater treatment. Acenaphthene, acenaphthylene, fluorene, phenanthrene and anthracene have Henry’s Law constants of about 10-3, and their solubilities range from 0.045 to 16.1 mg/L. Volatilization and oxidation were the primary means of reported treatment for PAHs having lower molecular weights. High molecular weight (HMW) PAH compounds (such as pyrene, fluoranthene, chrysene, and benzo(a)pyrene) had higher reduction percentages ranging from 83 to 91 percent. Adsorption is a primary removal factor for the HMW compounds. Influent concentrations for LMW PAHs ranged from 0.016 to 7.3 μg/L. Effluent concentrations for LMW PAHs had a range from 0.002 to 0.7 μg/L. Influent concentrations for the HMW PAHs ranged from 0.044 to 0.47 μg/L. Effluent concentrations for HMW PAHs ranged from 0.013 to 0.06 μg/L.